3GPP TSG-RAN WG4 Meeting # 104-e						R4-2214386

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.11.8

Source:	Qualcomm Incorporated

Title:	WF for NTN demodulation requirements - general and PDSCH

Document for:	Approval

# Introduction

According to the email discussion summary in [1], this document is to capture the WF on general and NTN UE demodulation requirements.

# WF on general aspects

### Sub-topic 1-1

Issue 1-1: Channel model for NTN-TDLA (NLOS)

- Proposals: to confirm the following tentative agreements:

  - Option 1: DS =100ns, Doppler = 200Hz (residual Doppler after UE compensation)

  - Option 2: specify other option if any.

- Agreement:

  - DS =100ns, Doppler = 200Hz

### Sub-topic 1-2

Issue 1-2: Channel model for NTN-TDLC (LOS)

- Proposals: continue to discuss DS and K-factor for NTN-TDLC (LOS)

  - Option 1: DS = 10ns, Doppler =200Hz, K-factor=3.59dB

  - Option 2: DS=12ns, Doppler=200Hz, K-factor=3dB

  - Option 3: DS=100ns, Doppler=200Hz, K-factor=21.6dB

  - Option 4: DS=3.5ns, Doppler=200Hz, K-factor=8.05dB

- Agreement:

  - Assuming elevation angle: 30 degree to decide K_offset and K-factor

    - DS = [3.5ns],   Doppler =200Hz, K_factor= 8.05 dB , K_offset = [8 slots]

### Sub-topic 1-3

Issue 1-3: Channel model parameter combination

- Proposals: to confirm the following tentative agreements:

  - Option 1: Use NTN-TDLX <DS>-<Fd> for channel parameter combination for NTN demodulation

  - Option 2: specify other option if any.

- Agreement:

  - Option 1 agreed

### Sub-topic 1-4

Issue 1-4: Doppler shift due to satellite motion for DL in service link

- Proposals: To confirm the following tentative agreements

  - Option 1: Confirm the agreement that do not verify the UE compensation prior to the baseband processing.

  - Option 2: specify other option if any.

- Agreement:

  - Option 1 agreed

### Sub-topic 1-5

Issue 1-5: Timing drift and sampling frequency offset

- Proposals: to confirm the following tentative agreements:

  - Option 1: Confirm the agreement that do not define sampling offset model.

  - Option 2: specify other option if any.

- Agreement:

  - Option 1 agreed

# WF NTN PDSCH demodulation requirements

### Sub-topic 2-1

Issue 2-1: PDSCH requirements for GEO and LEO

- Proposals: to confirm the following tentative agreements:

  - Option 1: Define PDSCH Demod requirements for LEO-600. Do not define separate set of requirements for GEO

- Agreements:

  - Agree with Option 1

### Sub-topic 2-2

Issue 2-2: Applicability rules for LEO requirements

- Proposals: to confirm the following tentative agreements:

  - Option 1: The applicability for GSO only is proposed to be the same as ‘nonTerrestrialNetwork-r17’: UE needs to pass TS38.101-4 requirements only

- Agreement:

  - From RAN4 UE demodulation requirement perspective, the applicability for GSO only is proposed to be the same as ‘nonTerrestrialNetwork-r17’: UE needs to pass TS38.101-4 requirements only

### Sub-topic 2-3

Issue 2-3: K-offset value

- Proposals: Only specify the k_offset values for LEO-600. Encourage TE vendors to provide the input on the feasibility of supporting a large k_offset value referring to 30o elevation angle for NTN-TDLA test cases and refers to 90° elevation angle for NTN-TDLC test cases.

  - Option 1: Provide the specific values of k_offset for HARQ disabled and HARQ process of 32

- Agreement:

  - K_offfset = [8 slots] applied for all HARQ configurations

### Sub-topic 2-4

Issue 2-4: AoA of the LOS Path for the NTN TDL-C Channel

- Proposals: To further discuss whether to consider the additional parameters for AoA in NTN TDL-C considering 200Hz was agreed as the maximum doppler shift

  - Option 1: Yes, assume an AoA of Ө = 60 degree for the LOS path in NTN TDL-C channel

  - Option 2: No, there is no need to define AoA

- Agreement:

  - Agree with Option 2 as starting point. Revisit if any issue is identified.

### Sub-topic 2-5

Issue 2-5: Modulation order

- Proposals: to confirm the following tentative agreements:

  - Option 1: Do not specify PDSCH for performance requirements for 64QAM

  - Option 2: Specify PDSCH for performance requirements for 64QAM

- Agreement:

  - FFS whether specify PDSCH requirements for 64QAM in Rel-17, operators’ feedback on the demand are encouraged.

### Sub-topic 2-6

Issue 2-6: SCS/CBW set

- Proposals: to confirm the following tentative agreements:

  - Option 1: Do not specify requirements with additional 30KHz SCS for NTN.

  - Option 2: Specify requirements with additional 30KHz SCS for NTN

### Sub-topic 2-7

Issue 2-7: MCS for NTN UE demod requirements

- Proposals

  - Option 1:

    - Consider the following test cases for NTN-TDLA channel model:

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) DMRS |
| 1-1 | R.PDSCH.1-1.1 FDD (QPSK) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |
| 1-2 | R.PDSCH.1-2.1 FDD (16QAM) | 10 / 15 | 16QAM, 0.48 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |

    - Consider the following test cases for NTN-TDLC channel model:

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) DMRS |
| 2-1 | R.PDSCH.1-1.1 FDD (QPSK) | 10 / 15 | QPSK, 0.30 | NTN-TDLC3.5-200 | 1x2, ULA Low | 70 | TBD |
| 2-2 | R.PDSCH.1-2.1 FDD (16QAM) | 10 / 15 | 16QAM, 0.48 | NTN-TDLC3.5-200 | 1x2, ULA Low | 70 | TBD |

  - Option 2: Specify other option if any

- Agreement:

  - Option 1 agreed

### Sub-topic 2-8

Issue 2-8: Simulation assumption for PDSCH performance with disabled HARQ

- Proposals

  - Option 1:

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 [30 kHz SCS: 40 for CSI-RS resource 1,2,3,4] |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 [30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4] |
| Number of HARQ Processes |  |  | 16 (8 with HARQ enabled, 8 with HARQ disabled) |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2+K_offset for enabled HARQ process, otherwise N/A |
| Maximum number of HARQ transmission |  |  | 4 for enabled HARQ process, 1 for disabled HARQ process |

  - Option 2:

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 [30 kHz SCS: 40 for CSI-RS resource 1,2,3,4] |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 [30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4] |
| Number of HARQ Processes |  |  | Disabled |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | N/A |
| Maximum number of HARQ transmission |  |  | 1 |

- Agreement:

  - Option 2

### Sub-topic 2-9

Issue 2-9: Simulation assumption for PDSCH performance with HARQ Processes 32

- Proposals

  - Option 1:

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 [30 kHz SCS: 40 for CSI-RS resource 1,2,3,4] |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 [30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4] |
| Number of HARQ Processes |  |  | 32 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2+K_offset |
| Maximum number of HARQ transmission |  |  | 4 |

  - Option 2: specify other option if any

- Agreement:

  - Option 1 agreed

# Reference

[1]	R4-2214311, Email discussion summary for [104-e][322] NR_NTN_Demod_Part1, Moderator (Qualcomm)