3GPP TSG-RAN WG4 Meeting #104-e	R4-2214387

# Electronic Meeting, 15th – 26th Aug, 2022

Title:	WF on NTN SAN demodulation requirements

Source:	Huawei, HiSilicon

Agenda item:			9.11.8

Document for:	Approval

Introduction

This WF capture all agreements and open issues for the following topics discussed in [104-e][323] NR_NTN_Demod_Part2.

- Satellite Access Node demodulation requirements

  - PUSCH requirements

  - PUCCH requirements

  - PRACH requirements

Satellite Access Node demodulation requirements

PUSCH performance requirements

NTN normal PUSCH requirements

SCS/CBW set

Agreements

- 15kHz SCS: 5MHz, 30kHz SCS: 10MHz

Test applicability rule for different antenna configuration

Candidate options:

- Option 1: Unless otherwise stated, for a SAN supporting different numbers of antenna connectors (for SAN type 1-C) or TAB connectors (for SAN type 1-H) (see D.xxx in table yyy), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

- Option 2: Unless otherwise stated, for a SAN supporting different numbers of antenna connectors (for SAN type 1-C) or TAB connectors (for SAN type 1-H) (see D.xxx in table yyy) by same polarization type, the tests with low MIMO correlation level shall apply only for either one connector or the second lowest number of supported connecters, in addition to the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration

- Option 3: Only 1Rx should be considered for SAN.

Transform precoding

Agreements

- Both CP-OFDM and DFT-s-OFDM

NTN UL timing adjustment requirements

Transform precoding

Agreements

- Only CP-OFDM

Test parameters

Agreements

- Take  = 0.04 S-1 for 15kHz SCS and 0.08 S-1 for 30kHz SCS i.e. (Reuse the parameters of Scenario X)

NTN PUSCH repetition type A requirements

Transform precoding

Agreements

- Only CP-OFDM

PUCCH requirements

DM-RS configuration for PUCCH format 3/4

Agreements

- Both DM-RS 1+0 and 1+1 with SAN manufacture declaration, i.e. supporting additional DM-RS for PUCCH format 3/4 or not.

Antenna configuration

Candidate options:

- Option 1: RAN4 consider SAN PUCCH format 0 and format 2 requirements for only 2Rx configuration. Corresponding manufacture declarations and applicability rules should be further discussed.

- Option 2: 2Rx only for PUCCH requirement

- Option 3: 1Rx only for PUCCH requirement

Channel model

Agreements

- RAN4 to only consider NTN-TDLA channel model for PUCCH requirements definition

PRACH requirements

Preamble Format

Agreements

- Include format 0, B4 and C2 and 2 for SAN PRACH demodulation requirements, further discuss the test applicable rules

- Not include PRACH format A2

Time error tolerance

Agreements

- Use the same method to calculate time error tolerance of multi-path channel as Rel-15: 0.522μ+Tdelay where Tdelay is corresponding to the tap delay of the propagation channel

  - Take the largest tap delay of NTN-TDL-A for time error tolerance calculation.

Simulation assumption

PUSCH

NTN normal PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled, Enabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 group hopping and sequence hopping are disabled for transform precoding enabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low, NTN-TDLC[3.5]-200 Low (K_factor = 8.05) |
| Test metric |  | SNR@70%maximum throughput |

NTN UL timing adjustment

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 12 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 12 |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | CSRS = 5, BSRS =0, for 20 RB |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low |
| Test metric |  | SNR@70%maximum throughput |
| NOTE 1. The transmission of SRS is optional. The transmission comb is configured as KTC = 2. The SRS periodic is configured as TSRS = 10 for 15kHz SCS and 20 for 30kHz SCS respectively. |  |  |

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: NTN-TDLA100-200 Low |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | 0.04 S-1 for 15kHz SCS and 0.08 S-1 for 30kHz SCS |

NTN PUSCH repetition type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 5 in Table 3 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low |
| Test metric |  | SNR@70%maximum throughput |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

PUCCH

NTN PUCCH format 0

| Parameter | Test |
| --- | --- |
| PUCCH format | 0 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 |
| Number of OFDM symbols | 2 |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | ACK missed detection probability <1% |

NTN PUCCH format 1

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

NTN PUCCH format 2 (ACK missed detection)

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

NTN PUCCH format 2 (UCI BLER)

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

NTN PUCCH format 3

| Parameter | Value |
| --- | --- |
| PUCCH format | 3 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DM-RS, Additional DM-RS |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

NTN PUCCH format 4

| Parameter | Value |
| --- | --- |
| PUCCH format | 4 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DMRS, Additional DM-RS |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

NTN PUCCH format 1 (multi-slot)

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 (multi-slot) |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

PRACH

| Parameter | Value |
| --- | --- |
| Preamble format | 0/B4/C2/2 |
| SCS | 1.25kHz for long preamble format 15kHz and 30kHz SCS for short preamble format (LRA=139) |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | AWGN, NTN-TDLA100-200 Low |
| Time error tolerance | 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel |
| Frequency offset | 0Hz for AWGN, 200Hz for fading channel |
| Test metric | false alarm probability<0.1%, detection probability>99% |

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 | 32 |
| 2 | 1.25 | 13 | 22 | 32 |
| B4,C2 | 15 | 23 | 0 | 0 |
|  | 30 | 46 | 0 | 0 |

CR work spliting

There is heavy workload for CR drafting, to faciliate the CR cross checking, interesting companies are welcome to take different clauses in different specifications.

|  | Title | Company | Notes |
| --- | --- | --- | --- |
| 38.108 Satellite Access Node radio transmission and reception |  |  |  |
| bigCR for TS 38.108 |  | Huawei | CR X1 |
| 8 Conducted performance requirements |  |  |  |
| 8.1 | General |  | CR X2 |
| 8.2 | Performance requirements for PUSCH | Huawei | CR X3 |
| 8.3 | Performance requirements for PUCCH | Ericsson | CR X4 |
| 8.4 | Performance requirements for PRACH |  | CR X5 |
| 11. Radiated performance requirements |  |  |  |
| 11.1 | General |  | CR X2 |
| 11.2 | Performance requirements PUSCH | Huawei | CR X3 |
| 11.3 | Performance requirements for PUCCH | Ericsson | CR X4 |
| 11.4 | Performance requirements for PRACH |  | CR X5 |
| Annex Reference measurement channels |  |  |  |
| A.3 | Fixed Reference Channels for performance requirements (QPSK R=193/1024) |  | CR X2 |
| A.4 | PRACH Test preambles |  | CR X5 |
| Annex [D] (Normative): Propagation conditions |  |  |  |
| [D.1] | Static propagation condition |  | CR X6 |
| [D.2] | Multi-path fading propagation conditions |  | CR X6 |
| [D.3] | Moving propagation conditions |  | CR X6 |
| 38.181 Satellite Access Node conformance testing |  |  |  |
| Big CR for TS 38.181 |  | Ericsson | CR Y1 |
| 4. General test conditions and declarations |  |  |  |
| 4.6 | Manufacturer declarations |  | CR Y2 |
| 8 Conducted performance requirements |  |  |  |
| 8.1 | General |  | CR Y2 |
| 8.2 | Performance requirements for PUSCH | Ericsson | CR Y3 |
| 8.3 | Performance requirements for PUCCH | Huawei | CR Y4 |
| 8.4 | Performance requirements for PRACH | Ericsson | CR Y5 |
| 11. Radiated performance requirements |  |  |  |
| 11.1 | General |  | CR Y2 |
| 11.2 | Performance requirements PUSCH |  | CR Y6 |
| 11.3 | Performance requirements for PUCCH |  | CR Y7 |
| 11.4 | Performance requirements for PRACH | Huawei | CR Y8 |
| Annex A |  |  |  |
| A.3 | Fixed Reference Channels for performance requirements (QPSK R=193/1024) |  | CR Y9 |
| A.4 | PRACH test parameters |  | CR Y9 |
| Annex C Test tolerances and derivation of test requirements |  |  |  |
| C.3 | Measurement of performance requirements |  | CR Y2 |
| Annex D Measurement system set-up |  |  |  |
| [D.5] | SAN type 1-H performance requirements |  | CR Y10 |
| [D.6] | SAN type 1-O performance requirements |  | CR Y10 |
| Annex G Propagation conditions |  |  |  |
| G.1 | Static propagation condition |  | CR Y11 |
| G.2 | Multi-path fading propagation conditions |  | CR Y11 |
| G.3 | Moving propagation conditions |  | CR Y11 |

Reference

- R4-2210662, WF on NTN SAN demodulation requirements, RAN4#103-e, Huawei, HiSilicon

- R4-2210329, Email discussion summary for [103-e][323] NR_NTN_Demod_Part2, RAN4#103-e, Moderator (Huawei)

- R4-2207198, WF on NTN SAN demodulation requirements, RAN4#102-e, Huawei, HiSilicon

- R4-2207464, WF on general and NTN UE demodulation requirements, RAN4#102-e, Qualcomm

- R4-2207169, Email discussion summary for [102-e][325] NR_NTN_Demod, RAN4#102-e, Moderator (Qualcomm Incorporated)

- R4-2203043, WF on NTN SAN demodulation requirements, RAN4#101bis-e, Huawei, HiSilicon