3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214390

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.14.8.2

Source:	Qualcomm

Title:	WF on FR2-2 UE demodulation requirements

Document for:	Information

# Topic #1: General Scope of the Demodulation Requirements

### Conclusions from From Way Forward on general aspects for FR2-2 BS demodulation requirements (thread [324] NR_exto71GHz_Demod_Part1)

Moderator: The feedback from TE vendors and related agreement on channel model resolution and tap numbers from the BS thread are included here because they are expected to also apply to UE testing;

Channel model tap delay resolution

- Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW

- [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger  than 200MHz;

- Adopt channel model according to the channel bandwidth as:

| SCS [kHz] | CBW [MHz] | Applicable channel models | Tap resolution [ns] | [Maximum Number of Taps] |
| --- | --- | --- | --- | --- |
| 120 | 100 | TDLA30-650 TDLD30-200 | 5 | 12 |
| 120 480 [960] | ≥400 | TDLA10-650 TDLD10-200 | 2 | 16 |

Companies can bring simulation results based on following simplified TDL model:

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table 1.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table 1.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table 1.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

### Sub Topic 1-1: General on FR2-2 Requirements

Issue 1-1-2a: Whether to include TDLA20 Channel Model for FR2 UE Demodulation requirements

- Agreement: No;

Moderator’s recommendation for Issues 1-1-2b and 1-1-3: Discuss Channel Model, Doppler spread and Modulation Order for different SCS separately in the next meeting. Interested companies are encouraged to bring simulation results to support their views;

Issue 1-1-2b: Highest Modulation order for TDLA10 Channel Model for FR2 UE Demodulation requirements

- Option 1: QPSK;

- Option 2: Decide based on simulation results;

Issue 1-1-3: MCS to use with 650Hz Doppler Frequency for TDLA

- Option 1: QPSK;

- Option 2: Decide based on simulation results;

- Option 3: Do not introduce requirements with 650Hz Doppler Frequency;

Maximum Doppler Frequency for TDLA Channel Model

- Agreement: include 200Hz;

Issue 1-1-4: Whether to define UE Demodulation requirements for 960kHz SCS

- Agreement (GTW August 22, 2022): RAN4 focuses on others SCSs before test feasibility for supporting 960kHz SCS is confirmed.

Issue 1-1-5: Max CBW for UE Demodulation Requirements definition for SCS 120kHz

- Agreement: 100MHz for 120kHz and keep 400MHz FFS

Issue 1-1-6: Max CBW for UE Demodulation Requirements definition for SCS 480kHz

- Agreement: 400MHz for 480kHz and keep 800MHz FFS;

Issue 1-1-7: Whether to define UE Demodulation Requirements with CBW = 1600MHz

- Agreement:

  - Don’t define for SCS 480kHz;

  - FFS the discussion on 960kHz if requirements with 960kHz are agreed;

Issue 1-1-8: Whether to update the number of PRBs for UE Demodulation requirements for 960kHz/2000MHz to follow RF agreements

- Agreement: In case requirements are introduced, follow agreements in RF and update size to 148 PRBs

### Sub Topic 1-2: PDSCH Demodulation Requirements

Issue 1-2-1: Scenarios for FR2-2 PDSCH Requirements definition

- Agreement: Introduce requirements for FR2-2 Single Carrier and FR1+FR2-2 CA Scenarios;

Issue 1-2-2a: Whether to introduce Demodulation requirements with partial bandwidth allocation

- Agreement (GTW August 22, 2022): Yes agreed.

Issue 1-2-2b: Partial bandwidth allocation position in the CBW;

  - Option 1: Center;

  - Option 2: TBA;

Issue 1-2-2c: Whether to align RB size with PRB bundling size;

- Agreement: RB size should be integer multiple of the bundling size;

Moderator’s recommendation for Issues 1-2-2d and 1-2-2e: Discuss RB Allocation size and MCS for different SCS in the next meeting. Interested companies are encouraged to bring simulation results to support their views;

Issue 1-2-2d: RB Allocation Table for SCS 120kHz / CBW 100MHz

  - Option 1: MCS 4 and 13: 66 RBs; MCS 17: 33RBs

  - Option 2: MCS 4,13: 66 PRBs; MCS17: 30PRBs

  - Option 3: MCS 4, 13, 17, and 20: Configure 66 RBs;

Issue 1-2-2e: RB Allocation Table for SCS 480kHz / CBW 400MHz

  - Option 1: MCS 4: 66 RBs; MCS 13: 16 RBs

  - Option 2: MCS 4, 9: 66 RBs; MCS13: 33RBs; MCS 17: 16 RBs;

Issue 1-2-3: Whether RAN4 needs to specify HARQ-ACK Codebook and HARQ Bundling for multi-PDSCH scheduling

- Agreement: Leave specification up to R5, but R4 to ensure no issue in the configuration of the with respect to HARQ bundling

Issue 1-2-4: Whether RAN4 should introduce requirements for 30% of peak throughput

- Agreement (GTW August 22, 2022): FFS whether introduce requirements for 30% of peak throughput

Issue 1-2-5: UE Phase Noise Model for Simulation Alignment

- Both PN Models in TR 38.808;

- Companies clarified that PN modelling can be used to run simulations for the choice of simulation parameters, while simulation results collection for alignment is not expected to use PN models (and expected PN impact is added as a margin to the impairment results). Unless there are diverging views, do not discuss this issue further in the second round;

Issue 1-2-6: UE Phase Noise compensation Model for Simulation Alignment

- Agreement: CPE compensation only is agreed.

- Agreement: Do not consider ICI compensation;

### Sub Topic 1-3: PDSCH Coverage of the requirements

Moderator’s recommendation for Issues 1-3-2: Discuss RB Allocation size and MCS for different SCS in the next meeting. Interested companies are encouraged to bring simulation results to support their views;

Issue 1-3-2: MCS choice for PDSCH requirements for 64QAM (if requirements with 64QAM are agreed)

- Agreement: RAN 4 to select the max MCS that satisfies the following criteria: performance degradation due to phase noise is less than 1dB with the agreed PN compensation technique;

  - Option 1:

    - 120kHz: MCS 22;

    - 480kHz: MCS 20;

  - Option 2:

    - 120kHz: MCS17;

    - 480kHz: MCS 13;

  - Option 3:

    - 120kHz: MCS 20 (66 RBs);

    - 480kHz: MCS 13 (66 RBs), and 17 (16 RBs);

  - Option 4:

    - 120kHz: MCS 20 (66 RBs or 33 RBs);

    - 480kHz: MCS 9 (33 or 16 RBs);

Issue 1-3-3: Rank to be used for the definition of the requirements

- Agreement: Define Rank 1 requirements only;

Issue 1-3-4: Whether to define Rank 1 requirements assuming that the feature ‘FD-OCC Disabled’

- Agreement: Define Rank 1 requirement assuming that RRC is signaling FD-OCC ON for the DMRS configuration chosen;

Issue 1-3-5: PDSCH Requirements Coverage for SCS 120kHz, CBW = 100MHz;

- Agreement: Include up to 64QAM for Rank 1.

Issue 1-3-6: PDSCH Requirements Coverage for SCS 480kHz, CBW = 400MHz;

- Agreement: Include up to 16QAM for Rank 1; Discuss FD allocation size;

Issue 1-3-7: Number of HARQ processes to be used for SCS=960kHz requirement definition if requirements are agreed;

  - Do not discuss further unless 960kHz SCS requirements are agreed;

Issue 1-3-8: Scheduling pattern for CA Scenario

- Agreement: Configure following parameters for FR1 Pcell as baseline. Revisit only in case of issues

    - TDD

    - 30kHz SCS with 7D1S2U

    - All HARQ feedback should be transmitted on UL slot of FR1 carrier

### Sub Topic 1-4: PDSCH Test cases and simulation assumptions

Issue 1-4-1: List of test cases to be used for requirement definition;

Option 1: Table below as baseline and update it based on the outcome of the detailed issues regarding MCS, Doppler Spread, Channel Model, RB size, etc.

| Test | CBW(MHz) / SCS (kHz) | MCS/Rank | Channel Model | Antenna Configuration and Correlation Matrix | RB Allocation Size | % of peak thpt |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100/120 | MCS4/Rank1 | TDLA30-200 | 2x2 ULA Low | [66] | 70 |
| 1-2 | 100/120 | MCS13/Rank1 | TDLA30-200 | 2x2 ULA Low | [x] | 70 |
| 1-4 | 100/120 | MCS17/Rank1 | TDLD30-200 | 2x2 ULA Low | [x] | 70 |
| 2-1 | 400/480 | MCS4/Rank1 | TDLA10-200 | 2x2 ULA Low | [x] | 70 |
| 2-2 | 400/480 | MCS13/Rank1 | TDLA10-200 | 2x2 ULA Low | [x] | 70 |
| 2-4 | 400/480 | MCS17/Rank1 | TDLD10-200 | 2x2 ULA Low | [x] | 70 |

Option 2:

| Test | CBW(MHz) / SCS (kHz) | MCS/Rank | Channel Model | Antenna Configuration and Correlation Matrix | RB Allocation Size | % of peak thpt |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100/120 | MCS4/Rank1 | TDLA30-200 | 2x2 ULA Low | 66 | 70 |
| 1-2 | 100/120 | MCS13/Rank1 | TDLD30-200 | 2x2 ULA Low | 66 | 70 |
| 1-4 | 100/120 | MCS17/Rank1 | TDLD30-200 | 2x2 ULA Low | 30 | 70 |
| 2-1 | 400/480 | MCS4/Rank1 | TDLD10-200 | 2x2 ULA Low | 66 | 70 |
| 2-2 | 400/480 | MCS13/Rank1 | TDLD10-200 | 2x2 ULA Low | 16 | 70 |

Option 3

| Test | CBW(MHz) / SCS (kHz) | MCS/Rank | Channel Model | Antenna Configuration and Correlation Matrix | RB Allocation Size | % of peak thpt |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100/120 | MCS4/Rank1 | TDLA30-650 | 2x2 ULA Low | 66 | 70 |
| 1-2 | 100/120 | MCS13/Rank1 | TDLA30-200 | 2x2 ULA Low | 66 | 70 |
| 1-3 | 100/120 | MCS17/Rank1 | TDLD10-200 | 2x2 ULA Low | 66 | 70 |
| 1-4 | 100/120 | MCS20/Rank1 | TDLD10-200 | 2x2 ULA Low | 33 | 70 |
| 2-1 | 400/480 | MCS4/Rank1 | TDLD10-200 | 2x2 ULA Low | 66 | 70 |

Issue 1-4-3: Test Setup for 70% peak throughput PDSCH Requirements ;

- Agreements: Use the table below as baseline for test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | [TDLA (10 ns and 30 ns delay spread) and TDLD (10 and 30 ns delay spread)] Doppler Spread: [200, 650] Hz |
| Antenna configuration | 2x2 ULA Low |
| Velocity | [3 km/h, 10 km/h] |
| PA Model | None |
| gNB PN Model | None |
| [UE PN Model] | [TR38.808 PN model Set 1 and Set 2] |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| CSI-RS / TRS | Periodicity: [20] ms; Offset: [10] ms; |
| MCS/TBS | From MCS Table 1 (TS38.214): [Up to MCS 22 (64QAM)] Note: It is assumed that NohPRB = 0 for TBS calculations. |

Issue 1-4-4: Test Setup for 30% peak throughput PDSCH Requirements (if 30% peak throughput requirement is agreed)

If 30% throughput requirement is agreed, align the test setup for 70% percent throughput as baseline. Further discuss modifications if necessary.

# Topic #2: PDCCH and PBCH Demodulation Requirements

### Sub Topic 2-1: PDCCH Demodulation Requirements

Issue 2-1-1: Whether to define requirements for multi-slot PDCCH monitoring

- Agreement: RAN4 to follow the mandatory behaviour for multi-slot PDCCH scheduling and schedule PDCCH with (4,1) for 480kHz and (8,1) for 960kHz if agreed

Issue 2-1-2: Set of PDCCH Test Cases

- Agreement: Define PDCCH requirements according to the tablebelow.

| Test number | CBW/SCS | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | [TDLA30-200] | 1_0 | 40 | 1x2 Low |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | [TDLA30-200] | 1_1 | 56 | 1x2 Low |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | [TDLA30-200] | 1_1 | 56 | 2x2 Low |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | [TDLA30-200] | 1_0 | 40 | 2x2 Low |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | [TDLA10-200] | 1_1 | 56 | 1x2 Low |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | [TDLA10-200] | 1_1 | 56 | 1x2 Low |
| 7 | 400/480 | 2 | 60 | 3 | 2 | 16 | [TDLA10-200] | 1_0 | 40 | 2x2 Low |

Issue 2-1-1: Whether to align propagation conditions with PDSCH

- Agreement: Yes

### Sub Topic 2-2: PBCH Demodulation Requirements

Issue 2-2-1: SSB index assumption

- Agreement: Only with not known SSB index

Issue 2-2-2: Test setup for PBCH requirements

- Define requirements for the tests below. Propagation condition to be further discussed;

| Test number | SCS/CBW | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120kHz/100MHz | 1x2 Low | [TDLA10-200] | New | unknown |
| 1-2 | 480kHz/400MHz |  | [TDLA10-200] | New | unknown |

# Topic #2: SDR and CSI Demodulation Requirements

### Sub Topic 3-1: SDR Demodulation Requirements

Issue 3-1-1: Scope of the FR2-2 UE SDR Requirements:

  - Agreement (GTW August 22, 2022): FFS whether SDR test cases will be specified for FR2-2 pending further checking on the test feasibility on supporting SNR range

Issue 3-1-2: Maximum MCS (Scaling Factor = 1) for FR2-2 in the SDR tables

- Proposals:

  - Option 1: MCS 24 for Rank1 and rank2;

  - Option 2: MCS 27 for Rank 1;

Issue 3-1-3: Minimum MCS for FR2-2 in the SDR tables

- Proposals:

  - Option 1: MCS 0 for Rank1 and rank2;

  - Option 2: Depending on maximum Modulation Format:

| Maximum Modulation Format | Minimum MCS |
| --- | --- |
| 6 | 14 |
| 4 | 10 |
| 2 | 4 |

Issue 3-1-4: Rank to use for FR2-2 SDR Requirements

  - Option 1: Rank 1 and Rank 2;

  - Option 2: Rank 1 only;

Issue 3-1-5: Whether to define separate MCS to SNR mapping table for different CBW

- Agreement: define different MCS to SNR mapping table for different CBW;

Issue 3-1-6: Scenarios to consider for the FR2-2 SDR Requirements:

- Agreement: Companies agreed to consider the following scenarios for the definition of FR2-2 SDR requirements

    - SA FR2-2

    - FR1 (CA or single CC) CA with FR2-2 single CC

Issue 3-1-7: For FR1+FR2-2 CA scenarios, whether to transmit ACK/NACK separately on each CC

  - Option 1: Yes;

  - Option 2: No;

### Sub Topic 3-2: CQI Demodulation Requirements

Issue 3-2-1: Scope of the FR2-2 CQI Requirements:

- Agreement (GTW August 22, 2022): Only for SCS 120kHz, CBW=100MHz, further discuss test metric and test SNR points.

Issue 3-2-2: Target Modulation Order for FR2-2 CQI Requirements:

  - Option 1: Up to 16 QAM;

  - Option 2: Include 64QAM;

Issue 3-2-3: Whether to reuse CQI requirements test setup and metrics from FR2-1 for 120kHz/100MHz:

- Agreement: For FR2-2 CQI requirements for 120kHz/100MHz, reuse test setup and test metrics from FR2-1;

- Further discuss whether FR2-1 SNR test points can be reused

  - Option 1: Yes;

  - Option 2: Test points: SNR = 6/7 for 16QAM, -2/-1dB for QPSK;

# Topic #4: Specification Structure and comment collections on Draft CRs

### Sub-topic 4-1: Specification Structure

Issue 4-1-1: Overall Specification Structure

Agreement: Companies agreed on not introducing new subsections and changing applicability section (“Applicability of requirements for operating bands”) to only required testing of table with FR2-2 in FR2-2 bands.

Draft Specification structure:

| Requirements | Sub-clause |
| --- | --- |
| PDSCH requirements for FR2-2 single operation: | 7.2.2.2.1 |
| PDSCH requirements for FR1+FR2-2 CA: | 9.2A.1 |
| PDCCH requirements for FR2-2 single operation: | 7.3.2.2.2 |
| PDCCH requirements for FR1+FR2-2 CA | 9.3A.1 |
| PBCH requirements | 7.4.2.2 |
| [SDR requirements for FR2-2 single operation:] | [7.5.1] |
| [SDR requirements for FR1+FR2-2 CA] | [9.4A.1] |
| CQI requirements for FR2-2 single operation | 8.2.2.2.1 |
| CQI requirements for FR1+FR2-2 CA | 10.2A |

Issue 4-1-1a: Whether to update existing tables by specifying FR2-1 to differentiate from FR2-2

  - Option 1: Yes, update all existing tables (with and without indications to FR2) with the specific indication to FR2, FR2-1 or FR2-2;

  - Option 2: No, rely on clear applicability rules;

  - Option 3: Update only existing tables that explicitly refer FR2; Rely on applicability rules for tables that don’t mention FR2;

Specification Structure for PDSCH

- Option 1:

| Section number | Section name | Note |
| --- | --- | --- |
| 7.2 | PDSCH demodulation requirements | Update Table 7.2-1 |
| 7.2.1 | 1RX requirements |  |
| 7.2.2 | 2RX requirements |  |
| 7.2.2.2 | TDD |  |
| 7.2.2.2.[4] | Minimum requirements for UEs in FR2-2 | New section |
| 7.2A | PDSCH demodulation requirements for CA |  |
| 7.2A.1 | 1RX requirements |  |
| 7.2A.2 | 2RX requirements | Updates on FR2-2 |
| ……. | …… |  |
| A.3.2 | Reference measurement channels for PDSCH performance requirements |  |
| A.3.2.2 | TDD | Updates on FR2-2 |

Issue 4-1-2: Specification Structure for PDCCH:

- Agreement: Follow Table below;

- Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.3 | PDCCH demodulation requirements | Updates in Table 7.3-1 |
| 7.3.1 | 1RX requirements | Void |
| 7.3.2 | 2RX requirements |  |
| 7.3.2.2 | TDD |  |
| 7.3.2.2.1 | 1 Tx Antenna performances | Updates on FR2-2 |
| 7.3.2.2.2 | 2 Tx Antenna performances | Updates on FR2-2 |
| 7.4 | PBCH demodulation requirements |  |
| 7.4.1 | 1RX requirements |  |
| 7.4.2 | 2RX requirements |  |
| 7.4.2.2 | TDD | Updates on FR2-2 UEs |

Issue 4-1-3: Specification Structure for SDR:

- Agreement: 
Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.5 | Sustained downlink data rate provided by lower layers |  |
| 7.5.1 | FR2 single carrier requirements | Updates on FR2-2 |

Issue 4-1-4: Specification Structure for CQI:

- Agreement: Table 1	Specification structure for UE CSI reporting requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 8.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 8.2.2 | 2RX requirements |  |
| 8.2.2.2 | TDD |  |
| 8.2.2.2.1 | CQI reporting definition under AWGN | Updates on FR2-2 |

# CR Work split (for information only)

Table 3. Demod CR work split for TS 38.101-4

| Collecting and organizing simulation results |  |  |  | Ericsson |
| --- | --- | --- | --- | --- |
| draftCR and bigCR work split - TS 38.101-4 |  |  |  | Company |
| TS 38.101-4 bigCR |  |  |  | Huawei |
| TS 38.101-4 UE performance requirements (draftCRs) |  |  |  |  |
|  | 7 Demodulation performance requirements (Radiated requirements) |  |  |  |
|  |  | General (incl. applicability rule) |  | Qualcomm |
|  |  | PDSCH requirements for FR2-2 single operation (Section 7.2.2.2.1) |  | Ericsson |
|  |  | PDSCH requirements for FR1+FR2-2 CA (Section 9.2A.1) |  | Huawei |
|  |  | PDCCH requirements |  | Nokia |
|  |  | (PBCH requirements) |  | Qualcomm |
|  |  | SDR requirements |  | Ericsson |
|  | 8 CSI reporting requirements (Radiated requirements) |  |  |  |
|  |  |  | General (incl. applicability rule), CQI reporting requirements | Apple |
|  | Appendix |  |  |  |
|  |  | PDSCH, PDCCH, (PBCH) RMCs |  | Apple |
|  |  | CSI RMCs and propagation conditions |  | Apple |