3GPP TSG-RAN WG4 Meeting # 104-e														R4-2214396

Electronic Meeting, 15 – 26 Aug 2022

Agenda item:			9.21.4

Source:	Nokia, Nokia Shanghai Bell

Title:	WF on enhanced IIoT and URLLC support demodulation and CSI requirements

Document for:	Approval

# Background

- Previous WFs on this topic

  - R4-2210673, WF on enhanced IIoT and URLLC support demodulation and CSI requirements, Nokia, Nokia Shanghai Bell.

- Corresponding Email summary in RAN4#104-e

  - R4-2214318	Email discussion summary for [104-e][329] NR_IIOT_URLLC_enh_Demod

# WF on topic#1: PUCCH requirements

## Introduction of sub-slot PUCCH repetition requirements

Agreements:

Introduce PF0 performance requirements with sub slot repetition.

## Requirement parameters

Agreements:

PUCCH format: PF0.

Antenna configuration: 1T2R.

SCS/CBW: 30kHz/10MHz.

Channel model: TDLC-300-100 Low.

UCI information bits: 1bit, HARQ-ACK payload.

Number of PRBs: 1 PRB.

PUCCH repetitions across “14-symbol slot” boundaries: No.

Symbol level PUCCH resource configuration [Number of PUCCH symbols per sub-slot]:

- Symbols per sub-slot / PUCCH length = 2

- Sub-slot repetitions (nrofSlots) = 2

- Sub-slot length (subslotLengthForPUCCH-r16) = 7

- First symbol of PUCCH (startingSymbolIndex) = 5.

(sub)Slot frequency hopping:

- Hop between each sub-slot, i.e., interSlotFrequencyHopping=enabled.

- Do not hop within each PUCCH allocation, i.e., intraslotFrequencyHopping=disabled.

- First PRB prior to frequency hopping = 0.

- First PRB after frequency hopping = The largest PRB index – (Number of PRBs – 1).

Other parameters:

- Group and sequence hopping = neither

- Hopping ID =0

- Initial cyclic shift = 0.

KPIs (Issue 1-2-2)

- Use Prob(DTX to ACK)<1%.

- FFS: Prob(ACK miss)<1%.

- FFS: Prob(PUCCH NACK to ACK bits) < 0.1%.

## CR work split (informative)

| NR_IIOT_URLLC_enh demodulation draftCR, bigCR, summary split |  |  |  | Contributor |
| --- | --- | --- | --- | --- |
| TS 38.104 bigCR |  |  |  | Nokia |
| TS 38.104 Performance requirements (draftCRs) |  |  |  |  |
|  | 8 Conducted performance requirements |  |  | Nokia |
|  |  | 8.3 Performance requirements for PUCCH |  |  |
|  |  |  | 8.3.X [Performance requirements for sub-slot repetition PUCCH format 0] |  |
| TS 38.141-1 bigCR |  |  |  | Ericsson |
| TS 38.141-1 Conducted conformance testing (draftCRs) |  |  |  |  |
|  | 4.6 Manufacturer declarations (if required) |  |  | Ericsson |
|  | 8 Conducted performance requirements |  |  | Ericsson |
|  |  | 8.1 General |  |  |
|  |  |  | 8.1.2 Applicability rule (if required) |  |
|  |  | 8.3 Performance requirements for PUCCH |  | Ericsson |
|  |  |  | 8.3.X [Performance requirements for sub-slot repetition PUCCH format 0] |  |
| TS 38.141-2 bigCR |  |  |  | Samsung |
| TS 38.141-2 Radiated conformance testing (draftCRs) |  |  |  |  |
|  | 4.6 Manufacturer declarations (if required) |  |  | Huawei |
|  | 8 Radiated performance requirements |  |  | Huawei |
|  |  | 8.1 General |  |  |
|  |  |  | 8.1.2 Applicability rule (if required) |  |
|  |  | 8.3 OTA Performance requirements for PUCCH |  | Samsung |
|  |  |  | 8.3.X [Performance requirements for sub-slot repetition PUCCH format 0] |  |
| Simulation summary for sub-slot repetition PUCCH format 0 (Sharing of draft well ahead of RAN4#104-bis is encouraged.) |  |  |  | Huawei |