3GPP TSG-RAN WG4 Meeting #104-e	R4-2214401

# Electronic Meeting, 15th – 26th Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PDSCH requirement for HST-SFN scheme B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Perf |  |  |  |  |  | Date: |  |  | 2022-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce PDSCH requirement for HST-SFN scheme B |  |  |  |  |  |  |  |  |
| Summary of change: |  | For PDSCH requirement for HST-SFN scheme B, add clause 5.2.2.1.x, 5.2.2.2.x, 5.2.3.1.x, 5.2.3.2.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsistence between RAN4 agreements and the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.x, 5.2.2.2.x, 5.2.3.1.x, 5.2.3.2.x. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | New clause: 5.2.2.1.x, 5.2.2.2.x, 5.2.3.1.x, 5.2.3.2.x |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF THE CHANGE 1>

##### 5.2.2.1.x	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.2.1.x-3, with the addition of test parameters in Table 5.2.2.1.x-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.x-1.

Table 5.2.2.1.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.x | 1-1 |

Table 5.2.2.1.x-2: Test parameters

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
| Duplex mode |  |  |  |  | FDD |
| Active DL BWP index |  |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  |  | Type A |
|  | k0 |  |  |  | 0 |
|  | Starting symbol (S) |  |  |  | 2 |
|  | Length (L) |  |  |  | 12 |
|  | PDSCH aggregation factor |  |  |  | 1 |
|  | PRB bundling type |  |  |  | Static |
|  | PRB bundling size |  |  |  | 2 |
|  | Resource allocation type |  |  |  | Type 0 |
|  | RBG size |  |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |
|  | TCI state |  |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  |  | Type 1 |
|  | Number of additional DMRS |  |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |
| CSI-RS for tracking | Resource set #1 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  |  | CSI-RS periodicity | Slots | 20 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  |  | CSI-RS periodicity | Slots | 20 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  |  | CSI-RS periodicity | Slots | 20 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #2' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| Number of HARQ Processes |  |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |  |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 3) are transmitted by kth RRH. {TCI state #0, TCI State #1} is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1. {TCI state #1, TCI State #2} is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2. {TCI state #2, TCI State #0} is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3. The second indicated TCI state is not used for QCL indication. |  |  |  |  |  |

Table 5.2.2.1.x-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 10/15 | TBD | HST-SFN-SchemeB | 2x2 | 70 | TBD |

# <END OF THE CHANGE 1>

<START OF THE CHANGE 2>

##### 5.2.2.2.x	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.2.2.x-3, with the addition of test parameters in Table 5.2.2.2.x-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.x-1.

Table 5.2.2.2.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.x | 1-1 |

Table 5.2.2.2.x-2: Test parameters

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
| Duplex mode |  |  |  |  | TDD |
| Active DL BWP index |  |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  |  | Type A |
|  | k0 |  |  |  | 0 |
|  | Starting symbol (S) |  |  |  | 2 |
|  | Length (L) |  |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  |  | 1 |
|  | PRB bundling type |  |  |  | Static |
|  | PRB bundling size |  |  |  | 2 |
|  | Resource allocation type |  |  |  | Type 0 |
|  | RBG size |  |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |
|  | TCI state |  |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  |  | Type 1 |
|  | Number of additional DMRS |  |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |
| CSI-RS for tracking | Resource set #1 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  |  | QCL info |  | TCI state #3 |
|  |  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  |  | Number of PRB = 52 |
|  | Resource set #2 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  |  | QCL info |  | TCI state #4 |
|  |  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  |  | Number of PRB = 52 |
|  | Resource set #3 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  |  | QCL info |  | TCI state #5 |
|  |  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  |  | Number of PRB = 52 |
| NZP CSI-RS for CSI acquisition | Resource set #4 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  |  | CSI-RS periodicity | Slots | 40 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  |  | CSI-RS periodicity | Slots | 40 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  |  | CSI-RS periodicity | Slots | 40 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #2' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| Number of HARQ Processes |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 3) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with {TCI state #0, TCI State #1}. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with {TCI state #1, TCI State #2}. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with {TCI state #2, TCI State #0}. In the codepoint, |  |  |  |  |  |

Table 5.2.2.2.x-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 40/30 | TBD | HST-SFN-SchemeB | 2x2 | 70 | TBD |

# <END OF THE CHANGE 2>

# <START OF THE CHANGE 3>

##### 5.2.3.1.x	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.3.1.x-3, with the addition of test parameters in Table 5.2.3.1.x-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.x-1.

Table 5.2.3.1.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.x | 1-1 |

Table 5.2.3.1.x-2: Test parameters

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
| Duplex mode |  |  |  |  | FDD |
| Active DL BWP index |  |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  |  | Type A |
|  | k0 |  |  |  | 0 |
|  | Starting symbol (S) |  |  |  | 2 |
|  | Length (L) |  |  |  | 12 |
|  | PDSCH aggregation factor |  |  |  | 1 |
|  | PRB bundling type |  |  |  | Static |
|  | PRB bundling size |  |  |  | 2 |
|  | Resource allocation type |  |  |  | Type 0 |
|  | RBG size |  |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |
|  | TCI state |  |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  |  | Type 1 |
|  | Number of additional DMRS |  |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |
| CSI-RS for tracking | Resource set #1 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  |  | CSI-RS periodicity | Slots | 20 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  |  | CSI-RS periodicity | Slots | 20 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  |  | CSI-RS periodicity | Slots | 20 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #2' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| Number of HARQ Processes |  |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |  |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 3) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with TCI {TCI state #0, TCI State #1}. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with {TCI state #1, TCI State #2}. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with {TCI state #2, TCI State #0}. |  |  |  |  |  |

Table 5.2.3.1.x-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 10/15 | TBD | HST-SFN-SchemeB | 2x4 | 70 | TBD |

# <END OF THE CHANGE 3>

# <START OF THE CHANGE 4>

##### 5.2.3.2.x	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.3.2.x-3, with the addition of test parameters in Table 5.2.3.2.x-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.x-1.

Table 5.2.3.2.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.x | 1-1 |

Table 5.2.3.2.x-2: Test parameters

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
| Duplex mode |  |  |  |  | TDD |
| Active DL BWP index |  |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  |  | Type A |
|  | k0 |  |  |  | 0 |
|  | Starting symbol (S) |  |  |  | 2 |
|  | Length (L) |  |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  |  | 1 |
|  | PRB bundling type |  |  |  | Static |
|  | PRB bundling size |  |  |  | 2 |
|  | Resource allocation type |  |  |  | Type 0 |
|  | RBG size |  |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |
|  | TCI state |  |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  |  | Type 1 |
|  | Number of additional DMRS |  |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |
| CSI-RS for tracking | Resource set #1 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  |  | QCL info |  | TCI state #3 |
|  |  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  |  | Number of PRB = 52 |
|  | Resource set #2 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  |  | QCL info |  | TCI state #4 |
|  |  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  |  | Number of PRB = 52 |
|  | Resource set #3 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  |  | QCL info |  | TCI state #5 |
|  |  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  |  | Number of PRB = 52 |
| NZP CSI-RS for CSI acquisition | Resource set #4 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  |  | CSI-RS periodicity | Slots | 40 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  |  | CSI-RS periodicity | Slots | 40 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  |  | CSI-RS periodicity | Slots | 40 |
|  |  |  | CSI-RS offset | Slots | 0 |
|  |  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #2' configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| Number of HARQ Processes |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 3) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with TCI {TCI state #0, TCI State #1}. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with {TCI state #1, TCI State #2}. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with {TCI state #2, TCI State #0}. |  |  |  |  |  |

Table 5.2.3.2.x-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 40/30 | TBD | HST-SFN-SchemeB | 2x4 | 70 | TBD |

# <END OF THE CHANGE 4>

# <START OF THE CHANGE 5>

## B.3.x	HST-SFN Scheme B Channel Profile

There is an infinite number of RRHs distributed equidistantly along the track with the same Cell ID as depicted in figure B.3.x-1.

Figure B.3.x-1: Deployment of HST-SFN Scheme B

The location of RRH k is given as:

xk=k*Ds+j*Dmin	(B.3.x.1)

where:k∈[-∞,  +∞], j=sqrt(-1) and Dmin is the distance between the RRHs and railway track, while Ds is the distance of two RRHs, both in meters.

The train location is denoted as:

y=a+j*0	(B.3.x.2)

where:	a∈[0,  +∞] and a means distance in meters, which means the train is right on the track.

The HST-SFN Scheme B scenario for the test of the baseband performance is a non-fading propagation channel with two taps, namely the two nearest RRHs. Thus, RRH k is visible for the train only in the range:

k*Ds-Ds≤a<k*Ds+Ds	(B.3.x.3)

Power level Pk (dB) for the signal from kth RRH, normalized to the total power received from all visible RRHs, is given by:

Pk= -20logy-xk-10logi∈ii*Ds-Ds≤a<i*Ds+Ds1y-xi2for k*Ds-Ds≤a<k*Ds+Ds			(B.3.x.4)

Doppler shift FD,k (Hz) from kth RRH is given by:

[TBD]	(B.3.x.5)

The relative delay Tk (s) for the signal from kth RRH can be derived as:

Tk= |y-xk|C for k*Ds-Ds≤a<k*Ds+Ds	(B.3.x.6)

In the above v (m/s) is the moving speed of the train, fC (Hz) is the center frequency, and C (m/s) is the velocity of light.

Power level, Doppler shift and relative delay are given by equations B.3.x.4 ~ B.3.x.6 respectively, where the required input parameters listed in table B.3.x-1 and the resulting Doppler shift shown in Figures B.3.x-3 and B.3.x-4 are applied for all requency bands.

Table B.3.x-1: HST-SFN Scheme B scenario

| Parameter | Value |
| --- | --- |
| Ds | 700 m |
| Dmin | 150 m |
| v | 500 km/h |
| fd | 870 Hz for 15 kHz SCS test; 1667 Hz for 30 kHz SCS test |

NOTE 1:	The trajectories of ralative power, Doppler shifts and absolute delays presented in Figures B.3.x-2, B.3.x-3, B.3.x-4 and B.3.x-5 are derived from the equations B.3.x.4 ~ B.3.x.6 respectively.

[TBD]

Figure B.3.x-2 Relative power level trajectories

[TBD]

Figure B.3.x-3 Doppler shift trajectories (fd = 870 Hz)

[TBD]

Figure B.3.x-4 Doppler shift trajectories (fd = 1667 Hz)

[TBD]

Figure B.3.x-5 Absolute delay trajectories

Static channel matrix will be used as defined in Annex B.1.

# <END OF THE CHANGE 5>