3GPP TSG-RAN WG4 Meeting #104-e 	       R4-2214434

# E-meeting, 15th – 26th May 2022

Source: 		Nokia, Nokia Shanghai Bell

Title: 	WF on BS requirements for APT 600 MHz

Agenda Item: 	10.19.4

Document for:	Approval

# Introduction

During RAN4#104-e, documents [1] and [2] were discussed on the expected changes to 38.104 due to introduction of APT 600 MHz band.

# Way forward

The following 38.104 changes are agreed due to introduction of APT 600 MHz band:

##### 6.6.4.2.1	Basic limits for Wide Area BS (Category A)

For BS operating in Bands n5, n8, n12, n13, n14, n18, n26, n28, n29, n71, n85, APT 600 MHz, basic limits are specified in table 6.6.4.2.1-1.

##### 6.6.5.2.3	Additional spurious emissions requirements

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 71 or NR Band n71 | 617 – 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or APT 600 MHz |
|  | 663 – 698 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or APT 600 MHz, since it is already covered by the requirement in clause 6.6.5.2.2. |
| APT 600 MHz | 612 – 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or APT 600 MHz. |
|  | 663 – 703 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band APT 600 MHz, since it is already covered by the requirement in clause 6.6.5.2.2. |

##### 6.6.5.2.4	Co-location with other base stations

| Type of co-located BS | Frequency range for | Basic limits |  |  | Measurement | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  | co-location requirement | WA BS | MR BS | LA BS | bandwidth |  |
| APT 600 MHz | 663 – 703 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

The following 38.104 change is to be discussed and concluded during RAN4#104-bis-e:

6.6.4.2.2.1	Category B requirements (Option 1)

For BS operating in Bands n5, n8, n12, n20, n26, n28, n29, n67, n71, n85, APT 600 MHz, the basic limits are specified in table 6.6.4.2.2.1-1:

# References

[1] R4-2213582; BS requirements for APT 600 MHz band; Nokia, Nokia Shanghai Bell; RAN4#104-e

[2] R4-2213680; Discussion on BS RF requirements for APT600MHz; ZTE; RAN4#104-e