3GPP TSG-RAN WG4 Meeting #104-e	R4-2214444

Electronic Meeting, August 15th – 26th, 2022

Title: 	WF on system parameters for 5G broadcast

Agenda Item:	12.4.5

Source: 	Qualcomm Incorporated

Document for:	Approval

# System parameters for 5G broadcast

<Way forward/Agreement>:

- A new band type is needed for Standalone Downlink Only (SDO).  No impact to RAN1 and RAN2 specifications is expected.

- For BS which is used for LTE based 5G terrestrial broadcast in Standalone Downlink Only band, define new channel bandwidths 6, 7 and 8 MHz.  FFS for the UE.

- Adopt the 100 kHz channel raster and specify the EARFCN correspondingly

- 30, 35, and 40 RB’s is the maximum transmission bandwidth configuration for 15 kHz SCS in 6, 7, and 8 MHz channel bandwidths.

- The existing Nominal Channel spacing definition as following applies:

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective E-UTRA carriers.