3GPP TSG-RAN WG4 Meeting # 104-e                                                            R4-2214445

Electronic Meeting, 15– 26 August 2022

Agenda item:			11.2.4

Source:	CATT

Title:	WF on study on FS_NR_700800900

Document for:	Approval

- Introduction

This WF is to capture the agreements for the SI FS_NR_700800900 discussion in RAN4#104e. The WF was drafted according the 1st round discussion summary [1] and was discussed in the 2nd round.

- Way Forward

  - Topic #1: TP skeleton and work plan

    - Work plan

The following work plan is approved.

RAN4#104-e:

- Identify the critical issues for the band combinations.

- Discuss the skeleton of the TR.

RAN4#104b-e:

- Discuss the potential solutions for the identified issues for each band combination.

- Approve the TR skeleton.

RAN4#105:

- Further discuss the solutions.

- Approve some TPs.

RAN4#106:

- Approve the conclusions for each band combination of the SI.

  - Approve all of the TPs and the first version of TR.

2.1.2 TR skeleton

The TR skeleton in R4-2211714 can be used as the starting point to draft the TR skeleton in next meeting.

  - CA_n5-n8

2.2.1 Spectrum restriction assumption for the analysis

Agreement:

- UE should support the full range of spectrum for single band operations on both n5 and n8

- Use the following frequency ranges for further discussion for spectrum restriction to support uplink CA_n5-n8.

|  | UL | DL |
| --- | --- | --- |
| Frequency 1 (800MHz) | 824MHz - 835MHz | 869MHz - 880MHz |
| Frequency 2 (900MHz) | 904MHz - 915MHz | 949MHz - 960MHz |

2.2.2 UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n8.

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

2.2.3 CBW assumption

The following is the assumption of BCS0:

n5: {5,10}MHz, n8{5, 10}MHz

2.2.4 RF parameters assumption

Continue the RF parameters discussion when the UE RF architecture and CBW are agreed.

2.2.5 Feasibility issues need to be analysed

The following issues will be analyzed in the feasibility study.

  - The MSD due to IMD and cross band isolation.

  - The impact on n5 out-of-band blocking requirement.

2.2.6 Other observations and proposals

The following RF requirement framework applies to CA_n5-n8, CA_n5-n28 and CA_n8-n20-28.

  - Operating bands for CA

  - Channel bandwidths per operating band for CA

  - UE co-existence requirement

  - ∆TIB and ∆RIB values

  - REFSENS requirements (MSD)

  - Maximum output power

2.2.7 UL configuration

The CBW of UL configuration CA_n5-n8 is the same as the DL. The same RB allocation with single band REFSENS UL configuration is considered as the starting point.

  - CA_n5-n28

2.3.1 UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n28

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

2.3.2 CBW assumption

n5: 5, 10, 15, 20

n28: 5, 10, 15, 20, 25, 30

2.3.3 Feasibility issues need to be analysed

The following issues can be analyzed in the feasibility study.

  - The MSD due to cross band isolation

2.3.4 Other observations and proposals

The following proposal is agreed: No IMD for 2UL CA n5-n28

2.3.5 UL configuration

The CBW of UL configuration CA_n5-n28 is the same as the DL. The same RB allocation with single band REFSENS UL configuration is considered as the starting point.

  - CA_n8-n20-n28

2.4.1 Spectrum restriction assumption for the analysis

- UE should support the full range of spectrum for single band operations on n8, n20 and n28

- Use the follow frequency ranges for further discussion for n28 spectrum restriction to support CA_n8-n20-n28.

UL: 703MHz~733MHz

DL: 758MHz~788MHz

2.4.2 UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n8-n20-n28.

2 antenna, 3 antenna, 4 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

2.4.3 CBW assumption

The following CBW assumption can be used in future study

n8: 5, 10, 15

n20: 5, 10, 15, 20

n28: 5, 10, 15, 20

2.4.4 Feasibility issues need to be analysed

The following issues will be analyzed in the feasibility study.

  - UL CA_n8-n20 (IMD3 and IMD5), UL CA_n20-n28 (IMD3).

  - The impact on n20 and n28 out-of-band blocking requirement

2.4.5 UL configuration

UL configuration of CA_n8-n20, CA_n8-n28, CA_n20-n28 are supported for CA_n8-n20-n28. The CBW assumption for UL and DL are the same. The same RB allocation with single band REFSENS UL configuration is considered as the starting point.

Reference

[1] R4-2214108, Email discussion summary for [104-e] [130] FS_NR_700800900