3GPP TSG-RAN WG4 Meeting #104-e	R4-2214454

<Electronic Meeting>, 15 ‒ 26 August, 2022

Title: 	WF on BC in RRC_INACTIVE and initial access

Agenda Item:	11.7.4

Source: 	Nokia

Document for:	Approval

# Sub-topic 1: Rel-16 RRC_Connected Beam Correspondence applicability to Rel-18 RRC_INACTIVE/IA Beam Correspondence

Way forward/Agreements:

- There is no UL beam sweep for IA BC requirements

- At least Msg1 will be tested.

- Use PC3 as baseline for testing and requirements and handle specific values for other PC afterwards and based on the same method

- BC is defined at maximum output power

Way forward/FFS:

- FFS: Discuss whether BC requirements values will be the same for RA-SDT, CG-SDT and initial access, if yes should all be tested?

- FFS: Discuss whether Msg1 and Msg A should have the same requirements? If yes, should both be tested?

- FFS: BC side conditions

- FFS whether RAR reception need to be also tested to verify the similarity between Tx and Rx beams.

# Sub-topic 2: UE beam type and DRX implications in Rel-18 Inactive Beam Correspondence

Way forward/Agreement:

None

Way forward/FFS:

- FFS: How beam refinement may work in RRC_INACTIVE (RA-SDT and CG-SDT) and initial access

- FFS: Discuss the impact of fine beam vs rough beam for UE UL Tx beam on the test(s)

- FFS: Discuss if the refinement is the same as Rel-16 SSB only case

- FFS: Whether the peak EIRP the same as Rel-16 SSB only case achieved.

- FFS: Discuss DRX implications on UE beam refinement and on BC accuracy

# Sub-topic 3: Rel-18 Beam Correspondence Test Issues

Way forward/Agreement:

- BC test for RRC_INACTIVE (if applicable) and RRC_Idle is at UE maximum output power

- BEAM_LOCK is not available in RRC_INACTIVE and IA mode

Way forward/FFS:

- FFS: discuss if and how UE Tx beam should/would change during IA and what are the impact on the test method.

- FFS: Feasibility of testing UEs in Inactive mode

- FFS: Feasibility of triggering SDT in test mode for different UE implementations

- FFS: Whether and how UE Tx beam should/would change without beam lock function during IA when test equipment is holding RAR to achieving UE max output power.

- FFS: How to achieve and keep max power in testing

- FFS: Number of tests, test time implications and proposed test time reductions

- FFS: polarization aspects without beam lock function in testing