3GPP TSG-RAN WG4 Meeting # 104-e	 R4-2214462

Electronic Meeting, August 15 ‒ 26, 2022

Title: 	WF on expanded and improved NR positioning study

Agenda Item:	11.14.4

Source: 	Intel Corporation

Document for:	Approval

# General aspects

## RAN4 work plan

A tentative work plan for RAN4 tasks is included in R4-2212149. For RAN4 #104Bis-e, the following is captured:

| RAN4 Meeting | Tentative Work Plan |
| --- | --- |
| RAN4#104bis-e October 2022, (RF 0.25 TUs RD 0.5 TUs) | - Evaluation work on potential solutions for PRS/SRS bandwidth aggregation for intra-band carriers considering potential timing errors, phase coherency, frequency errors, power imbalance, etc. - Continue study solutions for accuracy improvement based on NR carrier phase measurements - Focus on RAN4 RF aspects and potential inputs to RAN1, if any - Preparation of text proposals for 3GPP TR 38.859, if there are any |

Given the work scope, the above discussion split may not be the most effective. It was suggested to not split discussions between the two sessions in the upcoming meeting.

Way forward: Consider suggested change to discussion split and focus for upcoming RAN4 #104Bis-e meeting

# Accuracy improvement study based on PRS/SRS bandwidth aggregation

## Intra-band CA scenario

Agreement:

- Intra-band contiguous CA scenario will be prioritized in study.

## Scope of study based on PRS/SRS bandwidth aggregation

Agreement:

- Deprioritize power imbalance discussion

Way forward: Further discuss the following in the next RAN4 meeting:

- RF architecture – can we focus on a single RF architecture (i.e., single Tx/Rx chain), align on target architectures

- Studying RF impairment model (timing/group delay/frequency/phase) first to assess performance and accuracy gain with realistic impairments

- Studying achievable accuracy gain when TAE is within specified requirement for intra-band contiguous CA

- Notifying RAN1 of the UE transmit power limitation due to potential prioritization

## Baseline assumptions

Candidate option:

- Proposal: RAN4 assumes that the legacy FFT processing strategy of legacy RXs, that is one FFT processing per CC with standard FFT size, must be baseline. Processing with extended FFT-size specifically for high accuracy positioning measurement is not assumed as baseline.

Way forward:

- Continue discussion in RAN4 #104Bis-e

## Initial conclusion on feasibility

Agreement:

- PRS/SRS bandwidth aggregation for intra-band contiguous carrier is feasible for single chain Tx/Rx architectures

# Accuracy improvement study based on carrier phase measurements

## Scope of study based on carrier phase measurements

Candidate options:

- Proposal 1: RAN4 should study how to model impacts of antenna/beam phase response, residual carrier-frequency offset, and frequency drift on carrier phase positioning

- Proposal 2: RAN4 to wait for further progress in RAN1 to evaluate and assess the scope of solutions based on NR carrier phase measurements to be studied by RAN4.

- Proposal 3: RAN4 need to study the timing error and phase errors among different TRPs and provide the corresponding information to RAN1.

Agreement:

- Wait for RAN1 conclusion or RAN1 LS to start RAN4 work on accuracy improvement study based on carrier phase measurements

# References

- R4-2214248, “Email discussion summary for [104-e][137] FS_NR_pos_UERF,” Moderator (Intel Corporation), RAN4 #104e, August 2022