3GPP TSG-RAN WG4 Meeting # 104-e 		R4-2214467

# Electronic Meeting, 15th – 26th August, 2022

Title:	Way forward on NB-IoT/eMTC NTN agenda item 12.5.1 & 12.5.4

Agenda Item:	12.5

Source:	MediaTek Inc.

Document for:		Approval

# Introduction

This document highlights aspects that the moderator believes can be approved, and other aspects for further discussion. Items in green approved in GTW 19/08.

- Discussion

## 2.1	Topic#1: Work Plan and Scope

The following is proposed to be agreed in each sub-topic:

Sub-topic #1-1

Issue 1-1-1: Work Plan

Work plan to be endorsed.

Issue 1-1-2: Way of working

Agree not to further discuss the separation of work between NB1/2 and cat-M1 at this stage, unless a specific problem is identified with WI progress.

Sub-topic #1-2

Issue 1-2-1: Deployment scenario (standalone vs other operating modes)

Agree to focus the WI requirements work on the standalone deployment scenario for NB-IoT and cat-M1 operation.

## 2.2	Topic#2: Specification drafting general aspects

Sub-topic#2-1

Issue 2-1-1: 36.102 UE spec drafting approach

Agree Option 1: For TS36.102, follow NR NTN approach, with same overall requirement framework and referencing 36.101 where requirements from 36.101 apply to 36.102 and are not band-specific. If not, then include requirement in new TS.

Issue 2-1-3: UE spec structure

A slight majority view to follow Option 1 (Suffix approach). Some support for a non-suffix approach.

Suggest moving forward with Option 1 for now. As we become more familiar with the spec contents it should be very straightforward to adapt to a non-suffix approach if the opinion of the group changes. Do not include general and additional requirements in clause 4 text for these UE categories.

Agreement: For UE spec structure, follow suffix approach and do not include general and additional requirements in clause 4 text.

Sub-topic#2-2

Issue 2-2-1: SAN spec structure

Agree option 1: Adopt TS38.108 structure as baseline approach for E-UTRA SAN spec independently of whether NB-IoT OTA requirements are included.

Issue 2-2-2: Create a 37 series SAN spec to replace the already agreed 36 series SAN specification

No consensus was reached from email discussion or GTW discussion. If a 37 series spec was adopted, the moderator proposes that the scope of work should be limited to single RAT E-UTRA requirements to align with current WI scope.

2.3	Topic#3: UE RF requirements

Moderator Proposal: items in the table with a single option are proposed to be agreed. Items in red require further contribution or discussion.

| Requirement clause | Moderator Proposal for each item (items highlighted in green are agreed in 19/08 GTW) |
| --- | --- |
| Clause 6: Transmitter Characteristics |  |
| 6.1: General | Agreement: Reuse TN (cat-M1 and NB1/2 relevant aspects) |
| 6.2.1: Tx power | Option 2: Power class 3 and 5 as defined for TN for all categories Agreement: - Power class 3 and 5 as defined for all categories - Propose to agree +/-2dB power tolerance. |
| 6.2.2: MPR | Option 1: Depends on outcome of SEM/ACLR discussion |
| 6.2.3: A-MPR | Further discussion needed. Options for all categories were: For b255 - Option 1: Depends on outcome of SEM/ACLR discussion and spurious emission for UE coexistence. - Option 2: Reuse n255 requirements from NR NTN - Option 3: Already clear that A-MPR is not needed. For b256 - Option 1: Depends on outcome of SEM/ACLR discussion and spurious emission for UE coexistence. - Option 2: Reuse n256 requirements from NR NTN - Option 3: Already clear that A-MPR is not needed. |
| 6.2.4: Configured Tx power | Option 1: Reuse TN requirements for all categories Agreement: Reuse TN requirements for all categories |
| 6.3: Output Power Dynamics |  |
| 6.3.1 Minimum output power | Option 1: Reuse TN requirements for all categories Agreement: Reuse TN requirements for all categories |
| 6.3.2 OFF power | All categories: Option 1: Reuse TN requirements Agreement: Reuse TN requirements for all categories |
| 6.3.3 Transmit ON/OFF mask | Cat-M1: Options for further discussion: Option 1: Reuse TN requirements, and sTTI is applicable Option 2: Reuse TN requirements, but sTTI not applicable Agreement: - Do not explicitly preclude sTTI from IoT-NTN. |
|  | NB1/2: Option 1: Reuse TN requirements Agreement: Reuse TN requirements for NB1/2 |
| 6.3.4 Power control | Option 1: Reuse TN requirements for all categories Agreement: Reuse TN requirements for all categories |
| 6.4: Transmit signal quality |  |
| 6.4.1: Frequency error | Agreement: - Reuse 0.1 and 0.2 ppm requirements of frequency error and further discuss the condition where the requirements are applied. |
| 6.4.2: Transmit modulation quality | Cat-M1: Option 1: Reuse TN, but only QPSK and 16QAM in UL. |
|  | NB1/2: Option 1: Reuse TN (not beyond Rel-16 modulation schemes in line with WID) Agreement: Reuse TN (not beyond Rel-16 modulation schemes in line with WID) for NB1/2 |
| 6.5: Output RF spectrum emissions |  |
| 6.5.1: Occupied bandwidth | All categories: Option 1: Reuse TN Agreement: Reuse TN requirements for all categories |
| 6.5.2: Out of band emission |  |
| 6.5.2.1: SEM | All categories: Option 1: Assume TN as baseline, and reconfirm after coexistence verification |
| 6.5.2.2: Additional SEM | Option 1: For all categories and bands, this is not applicable. Agreement: For all categories and bands, this is not applicable. |
| 6.5.2.3: ACLR | Option 1: For all categories, wait for coexistence verification outcome |
| 6.5.3: Spurious emission |  |
| 6.5.3.1: Minimum requirements | Option 1: Reuse TN for all categories Agreement: Reuse TN requirements for all categories |
| 6.5.3.2: For UE co-existence | Option 1: Further input needed but consider NR NTN and existing E-UTRA TN as baselines. |
| 6.5.3.3 Additional spurious emissions |  |
| 6.6: Transmit intermodulation | Cat-M1: Further discuss between: - Option 1: Requirement needs to be defined for 1.4MHz channel bandwidth - Option2: Not applicable for cat-M1 Agreement: - Agree Option 2. |
|  | NB1/2: Option 1: Reuse TN Agreement: Reuse TN for NB1/2 |
| 7.1: General | Option 1: Reuse TN Agreement: Reuse TN requirements for all categories. |
| 7.2: Diversity characteristics | Option 1: Reuse aspects applicable for Cat-M1 and NB1/2 in TN Agreement: Reuse aspects applicable for Cat-M1 and NB1/2 in TN |
| 7.3: Reference sensitivity | Cat-M1 (1.4MHz): Use b24 value for b255 FFS on b256 |
|  | NB1/2: Option 1: Reuse TN (-108.2dBm for both bands) Agreement: Reuse TN (-108.2dBm for both bands) for NB1/2 |
| 7.4: Maximum input level | Option 1: Same relative relaxation (15dB) as for NR NTN for all categories |
| 7.5: ACS | Option 1: Depends on outcome of coexistence verification for all categories |
| 7.6: Blocking characteristics |  |
| 7.6.1: In-band blocking | All categories: Option 1: Reuse TN Agreement: Reuse TN for all the cateogries. |
| 7.6.2: Out-of-band blocking | Cat-M1: For b255: Option 1: reuse TN (some ambiguity in the responses here) Agreement: - For b255, agree Option 1. For b256: Option 2: wait for NR NTN outcome in RAN4#104-e. |
|  | NB1/2: For b255: Option 1: reuse TN (some ambiguity in the responses here) Agreement: - For b255, agree Option 1. For b256: Option 2: wait for NR NTN outcome in RAN4#104-e. |
| 7.6.3: Narrow band | Cat-M1: Option 1: Reuse TN Agreement: reuse TN requirements for Cat-M1. |
|  | NB1/2: Option 1: Not applicable (as for TN) Agreement: Not applicable (as for TN) for NB1/2. |
| 7.7: Spurious response | All categories: Option 1: Reuse TN Agreement: Reuse TN for all categories |
| 7.8: Intermodulation | All categories: Option 1: Reuse TN Agreement: Reuse TN for all categories |
| 7.9: Spurious emissions | Option 1: Reuse TN for all categories Agreement: Reuse TN for all categories. |