3GPP TSG-RAN WG4 Meeting # 104-e 										R4-2214481

Electronic Meeting, 15th-26th, Aug., 2022

Title:	WF on FeMIMO RRM impact for unified TCI state

Agenda item:			9.17.2

Source:	Intel

Document for:	Approval

# Way Forward on Unified TCI state

### Sub-topic 1-1 Active UL TCI state

Issue1-1-1a  Whether UE need to track UL time/frequency for UL TCI state activation when DL-RS is associated with serving cell

- Proposal:

  - Option 1:

    - No, UL timing for cell with different PCI if derived from DL timing of serving cell

  - Option 2:

    - Depends on whether source RS in active UL TCI state is a subset of source RS in DL active TCI list

Issue1-1-1b  Whether UE need to track UL time/frequency for UL TCI state activation when DL-RS is associated with cell with different PCI

- Proposal:

  - Option 1:

    - No, UL timing for cell with different PCI if derived from DL timing of serving cell in Rel-17

  - Option 2:

    - Depends on whether source RS in active UL TCI state is a subset of source RS in DL active TCI list

Issue1-1-2 PL-RS maintenance for active TCI

- Agreement

  - LS is sent to RAN1 for clarification

Issue1-1-3 The relation of active UL TCI state list with active DL TCI state list

- Agreement

  - Independent for separate UL/DL TCI state list indicated by dl-orJoint-TCI-ToAddModList for DL TCI state and ul-TCI-ToAddModList for UL TCI state . But for joint TCI state the same TCI is used for UL and DL indicated by dl-orJoint-TCI-ToAddModList

### Sub-topic 1-2 MAC CE based TCI state Switching delay requirements

Issue 1-2-1 Joint TCI switching delay requirement

- Agreement:

  - For UL TCI state switching,

    - In case of joint TCI state switch, UE is not expected to transmit on UL before UE completes the DL and UL TCI state switch.

- Proposal:

  - For DL TCI state switching,

    - [In case of joint TCI state switch, UE is not expected to receive on DL before UE completes the DL and UL TCI state switch.]

Issue 1-2-2 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2

- Proposals

  - Option 1:

    - Longer delay is expected.

  - Option 2:

    - Reuse the existing delay requirement of MAC CE based UL TCI state switch.

Issue 1-2-3  Wording Update of introduction for active DL/UL TCI state switch delay requirements

- Agreement:

  - Agreed in CR R4-2214631.

### Sub-topic 1-3 Common TCI state switching in CA case

Issue 1-3-1 Known condition on shared RS in CA scenario

- Agreement:

  - The known condition will depend on the associated RS in common TCI state.

Issue 1-3-2 Known condition on different RS in CA scenario

- Agreement:

  - Reuse the existing known condition. If the source RS is configured per CC, then the known condition is per CC.

Issue 1-3-3 Common TCI state switching delay requirement

- Proposals

  - Option 1:

    - Define the requirement without differentiating the triggering signaling, e.g. unifiedTCI-StateRef or simultaneousU-TCI-UpdateList1/2/3/4-r17.

  - Option 2:

    - Define the requirement indicated by IE simultaneousU-TCI-UpdateList1/2/3/4-r17.

  - Option 3:

    - Other options are not precluded

### Sub-topic 1-4 TCI state list update delay

Issue 1-4-1 Whether to consider unknown TCI state in the TCI state list

- Proposals

  - If one or more TCI states in the active TCI state list is unknown, active DL TCI state list update delay is FFS.

Issue 1-4-2 MAC CE based TCI state list update delay for unknown TCI state

- Depends on issue 1-4-1.

### Sub-topic 1-5 Clarification on the applicable TCI after DCI BWP switching

Issue 1-5-1 Clarification on the applicable unified TCI after DCI BWP switching

- Agreed in CR R4-2214594.