3GPP TSG-RAN WG4 Meeting #104-e	R4-2214490

Electronic Meeting, 15-26 August, 2022

Title: 	Simulation assumptions for PRS-RSRP and UE Rx-Tx Time difference for higher side condition

Source: 	Ericsson

Agenda item:	9.19.2.1

Document for:	Approval

# Introduction

In this paper, we present simulation assumptions for PRS-RSRP and UE Rx-Tx time difference for higher side condition with reduced latency.

# Simulation assumptions

Table 1: General parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 cells at distinct locations: <cell 1, cell 2, cell 3>, where cell 1 is the serving cell/reference cellNOTE 1 |  |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 15 kHz - FDD, TDD - 4 GHz - 30 kHz - FDD, TDD | - 40 GHz - 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, TDL-A (30 ns delay spread, 5Hz), TDL-D (30 ns delay spread, 5Hz) | AWGN, TDL-C (60 ns delay spread, 300 Hz), TDL-D (30 ns delay spread, 5Hz) |
| Es/Iot for tw cells (cell 1, cell 2, cell 3), [dB] | (-3, -13, -13) (-3, 0, 3) |  |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |
| UE measurement bandwidth | Full carrier bandwidth |  |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS muting | No muting, muting (comb-2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-UncertaintyNOTE 2 | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |
| NOTE 1: nr-DL-PRS-ReferenceInfo is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). NOTE 2: nr-DL-PRS-expectedRSTD-uncerainty is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). |  |  |

Table 2: PRS transmission configuration parameters

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 24 | 4 | 256 |
|  |  | 52 | 1 | 128 |
|  |  | 104 | 1 | 64 |
|  | 30 | 24 | 4 | 128 |
|  |  | 48 | 1 | 64 |
|  |  | 132 | 1 | 32 |
|  | 60, FR1 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 60, FR2 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 120 | 32 | 4 | 32 |
|  |  | 64 | 1 | 16 |
|  |  | 128 | 1 | 8 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| PRS periodicity | 40ms |  |  |  |
| Sample number | 1, 2, 4 |  |  |  |
| Note: Repetition 4 is achieved by PRS in 4 consecutive slots with PRS-ResourceTimeGap=1. |  |  |  |  |

# Performance metric

At least the following performance characteristics are to be provided for PRS RSRP:

- RSRP error CDFs for 3 cells

- 90%-ile of the RSRP errors for each cell

At least the following performance characteristics are to be provided for TUE-RX:

- TUE-RX error CDFs for the 3 cells

- 90%-ile of the TUE-RX errors for each cell

In the above,

- PRS-RSRP error = estimated PRS-RSRP – ideal PRS-RSRP (based on perfect channel knowledge).

- TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge).