3GPP TSG-RAN WG4 Meeting #104e	R4-2214494

Electronic Meeting, August 15 – August 26, 2022

Title: 	WF on R17 further Multi-RAT Dual-Connectivity enhancements

Agenda Item:	9.20.3

Source: 	Huawei, HiSilicon

Document for:	Approval

Topic #1: Efficient activation/de-activation mechanism for SCells (i.e., temporary RS for efficient SCell activation)

<Way forward >: Issue 1-1: Additional condition for scenario #3

Background

The below agreement is achieved in RAN4#102e. The only open issues is whether add one additional condition.

Scenario #3: SCell to be activated belongs to FR2, if there is no active serving cell on that FR2 band, and target SCell is known to UE. Assuming PDCCH TCI and PDSCH TCI (when applicable) shall be associated with the triggered temporary RS burst: - if semi-persistent CSI-RS is used for CSI reporting, Tactivation_time is 3ms + max (Ttemp_RS+ 2ms, Tuncertainty_SP) - if periodic CSI-RS is used for CSI reporting, Tactivation_time is max (Ttemp_RS + 5ms, Tuncertainty_RRC + TRRC_delay-THARQ) under the condition that - One of the candidate TCI states configured in TCI-StatesPDCCH-ToAddList has the same QCL source of the triggered A-TRS, - The QCL source of CSI-RS for CQI reporting is the same as the triggered A-TRS, - The TCI state for PDCCH/PDSCH that is the same as A-TRS is assumed during SCell activation until changed by network after SCell activation. FFS: whether add one additional condition: UE receives the SCell activation command and TCI state activation command at the same time.

- Proposals

  - Option 1: Add one additional condition

UE receives the SCell activation command and TCI state activation command at the same time.

  - Option 2: Allow the UE the additional activation time for receiving the TCI state activation command if not provided in the same MAC command as the SCell activation command.

Topic #2: Efficient activation/de-activation mechanism for one SCG

< Way forward >: Issue 2-2: Tsearch in RACH-less based PSCell activation delay

Background:

In current specification TS 38.133-h60 For RACH-less based PSCell activation, if RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known FR2 PScell. There are no requirements if PSCell is unknown.

- Proposals

  - Option 1: keep the current requirements.

  - Option2 (Nokia): Discuss the Tsearch for unknown PSCell if the UE is configured with RLM-and-bfd on the deactivated PSCell while no RLM or BFD has occurred before PSCell activation.

< Way forward >: Issue 2-3: Tsearch in RACH-based PSCell activation delay

Background

In current specification TS 38.133-h60 For RACH based PSCell activation, if the target cell is a known NR FR2 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR2 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms.

- Proposals

  - Option 1: keep the current requirements.

  - Option 2: discuss the Tsearch for unknown PSCell if the UE is configured with RLM-and-bfd on the deactivated PSCell while no RLM or BFD has occurred before PSCell activation

< Way forward >: Issue 2-4: known condition for PSCell activation

Background

In current specification TS 38.133-h60 In FR2, the PSCell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the SCG activation command: - the UE has sent a valid measurement report for the PSCell being activated and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3. otherwise it is unknown.

- Proposals

  - Option 1: Reuse the current agreement for PSCell activation.

  - Option 2: Add ‘If bfd_and_RLM is configured for the deactivated PSCell and the TCI state is known’ to PSCell known conditions.

Note: if option 2 is agreed, “PSCell is configured with bfd-and-RLM with value true” may also impact several places, e.g., section 8.1.1, 8.1.5 and 8.2.4.2.18. Please pay attention when drafting the CR.

  - Option 3: Change “UE has not detected beam failure” to ‘Corresponding beam keeps detectable’ in known TCI state condition.

Existing known TCI state condition in TS38.133 If the UE is configured to perform BFD while the SCG is deactivated - The TCI state is known if the following conditions are met: - During the period from the PSCell deactivation to the completion of PSCell activation, while PSCell was deactivated, - UE has not detected beam failure - Otherwise, the TCI state is unknown.

< Way forward >: Issue 2-5: Conditions of RACH-less SCG activation delay: PL-RS of PSCell to use is identical to SSB or source CSI-RS of the TCI state of PSCell

- Proposals

  - Option 1: The RACH-less SCG activation delay requirements are applicable under the condition that PL-RS of PSCell to use is identical to SSB or source CSI-RS of the TCI state of PSCell.

< Way forward >: Issue 2-6: Relax measurements on inter-frequency configured by SCG when SCG is deactivated

- Proposals

  - Option 1 (QC, Apple, MTK, vivo, OPPO): Use the parameter measCyclePSCell to relax measurements on inter-frequency configured only by SCG when SCG is deactivated

  - Option 2 (Nokia, Ericsson, Huawei): No, RAN4 shall keep the new introduced parameter measCyclePSCell within agreed scope

This issue was handled on GTW (2022-08-16). Chair’s note is duplicated:

Chair=> Encourage experts to address the issues as soon as possible. If there is no consensus by next meeting, suggest to discuss it in Rel-18.

< Agreement >: Issue 2-7: Change on measCyclePSCell range

Keep the low bound 160ms for measCyclePSCell.

< Agreement >: Issue 2-8: Conditions on Te requirements

  - There is no necessity to capture the clarification “The UE is not expected to receive SSB for maintaining the Te in the PSCell while the SCG is deactivated except at least one SSB during the last 160 ms before sending the first transmission on the PSCell for activating the SCG” in 38.133.

Topic #3: Test case

< Agreement >: Issue 3-1: Aperiodic CSI-RS configurations for fast SCell activation

Adding the following periodic CSI-RS configurations for fast SCell activation tests.

FR1 FDD:

- Table A.3.17.1.1-3: Aperiodic CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.3 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

- Table A.3.17.1.1-4: Aperiodic CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.4 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

FR1 TDD:

Table A.3.17.1.2-3: Aperiodic CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.3 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |

Table A.3.17.1.2-4: Aperiodic CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.4 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

FR2 TDD

Table A.3.17.2.1-3: Aperiodic CSI-RS for tracking for SCS=120kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.3 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

< Agreement >: Issue 3-2: gapBetweenBursts setup for ATRS based SCell activation test cases

gapBetweenBursts is configured for the scenarios/tests where two consecutive ATRS bursts are needed in fast SCell activation test.

where

gapBetweenBursts When this field is present, there are two bursts and it indicates the gap between the two bursts in number of slots. When this field is absent, there is a single burst.

< Agreement >: Issue 3-3: Whether to specify conditional PSCell change TC

As there are no CPC tests cases from R16, whether to develop test cases for inter-frequency and intra-frequency CPC test cases would not be discussed in this R17 WI.