3GPP TSG-RAN4 Meeting #104-e	R4-2214534

# Online, , August 15-26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | 0001 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.108: Corrections conducted Tx requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some symbols definition are missing. Base station is still used, some other mistakes remain. All sub-clauses title are not aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the missing symbols. Replace “base station” with “SAN”, fixes the identified mistakes Align sub-clauses title. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This might trigger some confusion. Mistakes will remain and this specification would be of poor quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.3.1, 4.4, 6.6.2.2, 6.6.3.2, 6.6.4.2, 6.6.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel.

BeWθ,REFSENS	Beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees. Applicable for FR1 only.

BeWφ,REFSENS	Beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees. Applicable for FR1 only.

SANChannel	SAN channel bandwidth.

BWConfig	Transmission bandwidth configuration, where BWConfig = NRB x SCS x 12.

BWContiguous	Contiguous transmission bandwidth, i.e. SAN channel bandwidth for single carrier.

BWGB,low	The minimum guard band defined in clause 5.3.3 for lowest assigned component carrier.

BWGB,high	The minimum guard band defined in clause 5.3.3 for highest assigned component carrier.

f	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

ΔFGlobal	Global frequency raster granularity.

fmax	f_offsetmax minus half of the bandwidth of the measuring filter.

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge.

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge.

ΔminSENS	Difference between conducted reference sensitivity and minSENS.

ΔOTAREFSENS	Difference between conducted reference sensitivity and OTA REFSENS.

ΔFRaster	Channel raster granularity.

EISminSENS	The EIS declared for the minSENS RoAoA.

EISREFSENS	OTA REFSENS EIS value.

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1.

FC,low	The Fc of the lowest carrier, expressed in MHz.

FC,high	The Fc of the highest carrier, expressed in MHz.

FDL,low	The lowest frequency of the downlink operating band.

FDL,high	The highest frequency of the downlink operating band.

Ffilter	Filter centre frequency.

Foffset,high	Frequency offset from FC,high to the upper SAN RF Bandwidth edge.

Foffset,low	Frequency offset from FC,low to the lower SAN RF Bandwidth edge.

f_offset	Separation between the channel edge frequency and the centre of the measuring.

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band.

FREF	RF reference frequency.

FREF-Offs	Offset used for calculating FREF.

FUL,low	The lowest frequency of the uplink operating band.

FUL,high	The highest frequency of the uplink operating band.

GBChannel	Minimum guard band defined in clause 5.3.3.

Physical resource block number.

NRB	Transmission bandwidth configuration, expressed in resource blocks.

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN).

NREF-Offs	Offset used for calculating NREF scaling per cell, as calculated in clause 6.1.

PEIRP,N	EIRP level for channel N.

Pmax,c,AC	Maximum carrier output power measured per antenna connector.

Pmax,c,TABC	The maximum carrier output power per TAB connector.

Pmax,c,TRP	Maximum carrier TRP output power measured at the RIB(s), and corresponding to the declared rated carrier TRP output power (Prated,c,TRP).

Pmax,c,EIRP	The maximum carrier EIRP when the SAN is configured at the maximum rated carrier output TRP (Prated,c,TRP).

Prated,c,sys 	Prated,c,sys,GEO for SAN GEO class or Prated,c,sys,LEO for SAN LEO class.

Prated,c,sys,GEO 	The sum of Prated,c,TABC for all TAB connectors for a single carrier of the SAN GEO class.

Prated,c,sys,LEO 	The sum of Prated,c,TABC for all TAB connectors for a single carrier of the SAN LEO class.

Prated,c,TABC 	Prated,c,TABC,GEO for SAN GEO class or Prated,c,TABC,LEO for SAN LEO class.

Prated,c,TABC,GEO 	The rated carrier output power per TAB connector of the SAN GEO class.

Prated,c,TABC,LEO 	The rated carrier output power per TAB connector of the SAN LEO class.

Prated,c,TRP	Rated carrier TRP output power declared per RIB.

Prated,t,TRP	Rated total TRP output power declared per RIB.

PREFSENS	Conducted Reference Sensitivity power level.

SSREF	SS block reference frequency position.

<End of the change>

<Start of the change>

### 4.3.1	SAN type 1-H

For SAN type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements.

Figure 4.3.1-1: Radiated and conducted reference points for SAN type 1-H

Radiated characteristics are defined over the air (OTA), where the radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

Conducted characteristics are defined at individual or groups of TAB connectors at the transceiver array boundary, which is the conducted interface between the transceiver unit array and the composite antenna.

The transceiver unit array is part of the composite transceiver functionality receiving and transmitting modulated signal to ensure radio links with users.

The satellite payload is composed by a transceiver unit array and a composite antenna array. The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network which distributes the RF power generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array, in an implementation specific way.

How a conducted requirement is applied to the transceiver array boundary is detailed in the respective requirement clause.

<End of the change>

<Start of the change>

## 4.4	Satellite Access Node classes

The requirements in this specification apply to Satellite Access Node unless otherwise stated. The associated deployment scenarios are exactly the same for SAN with and without connectors.

For SAN type 1-O and SAN type 1-H, two SAN classes (LEO and GEO) are currently defined in Table 4.4-1.

Table 4.4-1 SAN classes

| SAN Class | Satellite Constellation |
| --- | --- |
| GEO class | GEO satellite |
| LEO class | LEO 600 km satellite LEO 1200 km satellite |

<End of the change>

<Start of the change>

#### 6.6.2.2	Minimum requirement for SAN type 1-H

The occupied bandwidth for each carrier shall be less than the SAN channel bandwidth.

<End of the change>

<Start of the change>

#### 6.6.3.2	Minimum requirement for SAN type 1-H

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

<End of the change>

<Start of the change>

#### 6.6.4.2	Minimum requirements for SAN type 1-H

For SAN operating in Bands n256, n255, the requirements are specified in table 6.6.4.2-1 for GEO and LEO class respectively, in line with Annex 5 of ITU recommendation SM.1541-6 [9].

The SAN Operating Band Unwanted Emissions (OBUE) requirements for GEO and LEO classes are therefore defined as described in Table 6.6.4.2-1 below.

Table 6.6.4.2-1: SAN LEO and GEO Classes OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2× BWChannel | 0.002 MHz f_offset < 2× BWChannel + 0.002 MHz | maxSE limit, PSDchannel – ΔSat_ClassdB-40×log10 f_offset-0.002BWChannel×2+1dBm | 4 kHz |
| NOTE 1: PSDchannel = Prated,c, sys – 10log10(BWChannel) – 24, unit dBm/4kHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause 6.6.5. NOTE 3: PSD attenuation as in ITU-R SM.1541-6 [9], Annex 5 OoB domain emission limits for space services. NOTE 4: ΔSat_ClassdB=0 dB for GEO class and ΔSat_ClassdB=3 dB for LEO class. |  |  |  |

<End of the change>

<Start of the change>

#### 6.6.5.2	Minimum requirements for SAN type 1-H

##### 6.6.5.2.1	General transmitter spurious emissions requirements

The requirements in  table 6.6.5.2.1-1 shall apply. The application of those limits shall be the same as for operating band unwanted emissions in clause 6.6.4.

<End of the change>

<Start of the change>

<End of the change>