3GPP TSG-RAN WG4 Meeting # 104	R4-22xxxxx

Electronic Meeting, 15th – 26th August 2022

Title: 	Way Forward on general aspects for FR2-2 BS demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	9.14.8.3

Document for:	Discussion

Agreements and Open issues

Channel model tap delay resolution

- Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW

- Tap numbers: 12 taps for CBW smaller than 200MHz and [16] taps for CBW larger  than 200MHz

Doppler shift for demodulation requirements above 52.6 GHz

- 650Hz for TDLA and 200Hz for TDLD for PUSCH, PUCCH and PRACH requirements

RMS delay spread

- Adopt channel model according to the channel bandwidth as:

| SCS [kHz] | CBW [MHz] | Applicable channel models | Tap resolution [ns] | Tap number |
| --- | --- | --- | --- | --- |
| 120 | 100 | TDLA30-650 TDLD30-200 | 5 | 12 |
| 120 480 [960] | ≥400 | TDLA10-650 TDLD10-200 | 2 | 16 |

Simplified TDLA model for demodulation requirements above 52.6 GHz

Companies can bring simulation results based on following simplified TDL model.

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table 1.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table 1.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table 1.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

SCS and CBW  for demodulation requirements

- RAN 4 agreed to introduce BS demodulation requirements at least:

  - 100MHz CBW for 120kHz SCS

  - 400MHz CBW for 480kHz SCS

  - FFS for 1600MHz for 480kHz, 400MHz and 2000MHz CBW for 960kHz SCS (if introduced)

ICI compensation for demodulation requirements

- For test cases with minimum CBW, no need to consider ICI compensation,

- For other cases with larger CBW if introduced, FFS whether ICI compensation shall be considered or not

SNR limit

- Proposals

  - Option 1: Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage. Pending the decision until RF have agreements on the link budget.

  - Option 2: Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage.

  - Other options are not precluded

Base station structure

- Proposals

  - Proposal 1:. Define the FR2-2 requirements in the same clause as FR2-1 requirements but separate requirements tables. The legacy FR2-1 requirements table title should be updated to “xxx requirements for BS type 2-O operating in spectrum range of 24.25~52.6 GHz” and the table title for FR2-2 requirements should be said as “xxx requirements for BS type 2-O operating in spectrum range of  52.6 GHz~71 GHz”.

  - Proposal 2: Use Table 2.2 for TS 38.104 spec structure and Table 2.3 for TS 38.141-2 spec structure

- Proposed positions in TS 38.104 for FR2-2 requirements

| Requirements | Sub-clause |
| --- | --- |
| CP-OFDM PUSCH performance requirements | 11.2.2.1 |
| PUCCH OTA performance requirements for format 0 | 11.3.2.2 |
| PUCCH OTA performance requirements for format 1 | 11.3.2.3 |
| PUCCH OTA performance requirements for format 2 | 11.3.2.4 |
| PUCCH OTA performance requirements for format 3 | 11.3.2.5 |
| PUCCH OTA performance requirements for format 4 | 11.3.2.6 |
| PRACH OTA performance requirements | 11.4.2.2 |

- Proposed positions in TS 38.141-2 for FR2-2 requirements

| Requirements | Sub-clause |
| --- | --- |
| CP-OFDM PUSCH performance requirements | 8.2.1 |
| PUCCH OTA performance requirements for format 0 | 8.3.1 |
| PUCCH OTA performance requirements for format 1 | 8.3.2 |
| PUCCH OTA performance requirements for format 2 | 8.3.3 |
| PUCCH OTA performance requirements for format 3 | 8.3.4 |
| PUCCH OTA performance requirements for format 4 | 8.3.5 |
| PRACH OTA performance requirements | 8.4.1 |

CR split

Table 2-1: CR split for TS 38.104

| TS 38.104 BigCR | Intel |
| --- | --- |
| PUSCH requirements | Intel |
| PUCCH requirements | Huawei |
| PRACH requirements | Nokia |
| PUSCH FRC | Ericsson |
| PRACH Test preambles, propagation conditions | Samsung |

Table 2-2: CR split for TS 38.141-2

| TS 38.141-2 BigCR | Nokia |
| --- | --- |
| PUSCH requirements | Huawei |
| PUCCH requirements | Intel |
| PRACH requirements | Nokia |
| PUSCH FRC | Ericsson |
| PRACH Test preambles, propagation conditions | Samsung |