3GPP TSG-RAN4 Meeting #104-e	R4-2214682

# Electronic meeting, 15th-26th, Aug., 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS 38.133: NR FR1 interruptions at NR SRS antenna port switching with more than 1 SRS symbol in NR-CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-8-2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In Rel-17, NR interruptions requirement at NR SRS antenna port switching with more than 1 SRS symbol in NR-CA needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add NR FR1 interruptions test case at NR SRS antenna port switching with more than 1 SRS symbol in NR-CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - NR SRS antenna port switching interruptions requirement with more than 1 SRS symbol will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | - A.6.5.2.x2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X | X | Test specifications |  |  |  | TS/TR 38.533 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==========================Start of first change =============================

#### A.6.5.2.x2	SA interruptions at NR SRS antenna port switching

##### A.6.5.2.x2.1	Test Purpose and Environment

The purpose of this test is to verify the interruption requirement on victim CC during SRS antenna port switching with more than 1 SRS symbols on aggressor CC defined in clause 8.2.2.2.16. The interruption requirement is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand as specified in requirement applicability in clause 8.2.2.2.16.

##### A.6.5.2.x2.2	Test Parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the FR1 PCell and Cell 2 is FR1 SCell. The UE is configured with the SRS antenna port in FR1 PCell. The test parameters for PCell and SCell are given in Table A.6.5.2.x2.2-2 and A.6.5.2.x2.2-3 below. Common SRS configuration is given in A.3.24. Dedicated SRS configuration which is dependent on reported SRS capability supportedSRS-TxPortSwitch, is given in Table A.6.5.2.x2.2-4. The test consists of two successive time periods, with duration of T1 and T2, respectively. Immediately at the beginning of T2, the UE is triggered for SRS antenna port switching.

The test equipment verifies that potential interruption is carried out correctly by monitoring ACK/NACK sent in PCell.

Table A.6.5.x2.2-1: Supported test configurations

| Configuration | Description |  |
| --- | --- | --- |
|  | NR PCell Note 2 | NR SCell Note 2 |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: Test configuration for NR PCell and NR SCell are chosen independently. |  |  |

Table A.6.5.2.x2.2-2: General test parameters for SA interruptions at NR SRS antenna port switching

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1 |
| Configured SCell |  | Cell 2 | Activated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 5 |  |
| T2 | ms | 40 | UE shall perform SRS antenna port switching during T2 |

Table A.6.5.x2.2-3: Cell specific test parameters for SA interruptions at NR SRS antenna port switching

| Parameter |  |  |  | Unit | T1 |  |  | T2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| Duplex mode |  | Config 1 |  |  | FDD | TDD |  | FDD |  | TDD |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  | N/A | TDDConf.1.2 |  | N/A |  | TDDConf.1.2 |  |
|  |  | Config 2 |  |  | TDDConf.1.2 |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.3 |  |  |  |  |  |  |
| BWchannel |  | Config 1,2 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| Downlink initial BWP Configuration |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP Configuration |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| TCI state |  |  |  |  | TCI.State.0 |  |  |  |  |  |  |
| TRS Configuration |  |  |  |  | TRS.1.1 TDD |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD | SR.1.1 TDD |  | SR.1.1 FDD |  | SR.1.1 TDD |  |
|  |  | Config 2 |  |  | SR.1.1 TDD | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  |  | SR2.1 TDD | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.1 FDD | CCR.1.1 TDD |  | CCR.1.1 FDD |  | CCR.1.1 TDD |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  |  | CCR.2.1 TDD | CCR.2.1 TDD |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD | CR.1.1 TDD |  | CR.1.1 FDD |  | CR.1.1 TDD |  |
|  |  | Config 2 |  |  | CR.1.1 TDD | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  |  | CR2.1 TDD | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| ptrs-portindex |  |  |  |  | NA. | n0 |  | NA. |  | n1 |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  |  |  | SSB.1 FR1 |  |  |  |  |  |  |
|  | Config 3 |  |  |  | SSB.2 FR1 |  |  |  |  |  |  |
| SRS configuration | Config 1,2 |  |  |  | SRS.x1 Note 4 |  | - |  | SRS.x1 Note 4 |  | - |
|  | Config 3 |  |  |  | SRS.x2 Note 4 |  | - |  | SRS.x2 Note 4 |  | - |
| SMTC configuration |  |  |  |  | SMTC.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | Config 1,2 | dBm/15kHz | -104 |  |  |  |  |  |  |
|  |  |  | Config 3 |  | -101 |  |  |  |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |  |  |  |
| SS-RSRPNote3 |  |  | Config 1,2 | dBm/SCS | -87 |  |  |  |  |  |  |
|  |  |  | Config 3 |  | -84 |  |  |  |  |  |  |
| SCH_RP Note 3 |  |  |  | dBm/15 kHz | -87 |  |  |  |  |  |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Dedicated SRS configuration in Table A.6.5.x2.2-4 |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.x2.2-4: Dedicated Sounding Reference Symbol Configuration for xTyR configuration

| Parameter |  | xTyR configuration |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| srs-ResourceId |  | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| nrofSymbols |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| startPosition |  | 5 | 3 | 5 | 3 | 5 | 3 | 5 | 3 |
| nrofSRS-Ports |  | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | Config 1,2 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 6 | sl40, 6 |
|  | Config 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 13 | sl80, 13 |

##### A.6.5.2.x2.3	Test Requirements

The UE shall be scheduled on PCell continuously throughout the test. During the time duration T2, the interruption on SCell shall not be more than the values specified in Table 8.2.1.2.18-3 in clause 8.2.1.2.18 for each SRS transmission slot.

The rate of correct events observed during repeated tests shall be at least 90%.

==========================End of first change =============================