3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214760

# Electronic meeting,  15th  - 24th August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS38.104 | CR | DRAFT | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for PUSCH with DM-RS bundling demodulation requirement for FR1 and FR2-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-07-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The demodulation requirements for PUSCH with DM-RS bundling have been agreed to be introduced by RAN4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding minimum requirements for PUSCH with DM-RS bundling performance for conducted and radiated parts. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no demodulation requirement for PUSCH with DM-RS bundling for FR1 and FR2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2, 11.2.1, 11.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.141-1/2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2212245. |  |  |  |  |  |  |  |  |

###################### Start of change #1 ###############################

### 8.2.x	Requirements for PUSCH with DM-RS bundling

#### 8.2.x.1	General

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Table: 8.2.x.1-1 Test parameters for testing PUSCH with DM-RS bundling

| Parameter |  |  |  | Value |
| --- | --- | --- | --- | --- |
| Transform precoding |  |  |  | Disabled |
| Example UL-DL pattern [Note 1] |  |  |  | 15 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U 30 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U |
| HARQ |  | Maximum number of HARQ transmissions |  | 4 |
|  |  | RV sequence |  | 0, 0, 0, 0 for FDD 0, 3, 0, 3 [Note 2] for TDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos0, pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment |  | PUSCH mapping type |  | A, B |
|  |  | Start symbol |  | 0 |
|  |  | Allocation length |  | 14 |
|  |  | PUSCH aggregation factor |  | n8 for FDD n2 for TDD |
| pusch-TimeDomainWindowLength |  | 8 slots for FDD 2 slots for TDD |
| Frequency domain resource assignment |  | RB assignment |  | Full applicable test bandwidth |
|  |  | Frequency hopping |  | Disabled |
| Code block group based PUSCH transmission |  |  |  | Disabled |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pusch-TimeDomainWindowLength and PUSCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUSCH is only scheduled for the actual TDW including 2 consecutive UL slots.. Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

#### 8.2.x.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs stated in tables 8.2.x.2-1 to 8.2.x.2-8 at the given SNR for 1Tx. FRCs are defined in annex A.

Table 8.2.x.2-1: Minimum requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS FDD

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 4 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 8 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |

Table 8.2.x.2-2: Minimum requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS TDD

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 4 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 8 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |

Table 8.2.x.2-3: Minimum requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS FDD

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 4 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 8 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |

Table 8.2.x.2-4: Minimum requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS TDD

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 4 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 8 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |

Table 8.2.x.2-5: Minimum requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS FDD

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 4 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 8 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |

Table 8.2.x.2-6: Minimum requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS TDD

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 4 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 8 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |

Table 8.2.x.2-7: Minimum requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS FDD

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 4 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 8 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |

Table 8.2.x.2-8: Minimum requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS TDD

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 4 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |
|  | 8 | Normal | TDLA30-10 Low | 70% | G-FR1-Ax-y | pos0 | TBD |
|  |  |  |  |  |  | pos1 | TBD |

###################### End of change #1 ###############################

###################### Start of change #2 ###############################

#### 11.2.1.x	Requirements for PUSCH with DM-RS bundling

Apply the requirements for 2Rx defined in clause 8.2.x for 2Rx.

### 11.2.2	Requirements for BS type 2-O

###################### End of change #2 ###############################

###################### Start of change #3 ###############################

#### 11.2.2.x	Requirements for PUSCH with DM-RS bundling

##### 11.2.2.x.1	General

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ retransmissions.

Table 11.2.2.x.1-1: Test parameters for testing PUSCH with DM-RS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Example TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: DDSUU or DSUUU, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0,3 (Note 2) |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| pusch-TimeDomainWindowLength |  | 2 slots |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | Disabled |
| configuration | Time density (LPT-RS) | Disabled |
| NOTE 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pusch-TimeDomainWindowLength and PUSCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUSCH is only scheduled for the actual TDW including 2 consecutive UL slots. NOTE 2: The effective RV sequence is {0,2,3,1} with slot aggregation. |  |  |

##### 11.2.2.x.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput stated in the tables 11.2.2.x.2-1 and 11.2.2.x.2-2 at the given SNR for 1Tx. FRCs are defined in annex A. Unless stated otherwise, the MIMO correlation matrices for the gNB are defined in annex G for low correlation.

Table 11.2.2.x.2-1: Minimum requirements for PUSCH, TypeB, 50 MHz channel bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-75 | 70% | G-FR2-Ax-y | pos1 | TBD |

Table 11.2.2.x.2-2: Minimum requirements for PUSCH, TypeB, 50 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-75 | 70% | G-FR2-Ax-y | pos1 | TBD |

###################### End of change #3 ###############################