3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214802

Electronic Meeting, August 15 – 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.115-2 Manufacturer declarations for NR FR2 repeaters

Agenda item:	9.5.5.1.3

Document for:	Approval

# Introduction

In RAN4#103-e meeting, it was agreed that declarations can be made independently for DL and UL while same declaration table can be used for both DL and UL. A baseline for FR2 declarations was agreed, and suggested for further modifications, additions, deletions to the baseline agreement as necessary. The companies were encouraged to further discuss the at least following items and contribute [1]:

- Power declarations

- Declarations for contiguous and non-contiguous spectrum operation

- Table notes

- Group delay declaration

In this contribution, we focus on the above listed items.

# TP to TS 38.115-2

< Start of text proposal >

## 4.6	Manufacturer's declarations

The following repeater manufacturer's declarations listed in table 4.6-1, when applicable to the BS under test, are required to be provided by the manufacturer for radiated requirements testing for repeater type 2-O. Declarations can be made independently for UL and DL.

Table 4.6-1: Manufacturers declarations for repeater type 2-O radiated test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the repeater enclosure. |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the repeater enclosure. |
| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the repeater is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. (Note 1) |
| D.4 | Operating bands and passband frequency ranges | List of NR operating band(s) supported by the repeater and passband frequency range(s) within the operating band(s) that the repeater can operate in. Supported bands declared for every beam (D.3). (Note 2) |
| D.5 | Repeater class | Declared as Wide Area repeater, Medium Range repeater, or Local Area repeater. |
| D.6 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3). |
| D.7 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). |
| D.8 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). |
| D.9 | Rated beam EIRP | The rated EIRP level per passband (Prated,p,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.8), as well as the reference beam direction pair (D.8). Declared for every beam (D.3). (Note 5, 6, 7) |
| D.10 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D.3). |
| D.11 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D.4 – D.10) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. |
| D.12 | Parallel beams | List of beams which have been declared equivalent (D.11) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. |
| D.13 | OTA coverage range | Declared as a single range of directions within which selected TX OTA requirements are intended to be met. (Note 3) |
| D.14 | OTA coverage range reference direction | The direction describing the reference direction of the OTA coverage range (D.13). (Note 4) |
| D.15 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. |
| D.16 | The rated passband OTA repeater power, Prated,p,TRP | Prated,p,TRP is declared as TRP OTA power per passband, declared per supported operating band. (Note 5, 7) |
| D.17 | Rated transmitter TRP, Prated,t,TRP | Rated total radiated output power. Declared per supported operating band. (Note 5, 7) |
| D.18 | Spurious emission category | Declare the repeater spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. |
| D.19 | Additional operating band unwanted emissions | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.7.4 apply. |
| D.20 | Co-existence with other systems | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or PHS operating in another operating band are deployed. |
| [D.21 | Supported frequency range of the NR operating band | List of supported frequency ranges representing fractional bandwidths (FBW) of operating bands with FBW larger than 6%. |
| D.22 | Rated beam EIRP at lower end of the fractional bandwidth (Prated,out,FBWlow) | The rated EIRP level per passband at lower frequency range of the fractional bandwidth (Prated,out,FBWlow), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.6). Declared per beam for all supported frequency ranges (D.21). (Note 5, 6, 7) |
| D.23 | Rated beam EIRP at higher frequency range of the fractional bandwidth (Prated,out,FBWhigh) | The rated EIRP level per passband at higher frequency range of the fractional bandwidth (Prated,out,FBWhigh), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.6). Declared per beam for all supported frequency ranges in (D.21). (Note 5, 6, 7)] |
| D.24 | Long delay repeater | Declared only if the repeater internal delay between the input and output for this repeater does not fit within the TDD transient time. The repeater is intended for situations in which it will not cause interference to other nodes. This is achieved by RF isolation or by reservation of longer guard periods, which degrades frame utilization. The length of repeaters internal delay is declared using this declaration. |
| D.25 | Input signal EIRP for maximum output power | Declaration of input signal EIRP required to reach maximum output power. Declared per passband. |
| D.26 | Repeater radiating direction | Declaration on whether the repeater is intended to radiate in DL, UL or both. Testing shall be performed only for the direction(s) in which the repeater radiates. |
| NOTE 1: Depending on the capability of the system some of these beams may be the same. For those same beams, testing is not repeated. NOTE 2: These operating bands are related to their respective single-band RIBs. NOTE 3: OTA coverage range is used for conformance testing of such TX OTA requirements as frequency error or EVM. NOTE 4: The OTA coverage range reference direction may be the same as the Reference beam direction pair (D.8) but does not have to be. NOTE 5: If a Repeater type 2-O is capable of 64QAM operation but not capable of 256QAM operation, then up to two rated output power declarations may be made. One declaration is applicable when configured for 64QAM operation and the other declaration is applicable when not configured for 64QAM operation. NOTE 6: If D.22 and D.23 are declared for certain frequency range (D.21), there shall be no "Rated beam EIRP" declaration (D.9) for the operating band containing that particular frequency range. NOTE 7: If a BS type 2-O is capable of 256QAM operation, then up to three rated output power declarations may be made. One declaration is applicable when configured for 256QAM operation, a different declaration is applicable when configured for 64QAM operation and the other declaration is applicable when not configured neither for 256QAM nor 64QAM operation. |  |  |

## 4.7	Test configurations

< End of text proposal >

# References

- R4-2211151, WF on NR FR1/FR2 repeater declarations

- 3GPP TS 38.141-2, Base Station (BS) conformance testing Part 2: Radiated conformance testing