3GPP TSG-RAN4 Meeting #104-e	R4-2214837

# Electronic meeting, 15th Aug – 26th Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PUCCH JCE requirements for TS 38.104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The PUCCH requirement for format1 and format 3 with JCE have been introduced into Rel-17 Coverage Enhancement WI.There is no related requirement defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new section for PUCCH format 1 and format 3 requirement with JCE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-17 PUCCH requirement with JCE can not be verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.12, 8.3.13,11.3.1.8.11.3.1.9,13.2.6.7, 13.2.6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.141-1/38.141-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | Revsion of R4-221366 |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 8.3.12	Performance requirements for PUCCH format 1 with DM-RS bundling

#### 8.3.12.1	NACK to ACK requirements

##### 8.3.12.1.1	General

The NACK to ACK detection probability is the probability that an ACK bit is falsely detected when an NACK bit was sent on the particular bit position, where the NACK to ACK detection probability is defined as follows:

,

where:

-	denotes the total number of NACK bits transmitted

-	denotes the number of NACK bits decoded as ACK bits at the receiver, i.e. the number of received ACK bits

-	NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

The NACK to ACK detection probability performance requirement only apply to PUCCH format 1 with 2 UCI bits. The UCI information only contain ACK/NACK information.

The 2bits UCI information is further defined with bitmap as [0 1].

Table 8.3.12.1.1-1: Test Parameters

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Example TDD UL-DL pattern (Note1) | 15 kHz SCS: 7D1S1U, S=6D:4G:4U 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 15 kHz SCS: 7D1S1U, S=6D:4G:4U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| Number of information bits | 2 | 2 |
| Number of PRBs | 1 | 1 |
| Number of symbols | 14 | 14 |
| First PRB prior to frequency hopping | 0 | 0 |
| Intra-slot frequency hopping | diable | diable |
| Group and sequence hopping | neither | neither |
| Hopping ID | 0 | 0 |
| Initial cyclic shift | 0 | 0 |
| First symbol | 0 | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 | 0 |
| Number of slots for PUCCH repetition | 2 | 8 |
| PUCCH-TimeDomainWindowLength | 2 | 8 |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pucch-TimeDomainWindowLength and PUCCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUCCH is only scheduled for the actual TDW including 2 consecutive UL slots. |  |  |

##### 8.3.12.1.2	Minimum requirements

The NACK to ACK probability shall not exceed 0.1% at the SNR given in table 8.3.12.1.2-1 and table 8.3.12.1.2-2

Table 8.3.12.1.2-1: Minimum requirements for PUCCH format 1 with DMRS bundling, 15 kHz SCS, 5MHz channel bandwidth,

| Test number | Number of Tx antennas | Number of RX antennas | Cyclic-Prefix | Propagation conditions and correlation matrix (Annex G) | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | Normal | TDLA30-10 Low | TBD |
| 2 | 1 | 2 | Normal | TDLA30-10 Low | TBD |

Table 8.3.12.1.2-2: Minimum requirements for PUCCH format 1 with DMRS bundling, 30 kHz SCS, 10MHz channel bandwidth,

| Test number | Number of Tx antennas | Number of RX antennas | Cyclic-Prefix | Propagation conditions and correlation matrix (Annex G) | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | Normal | TDLA30-10 Low | TBD |
| 2 | 1 | 2 | Normal | TDLA30-10 Low | TBD |

#### 8.3.12.2	ACK missed detection requirements

##### 8.3.12.2.1	General

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent. The test parameters in table 8.3.12.1.1-1 are configured.

The ACK missed detection probability performance requirement only apply to PUCCH format 1 with 2 UCI bits. The UCI information only contain ACK/NACK information.

The 2bits UCI information is further defined with bitmap as [0 1].

##### 8.3.12.2.2	Minimum requirements

The ACK missed detection probability shall not exceed 1% at the SNR given in table 8.3.12.2.2-1 and table 8.3.12.2.2-2

Table 8.3.12.2.2-1: Minimum requirements for PUCCH format 1 with DMRS bundling, 15 kHz SCS, 5MHz channel bandwidth,

| Test number | Number of Tx antennas | Number of RX antennas | Cyclic-Prefix | Propagation conditions and correlation matrix (Annex G) | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | Normal | TDLA30-10 Low | TBD |
| 2 | 1 | 2 | Normal | TDLA30-10 Low | TBD |

Table 8.3.12.2.2-2: Minimum requirements for PUCCH format 1 with DMRS bundling, 30 kHz SCS, 10MHz channel bandwidth,

| Test number | Number of Tx antennas | Number of RX antennas | Cyclic-Prefix | Propagation conditions and correlation matrix (Annex G) | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | Normal | TDLA30-10 Low | TBD |
| 2 | 1 | 2 | Normal | TDLA30-10 Low | TBD |

### 8.3.13	Performance requirements for PUCCH format 3 with DMRS bundling

#### 8.3.13.1	General

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

Table 8.3.13.1-1: Test Parameters

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Eample TDD UL-DL pattern (Note1) | 15 kHz SCS: 7D1S1U, S=6D:4G:4U 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 15 kHz SCS: 7D1S1U, S=6D:4G:4U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| Modulation order | QPSK | QPSK |
| First PRB prior to frequency hopping | 0 | 0 |
| Intra-slot frequency hopping | disabled | disabled |
| Group and sequence hopping | neither | neither |
| Hopping ID | 0 | 0 |
| Number of PRBs | 1 | 1 |
| Number of symbols | 14 | 14 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Number of slots for PUCCH repetition | 2 | 8 |
| PUCCH-TimeDomainWindowLength | 2 | 8 |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pucch-TimeDomainWindowLength and PUCCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUCCH is only scheduled for the actual TDW including 2 consecutive UL slots. |  |  |

#### 8.3.13.2	Minimum requirements

The UCI block error probability shall not exceed 1% at the SNR given in Table 8.3.5.2-1 and Table 8.3.5.2-2.

Table 8.3.13.2-1: Minimum requirements for PUCCH format 3 with DMRS bundling, 15 kHz SCS, 5MHz channel bandwidth

| Test | Number of | Number of | Cyclic | Propagation | Additional | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Number | TX antennas | RX antennas | Prefix | conditions and correlation matrix (Annex G) | DM-RS configuration | 5 MHz |
| 1 | 1 | 2 | Normal | TDLC30-10 Low | No additional DM-RS | TBD |
|  |  |  |  |  | Additional DM-RS | TBD |
| 2 | 1 | 2 | Normal | TDLC30-10 Low | No additional DM-RS | TBD |
|  |  |  |  |  | Additional DM-RS | TBD |

Table 8.3.13.2-2: Minimum requirements for PUCCH format 3 with DMRS bundling, 30 kHz SCS, 10MHz channel bandwidth

| Test | Number of | Number of | Cyclic | Propagation | Additional | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Number | TX antennas | RX antennas | Prefix | conditions and correlation matrix (Annex G) | DM-RS configuration | 10 MHz |
| 1 | 1 | 2 | Normal | TDLC30-10 Low | No additional DM-RS | TBD |
|  |  |  |  |  | Additional DM-RS | TBD |
| 2 | `1 | 2 | Normal | TDLC30-10 Low | No additional DM-RS | TBD |
|  |  |  |  |  | Additional DM-RS | TBD |

<End of Change 1>

<Start of Change 2>

#### 11.3.1.8 Performance requirements for PUCCH format 1 with DMRS bundling

Apply the requirements defined in clause 8.3.12 for 2Rx.

#### 11.3.1.9 Performance requirements for PUCCH format 1 with DMRS bundling

Apply the requirements defined in clause 8.3.13 for 2Rx.

<End of Change 2>

<Start of Change 3>

### 11.3.2.7	Performance requirements for PUCCH format 1 with DMRS bundling

#### 11.3.2.7.1	NACK to ACK requirements

##### 11.3.2.7.1.1	General

The NACK to ACK detection probability is the probability that an ACK bit is falsely detected when an NACK bit was sent on the particular bit position, where the NACK to ACK detection probability is defined as follows:

,

where:

-	denotes the total number of NACK bits transmitted

-	denotes the number of NACK bits decoded as ACK bits at the receiver, i.e. the number of received ACK bits

-	NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

The NACK to ACK detection probability performance requirement only apply to PUCCH format 1 with 2 UCI bits. The UCI information only contain ACK/NACK information.

The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pucch-TimeDomainWindowLength and number of slots for PUCCH repetiton are configured as 2 slots.

[The UL slot is not scheduled and is not measured, on the actual TDW including only 1 UL slot.]

The 2bits UCI information is further defined with bitmap as [0 1].

Table 11.3.2.7.1.1-1: Test Parameters

### 11.3.2.7	Performance requirements for PUCCH format 1 with DMRS bundling

#### 11.3.2.7.1	NACK to ACK requirements

##### 11.3.2.7.1.1	General

The NACK to ACK detection probability is the probability that an ACK bit is falsely detected when an NACK bit was sent on the particular bit position, where the NACK to ACK detection probability is defined as follows:

,

where:

-	denotes the total number of NACK bits transmitted

-	denotes the number of NACK bits decoded as ACK bits at the receiver, i.e. the number of received ACK bits

-	NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

The NACK to ACK detection probability performance requirement only apply to PUCCH format 1 with 2 UCI bits. The UCI information only contain ACK/NACK information.

The 2bits UCI information is further defined with bitmap as [0 1].

Table 11.3.2.7.1.1-1: Test Parameters

| Parameter | Test |
| --- | --- |
| Example TDD UL-DL pattern (Note1) | 60 kHz SCS: DDSUU, S=10D:2G:2U or DSUUU 120 kHz SCS: DDSUU, S=10D:2G:2U, DSUUU or DSUUU |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | diable |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| PUCCH-TimeDomainWindowLength | 2 |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pucch-TimeDomainWindowLength and PUCCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUCCH is only scheduled for the actual TDW including 2 consecutive UL slots. |  |

##### 11.3.2.7.1.2	Minimum requirements

The NACK to ACK probability shall not exceed 0.1% at the SNR given in table 11.3.2.7.1.2-1 and table 11.3.2.7.1.2-2

Table 11.3.2.7.1.2-1: Minimum requirements for PUCCH format 1 with DMRS bundling, 60 kHz SCS, 50MHz channel bandwidth

| Number of Tx antennas | Number of demodulation branches | Cyclic-Prefix | Propagation conditions and correlation matrix (Annex G) | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-75 Low | TBD |

Table 11.3.2.7.1.2-2: Minimum requirements for PUCCH format 1 with DMRS bundling, 120 kHz SCS, 50MHz channel bandwidth

| Number of Tx antennas | Number of demodulation branches | Cyclic-Prefix | Propagation conditions and correlation matrix (Annex G) | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-75 Low | TBD |

#### 11.3.2.7.2	ACK missed detection requirements

##### 11.3.2.7.2.1	General

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent. The test parameters in table 11.3.2.7.2.1-1 are configured.

The ACK missed detection probability performance requirement only apply to PUCCH format 1 with 2 UCI bits. The UCI information only contain ACK/NACK information.

The 2bits UCI information is further defined with bitmap as [0 1].

##### 11.3.2.7.2.2	Minimum requirements

The ACK missed detection probability shall not exceed 1% at the SNR given in table 11.3.2.7.2.2-1 to table 11.3.2.7.2.2-2

Table 11.3.2.7.2.2-1: Minimum requirements for PUCCH format 1 with JCE, 60 kHz SCS, 50MHz channel bandwidth

| Test Number | Number of Tx antennas | Number of demodulation branches | Cyclic-Prefix | Propagation conditions and correlation matrix (Annex G) | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | Normal | TDLA30-75 Low | TBD |

Table 11.3.2.7.2.2-1: Minimum requirements for PUCCH format 1 with JCE, 120 kHz SCS, 100MHz channel bandwidth

| Test Number | Number of Tx antennas | Number of demodulation branches | Cyclic-Prefix | Propagation conditions and correlation matrix (Annex G) | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | Normal | TDLA30-75 Low | TBD |

### 11.3.2.8	Performance requirements for PUCCH format 3 with DMRS bunding

#### 11.3.2.8.1	General

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

Table 11.3.2.8.1-1: Test Parameters

| Parameter | Test 1 |
| --- | --- |
| Example TDD UL-DL pattern (Note 1) | 60 kHz SCS: DDSUU, S=10D:2G:2U or DSUUU 120 kHz SCS: DDSUU, S=10D:2G:2U or DSUUU |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Number of slots for PUCCH repetition | 2 |
| PUCCH-TimeDomainWindowLength | 2 |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pucch-TimeDomainWindowLength and PUCCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUCCH is only scheduled for the actual TDW including 2 consecutive UL slots. |  |

#### 11.3.2.8.2	Minimum requirements

The UCI block error probability shall not exceed 1% at the SNR given in Table 11.3.2.8.2-1 and Table 11.3.2.8.2-2.

Table 11.3.2.8.2-1: Minimum requirements for PUCCH format 3 with JCE, 60 kHz SCS, 50MHz channel bandwidth

| Test | Number of | Number of | Cyclic | Propagation | Additional | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Number | TX antennas | demodulation branches | Prefix | conditions and correlation matrix (Annex G) | DM-RS configuration | 50 MHz |
| 1 | 1 | 2 | Normal | TDLA30-75 Low | No additional DM-RS | TBD |
|  |  |  |  |  | Additional DM-RS | TBD |

Table 11.3.2.8.2-2: Minimum requirements for PUCCH format 3 with JCE, 120 kHz SCS, 50MHz channel bandwidth

| Test | Number of | Number of | Cyclic | Propagation | Additional | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Number | TX antennas | demodulation branches | Prefix | conditions and correlation matrix (Annex G) | DM-RS configuration | 50 MHz |
| 1 | 1 | 2 | Normal | TDLA30-75 Low | No additional DM-RS | TBD |
|  |  |  |  |  | Additional DM-RS | TBD |

<End of Change 3>