3GPP TSG-RAN4 Meeting #104-e	R4-2214891

# Electronic meeting, Aug. 15 – Aug. 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test set 2-4: PRS-RSRPP accuracy and report mapping in FR1 and FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - PRS-RSRPP measurement accuracy requirements and report mapping are defined in R17 ePOS WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce test set 2-4, PRS-RSRPP measurement accuracy requirements and report mapping in TS 38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The accuracy requirements and report mapping for the PRS-RSRPP measurement are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 10.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2211731 |  |  |  |  |  |  |  |  |

<Start of Change 1>

10.1.X	PRS-RSRPP Measurements

10.1.X.1	Introduction

The requirements in Clause 10.1.X shall apply, provided the UE has received nr-DL-AoD-RequestLocationInformation message from LMF via LPP [34] requesting the UE to report one or more DL PRS-RSRPP measurements defined in TS 38.215 [4]. The requirements in Clause 10.1.X shall apply:

- when UE is in RRC_CONNECTED state,

- when UE is in RRC_INACTIVE state.

The requirements in Clause 10.1.X apply for the first path PRS-RSRP measurement.

10.1.X.2	Measurement Accuracy Requirements

10.1.X.2.1	Absolute PRS RSRPP accuracy

The absolute accuracy requirements for PRS-RSRPP measurement for FR1 defined in Table 10.1.X.2.1-1 and Table 10.1.X.2.1-3 are valid under the following conditions:

-	Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.X for a corresponding Band

The absolute accuracy requirements for PRS-RSRPP measurement for FR2 defined in Table 10.1.X.2.1-2 and Table 10.1.X.2.1-4 are valid under the following conditions:

-	Conditions defined in 38.101-2 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.X for a corresponding Band

The absolute accuracy requirements for PRS-RSRPP measurement defined in Table 10.1.X.2.1-1 and Table 10.1.X.2.1-2 apply for the UE not supporting supportedDL-PRS-ProcessingSamples [34] or LMF does not indicate UE to perform positioning measurements with reduced number of samples.

The absolute accuracy requirements for PRS-RSRPP measurement defined in Table 10.1.X.2.1-3 and Table 10.1.X.2.1-4 apply for the UE supporting supportedDL-PRS-ProcessingSamples [34].

Note: The requriements in this clause are derived based on two-tap channel defined in 38.101-4 Annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz).

Table 10.1.X.2.1-1: PRS-RSRPP absolute accuracy for FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 8 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 | dBm/60kHz Note 6 |  |
| ±TBD | ±TBD | ≥-3 | TBD | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
|  |  |  |  |  | Note 4 |  |  |  |  |
|  |  |  |  |  | Note 4 |  |  |  |  |
| ±TBD | ±TBD |  | TBD |  |  |  |  |  |  |
| ±TBD | ±TBD | ≥-13 | TBD | All | Note 4 |  |  |  |  |
| ±TBD | ±TBD |  | TBD | All | Note 4 |  |  |  |  |
| ±TBD | ±TBD |  | TBD | All | Note 4 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ [24] RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 10.1.X.2.1-2: PRS-RSRPP absolute accuracy for FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |
|  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  | Maximum Io |
| dB | dB | dB | PRB | - | dBm / SCSPRS |  | dBm/BWChannel |
|  |  |  |  |  | dBm/120kHz Note 6 | dBm/60kHz Note 6 |  |
| ±TBD | ±TBD | ≥-3 | TBD | All | Same value as PRP in Table B.2.X-2, according to UE Power class, operating band and angle of arrival |  | -50 |
|  |  |  |  |  | Note 4 |  |  |
|  |  |  |  |  | Note 4 |  |  |
| ±TBD | ±TBD | ≥-13 | TBD | All | Note 4 |  |  |
| ±TBD | ±TBD |  | TBD | All | Note 4 |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ [24] RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

Table 10.1.X.2.1-3: PRS-RSRPP absolute accuracy for FR1 for reduced number of samples

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 8 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 | dBm/60kHz Note 6 |  |
| ±TBD | ±TBD | ≥TBD | TBD | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
|  |  |  |  |  | Note 4 |  |  |  |  |
|  |  |  |  |  | Note 4 |  |  |  |  |
| ±TBD | ±TBD |  | TBD |  |  |  |  |  |  |
| ±TBD | ±TBD | ≥[-6] | TBD | All | Note 4 |  |  |  |  |
| ±TBD | ±TBD |  | TBD | All | Note 4 |  |  |  |  |
| ±TBD | ±TBD |  | TBD | All | Note 4 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ [24] RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 10.1.X.2.1-4: PRS-RSRPP absolute accuracy for FR2 for reduced number of samples

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |
|  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  | Maximum Io |
| dB | dB | dB | PRB | - | dBm / SCSPRS |  | dBm/BWChannel |
|  |  |  |  |  | dBm/120kHz Note 6 | dBm/60kHz Note 6 |  |
| ±TBD | ±TBD | ≥TBD | TBD | All | Same value as PRP in Table B.2.X-2, according to UE Power class, operating band and angle of arrival |  | -50 |
|  |  |  |  |  | Note 4 |  |  |
|  |  |  |  |  | Note 4 |  |  |
| ±TBD | ±TBD | ≥[-6] | TBD | All | Note 4 |  |  |
| ±TBD | ±TBD |  | TBD | All | Note 4 |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ [24] RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

10.1.X.3	Report mapping

10.1.X.3.1	Absolute PRS-RSRPP Measurement Report Mapping

The reporting range of absolute PRS-RSRPP measurement is defined from -156 dBm to -31 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.X.3.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.X.3.1-1: Measurement report mapping for PRS-RSRPP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| PRS_RSRPP_0 | PRS-RSRPP<-156 | dBm |
| PRS_RSRPP_1 | -156≤PRS-RSRPP<-155 | dBm |
| PRS_RSRPP_2 | -155≤PRS-RSRPP<-154 | dBm |
| PRS_RSRPP_3 | -154≤PRS-RSRPP<-153 | dBm |
| PRS_RSRPP_4 | -153≤PRS-RSRPP<-152 | dBm |
| PRS_RSRPP_5 | -152≤PRS-RSRPP<-151 | dBm |
| PRS_RSRPP_6 | -151≤PRS-RSRPP<-150 | dBm |
| PRS_RSRPP_7 | -150≤PRS-RSRPP<-149 | dBm |
| PRS_RSRPP_8 | -149≤PRS-RSRPP<-148 | dBm |
| PRS_RSRPP_9 | -148≤PRS-RSRPP<-147 | dBm |
| PRS_RSRPP_10 | -147≤PRS-RSRPP<-146 | dBm |
| PRS_RSRPP_11 | -146≤PRS-RSRPP<-145 | dBm |
| PRS_RSRPP_12 | -145≤PRS-RSRPP<-144 | dBm |
| PRS_RSRPP_13 | -144≤PRS-RSRPP<-143 | dBm |
| PRS_RSRPP_14 | -143≤PRS-RSRPP<-142 | dBm |
| PRS_RSRPP_15 | -142≤PRS-RSRPP<-141 | dBm |
| PRS_RSRPP_16 | -141≤PRS-RSRPP<-140 | dBm |
| PRS_RSRPP_17 | -140≤PRS-RSRPP<-139 | dBm |
| PRS_RSRPP_18 | -139≤PRS-RSRPP<-138 | dBm |
| … | … | … |
| PRS_RSRPP_111 | -46≤PRS-RSRPP<-45 | dBm |
| PRS_RSRPP_112 | -45≤PRS-RSRPP<-44 | dBm |
| PRS_RSRPP_113 | -44≤PRS-RSRPP<-43 | dBm |
| PRS_RSRPP_114 | -43≤PRS-RSRPP<-42 | dBm |
| PRS_RSRPP_115 | -42≤PRS-RSRPP<-41 | dBm |
| PRS_RSRPP_116 | -41≤PRS-RSRPP<-40 | dBm |
| PRS_RSRPP_117 | -40≤PRS-RSRPP<-39 | dBm |
| PRS_RSRPP_118 | -39≤PRS-RSRPP<-38 | dBm |
| PRS_RSRPP_119 | -38≤PRS-RSRPP<-37 | dBm |
| PRS_RSRPP_120 | -37≤PRS-RSRPP<-36 | dBm |
| PRS_RSRPP_121 | -36≤PRS-RSRPP<-35 | dBm |
| PRS_RSRPP_122 | -35≤PRS-RSRPP<-34 | dBm |
| PRS_RSRPP_123 | -34≤PRS-RSRPP<-33 | dBm |
| PRS_RSRPP_124 | -33≤PRS-RSRPP<-32 | dBm |
| PRS_RSRPP_125 | -32≤PRS-RSRPP<-31 | dBm |
| PRS_RSRPP_126 | -31≤PRS-RSRPP | dBm |

10.1.X.3.2	Differential Report Mapping for PRS-RSRPP Measurement

The reporting range of differential PRS-RSRPP is defined from -30 dB to 0 dB with 1 dB resolution when nr-DL-AoD-RequestLocationInformation message is received.

The mapping of measured quantity is defined in Table 10.1.X.3.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

For differential reporting, PRS-RSRPP is reported as the difference in dB with respect to the first reported PRS-RSRPP.

Table 10.1.X.3.2-1: Measurement report mapping for differential PRS-RSRPP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| DIFFRSRPP_0 | -30≥ΔRSRPP | dB |
| DIFFRSRPP_1 | -29≥ΔRSRPP>-30 | dB |
| DIFFRSRPP_2 | -28≥ΔRSRPP>-29 | dB |
| DIFFRSRPP_3 | -27≥ΔRSRPP>-28 | dB |
| DIFFRSRPP_4 | -26≥ΔRSRPP>-27 | dB |
| DIFFRSRPP_5 | -25≥ΔRSRPP>-26 | dB |
| DIFFRSRPP_6 | -24≥ΔRSRPP>-25 | dB |
| DIFFRSRPP_7 | -23≥ΔRSRPP>-24 | dB |
| DIFFRSRPP_8 | -22≥ΔRSRPP>-23 | dB |
| DIFFRSRPP_9 | -21≥ΔRSRPP>-22 | dB |
| DIFFRSRPP_10 | -20≥ΔRSRPP>-21 | dB |
| DIFFRSRPP_11 | -19≥ΔRSRPP>-20 | dB |
| DIFFRSRPP_12 | -18≥ΔRSRPP>-19 | dB |
| DIFFRSRPP_13 | -17≥ΔRSRPP>-18 | dB |
| DIFFRSRPP_14 | -16≥ΔRSRPP>-17 | dB |
| DIFFRSRPP_15 | -15≥ΔRSRPP>-16 | dB |
| DIFFRSRPP_16 | -14≥ΔRSRPP>-15 | dB |
| DIFFRSRPP_17 | -13≥ΔRSRPP>-14 | dB |
| DIFFRSRPP_18 | -12≥ΔRSRPP>-13 | dB |
| DIFFRSRPP_19 | -11≥ΔRSRPP>-12 | dB |
| DIFFRSRPP_20 | -10≥ΔRSRPP>-11 | dB |
| DIFFRSRPP_21 | -9≥ΔRSRPP>-10 | dB |
| DIFFRSRPP_22 | -8≥ΔRSRPP>-9 | dB |
| DIFFRSRPP_23 | -7≥ΔRSRPP>-8 | dB |
| DIFFRSRPP_24 | -6≥ΔRSRPP>-7 | dB |
| DIFFRSRPP_25 | -5≥ΔRSRPP>-6 | dB |
| DIFFRSRPP_26 | -4≥ΔRSRPP>-5 | dB |
| DIFFRSRPP_27 | -3≥ΔRSRPP>-4 | dB |
| DIFFRSRPP_28 | -2≥ΔRSRPP>-3 | dB |
| DIFFRSRPP_29 | -1≥ΔRSRPP>-2 | dB |
| DIFFRSRPP_30 | 0≥ΔRSRPP>-1 | dB |

<End of Change 1>