3GPP TSG-RAN4 Meeting #104-e		R4-2215004

# Electronic Meeting, Aug 15 - Aug 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.133: Correction to conditional PSCell addition requirements (Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the conditional measurement event of CPA is triggered, the conditional PSCell addition would be directly initiated. UE is not required to report SSB based RRM measurement result related to MO for CPA. SSB index detection is not necessary from measurement report perspective as in section 9 for inter-frequency measurement. Therefore, it is unreasonable to split the measurement delay of CPA into two scenarios, i.e., with acquiring the index of the SSB or without acquiring the index of the SSB. However, SSB index needs to be known for target PSCell addition. Therefore, the time for CPA measurement needs to include SSB index detection time. Tmeasure in CPA should be Tidentify_inter_with_index. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete the case in which UE needs not to acquire the index of the SSB in measurement time in conditional PSCell addition requirement. - For inter-frenquency measurement, delete Tidentify_inter_without_index in measurment time in conditional PSCell addition requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-17 conditional PSCell additional requirements are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of change 1>

Conditional PSCell Addition Delay

8.9A.1	Introduction

This clause defines requirements for the delay within which the UE shall be able to perform conditional PSCell addition in EN-DC or NR-DC. The requirements in this clause are applicable to EN-DC and NR-DC.

8.9A.2	Conditional PSCell Addition Delay Requirement

The requirements in this clause shall apply for the UE configured with only PCell in FR1.

Upon receiving conditional PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell no later than in subframe n + Tconfig_PSCell_Addition_Conditional:

Where:

Tconfig_PSCell_Addition_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

TRRC_delay is the RRC processing delay defined in Clause 11.2 in 36.331 [16] which is the corresponding RRC message embedded in E-UTRA RRC message, otherwise it is the RRC procedure delay defined in clause 12 in TS 38.331 [2] for processing the conditional PSCell addition command.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional PSCell addition command until a condition exists at the measurement reference point which will trigger the conditional PSCell addition.

Tmeasure is the measurements time stated in clause 8.9A.2.1.

TUE_preparation is the UE preparation time for conditional PSCell addition, and starts after UE realizes the condition of PSCell addition is met and identity of the PSCell is determined. TUE_preparation is up to 10 ms.

Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20 ms when PSCell is in FR1, and Tprocessing = 40 ms when PSCell is in FR2.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms.

TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

The PCell interruption specified in clause 8.2 is allowed only after the UE starts to execute a conditional PSCell addition.

8.9A.2.1	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a PSCell addition and interruption time starts.

The measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index defined in clause 9.3.4. When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_inter_with_index for PSCell addition. If a cell, which has been detectable at least for the time period Tidentify_inter_with_index for PSCell addition, becomes undetectable for a period and then the cell becomes detectable again and triggers a PSCell addition, the measurement time delay shall be less than TSSB_measurement_period_inter provided the timing to that cell has not changed more than  3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

# <End of change 1>