3GPP TSG-RAN WG4 Meeting #104-e	R4-2215006

Electronic Meeting, 15 August – 26 August 2022

Source:	Ericsson, Verizon

Title:	TP for TR 36.718-02-01: CA_48-66

Agenda item:			12.1.3.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 36.718-02-01 to add dual UL for existing CA_48-66 combinations according to the request in [1].

# 2. Reference

- RP-221831, New WID: Rel-18 LTE Advanced Carrier Aggregation for x bands (x<= 6) DL with y bands (y=1, 2) UL, Huawei, HiSilicon

# 3. Text Proposal

---Start of changes---

## 5.3.x	CA_48-66

### 5.3.x.1	Channel bandwidths per operating band for CA

Table 5.2.1-1: E-UTRA CA configurations and bandwidth combination sets

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_48A-66A | CA_48A-66A | 48 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_48A-48A-66A | CA_48A-66A | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_48C-66A | CA_48A-66A | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_48D-66A | CA_48A-66A | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_48A-66A-66A | CA_48A-66A | 48 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_48C-66A-66A | CA_48A-66A | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_48D-66A-66A | CA_48A-66A | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  | 100 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_48E-66A-66A | CA_48A-66A | 48 | See CA_48E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  | 120 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

### 5.3.x.2	Co-existence studies

For 2UL / 2DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.3.x.2-1.

Table 5.3.x.2-1: Harmonic and IMD analysis

| UE UL carriers |  | fx_low |  | fx_high |  | fy_low |  | fy_high |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) |  | 1710 |  | 1780 |  | 3550 |  | 3700 |
| 2nd harmonics frequency limits |  | 2*fx_low |  | 2*fx_high |  | 2* fy_low |  | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 |  | 7100 |  | 7400 |  |
| 3rd harmonics frequency limits |  | 3*fx_low |  | 3*fx_high |  | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) |  | 5130 |  | 5340 |  | 10650 |  | 11100 |
| Two tone 2nd order IMD products |  | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ |  | ¦fy_low + fx_low¦ |  | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) |  | 1990 |  | 1770 |  | 5260 |  | 5480 |
| Two-tone 3rd order IMD products |  | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ |  | ¦2*fy_low – fx_high¦ |  | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) |  | 280 |  | 10 |  | 5320 |  | 5690 |
| Two-tone 3rd order IMD products |  | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ |  | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) |  | 6970 |  | 7260 |  | 8810 |  | 9180 |
| Two-tone 4th order IMD products |  | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ |  | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) |  | 3980 |  | 3540 |  | 10520 |  | 10960 |
| Two-tone 4th order IMD products |  | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ |  | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) |  | 1430 |  | 1790 |  | 8870 |  | 9390 |
| Two-tone 4th order IMD products |  | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) |  | 8680 |  | 9040 |  | 12360 |  | 12880 |
| Two-tone 5th order IMD products |  | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ |  | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) |  | 13090 |  | 12420 |  | 3570 |  | 3140 |
| Two-tone 5th order IMD products |  | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ |  | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) |  | 15910 |  | 16580 |  | 10390 |  | 10820 |
| Two-tone 5th order IMD products |  | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ |  | ¦2*fy_low – 3*fx_high¦ |  | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) |  | 7680 |  | 7090 |  | 1760 |  | 2270 |
| Two-tone 5th order IMD products |  | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ |  | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) |  | 14070 |  | 14660 |  | 12230 |  | 12740 |

Table 5.3.x.2-2: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |  |
| CA_48-66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |

### 5.3.x.3	∆TIB and ∆RIB values

Already included in TS 36.101.

### 5.3.x.4	REFSENS requirements

Based on co-existence analysis in 5.3.x.2 there are IMD5 into band 66 and 2nd harmonic into band 48.

MSD due to IMD5 to be added in Table 7.3.1A-0f in TS 36.101 as in Table 5.3.x.4-1 below:

Table 5.3.x.4-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_48-66 | 48 | 3660 | 5 | 25 | 3660 | N/A | TDD FDD | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | 5.0 |  | IMD5 |

Exceptions due to harmonics for CA_48-66 is already defined in TS 36.101.

Harmonic mixing due to 5th order DL 66 into 3rd order UL 48 can occur but is not specified.

---End of changes---