3GPP TSG-RAN WG4 Meeting #104-e	R4-2215012

Electronic Meeting, 15 August – 26 August 2022

Source:	Ericsson, Telstra

Title:	TP for TR 37.718-21-11: Including band combinations DC_3A-26A_n78A

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-21-11 to add DC_3A-26A_n78A and DC_3C-26A_n78A.

# 2. Text Proposal

---Start of changes---

## 5.x	DC_3-26_n78

### 5.x.1	Operating bands for EN-DC

Table 5.x.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-26_n78 | CA_3-26 | n78 | No |

### 5.x.2	Configuration for DC

Table 5.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-26A_n78A DC_3C-26A_n78A | DC_3A_n78A DC_26A_n78A | CA_3A-26A | n78A |

### 5.x.3		∆TIB and ∆RIB values

For DC_3-26_n78, the TIB,c and RIB,c values are reused from DC_3-5_n78 and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-26_n78 | 3 | 0.6 |
|  | 26 | 0.6 |
|  | n78 | 0.8 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-26_n78 | 3 | 0.2 |
|  | 26 | 0.2 |
|  | n78 | 0.5 |

### 5.x.4	REFSENS requirements

There are IMD3 impact from UL 26_n78 affecting DL band 3. MSD values are reused from DC_5A_n3A-n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-26A_n78A DC_3C-26A_n78A | 3 | 1767 | 5 | 25 | 1862 | 15.7 | IMD3 |
|  | 26 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-221832, New WID: Rel-18 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon