3GPP TSG-RAN4 Meeting #104-e	R4-2215032

# Electronic Meeting, 15-26 August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR set 2-1 to 38.133 RSTD accuracy and report mapping in FR1 and FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To implement measurement accuracy and report mapping for RSTD measurement for Rel. 17 features: reduced number of samples for PRS measurement, TEG based PRS measurement, and gapless PRS measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on work split document agreed in RAN4#103e, accuracy requirement and measurement report mapping for RSTD measurement are implemented. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Accuracy requirement for reduced number of samples is not clear. RSTD measurement report mapping for TEG based measurement and gapless measurement is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.23.1, 10.1.23.2 and 10.1.23.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------START OF CHANGE 1--------------------------------

### 10.1.23	RSTD Measurements

#### Introduction

The requirements in Clause 10.1.23 shall apply, provided the UE has received nr-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] requesting the UE to report one or more DL RSTD measurements defined in TS 38.215 [4].  The requirements in Clause 10.1.23 shall apply:

- when UE is in RRC_CONNECTED state and the measurement is performed with MG or without MG,

- when UE is in RRC_INACTIVE state.

#### 10.1.23.2	Measurement Accuracy Requirements

The accuracy requirements for RSTD measurement shall be within ±(X+Y) Tc.

X is defined in Table 10.1.23.2-1 for AWGN channel and Table 10.1.23.2-3 for fading channel for FR1, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

-    UE does not perform positioning measurement with reduced number of samples.

X is defined in Table 10.1.23.2-2 for AWGN channel and Table 10.1.23.2-4 for fading channel for FR2, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

- 	UE does not perform positioning measurement with reduced number of samples.

X is defined in Table 10.1.23.2-5 for AWGN channel in FR1 provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

- 	UE supports positioning measurement with reduced number of sample and is indicated by LMF to perform positioning measurement with reduced number of samples.

X is defined in Table 10.1.23.2-6 for AWGN channel in FR2 provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

- 	UE supports positioning measurement with reduced number of sample and is indicated by LMF to perform positioning measurement with reduced number of samples.

Note: The requirements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2 respectively.

When UE measures RSTD on PRS resources belonging to different PFLs, then the RSTD accuracy is defined as the accuracy corresponding to the largest accuracy value among different PFLs.

When UE measures RSTD on PRS resources belonging to same PFL, Y=32 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 160 ms.

When UE measures RSTD on PRS resources belonging different PFLs, Y=[256] Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than [1280] ms.

[Editor notes: The margins for measurements on different PFLs shall be considered in the group delay margin]

Editor’s Note: FFS whether and how to form the accuracy numbers considering enhanced requirements in future releases, e.g. capturing margin values in separate tables.

Table 10.1.23.2-1: RSTD absolute accuracy in FR1 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| [252] +ΔNote 7 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
| [170] +Δ |  |  | ≥ [52] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [78] +Δ |  |  | ≥ [104] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [147] +Δ |  | 30 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -117.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -117 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -115.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -114.5 | -50 |
| [84] +Δ |  |  | ≥ [48] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [40] +Δ |  |  | ≥ [132] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [86] +Δ |  | 60 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -114.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -114 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -113.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -113 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -111.5 | -50 |
| [40] +Δ |  |  | ≥ [64] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [22] +Δ |  |  | ≥ [132] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Δ=TBD. |  |  |  |  |  |  |  |

Table 10.1.23.2-2: RSTD absolute accuracy in FR2 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| [107] +ΔNote 6 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ [24] | ≥ [4] | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| [56] +Δ |  |  | ≥ [64] | ≥ [1] | Note 5 | Note 5 |
| [27] +Δ |  |  | ≥ [132] | ≥ [1] | Note 5 | Note 5 |
| [56] +Δ |  | 120 | ≥ [32] | ≥ [4] | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| [29] +Δ |  |  | ≥ [64] | ≥ [1] | Note 5 | Note 5 |
| [18] +Δ |  |  | ≥ [128] | ≥ [1] | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Δ=TBD. |  |  |  |  |  |  |

Table 10.1.23.2-3: RSTD absolute accuracy in FR1 for fading channel

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| [367] +ΔNote 7 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
| [212] +Δ |  |  | ≥ [52] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [122] +Δ |  |  | ≥ [104] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [190] +Δ |  | 30 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -117.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -117 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -115.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -114.5 | -50 |
| [145] +Δ |  |  | ≥ [48] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [44] +Δ |  |  | ≥ [132] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [183] +Δ |  | 60 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -114.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -114 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -113.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -113 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -111.5 | -50 |
| [43] +Δ |  |  | ≥ [64] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [33] +Δ |  |  | ≥ [132] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Δ=TBD. |  |  |  |  |  |  |  |

Table 10.1.23.2-4:  RSTD absolute accuracy in FR2 for fading channel

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| [155] +ΔNote 6 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ [24] | ≥ [4] | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| [96] +Δ |  |  | ≥ [64] | ≥ [1] | Note 5 | Note 5 |
| [62] +Δ |  |  | ≥ [132] | ≥ [1] | Note 5 | Note 5 |
| [80] +Δ |  | 120 | ≥ [32] | ≥ [4] | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| [70] +Δ |  |  | ≥ [64] | ≥ [1] | Note 5 | Note 5 |
| [48] +Δ |  |  | ≥ [128] | ≥ [1] | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Δ=TBD. |  |  |  |  |  |  |

Table 10.1.23.2-5: RSTD absolute accuracy in FR1 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| [170] +ΔNOTE 7 | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 15 | ≥ [52] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [78] +Δ |  |  | ≥ [104] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [84] +Δ |  | 30 | ≥ [48] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [40] +Δ |  |  | ≥ [132] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [40] +Δ |  | 60 | ≥ [64] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [22] +Δ |  |  | ≥ [132] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Δ=TBD. |  |  |  |  |  |  |  |

Table 10.1.23.2-6: RSTD absolute accuracy in FR2 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| [56] +ΔNOTE 6 | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 60 | ≥ [64] | ≥ [1] | Note 5 | Note 5 |
| [27] +Δ |  |  | ≥ [132] | ≥ [1] | Note 5 | Note 5 |
| [29] +Δ |  | 120 | ≥ [64] | ≥ [1] | Note 5 | Note 5 |
| [18] +Δ |  |  | ≥ [128] | ≥ [1] | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Δ=TBD. |  |  |  |  |  |  |

--------------------------------END OF CHANGE 1--------------------------------

--------------------------------START OF CHANGE 2--------------------------------

##### 10.1.23.3.3	Additional Path Report Mapping for DL RSTD

The reporting range for the additional path reporting for an RSTD measurement is defined up to the range from -8175Tc to 8175Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin=[2] and kmax=5, when configured PRS resource of at least one of the reference cell and neighbor cell measured for the RSTD measurement is in FR1,

kmin=0 and kmax=5, when configured PRS resource of both the reference cell and neighbor cell measured for the RSTD measurement are in FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the RSTD measurement.

The UE can report the timing of up to two additional paths with respect to the path timing determining the RSTD measurement.

A UE capable of  additionalPathsExtSupport-r17 can report the timing for a number additional paths, up to its capability, with respect to the path timing determining the RSTD measurement.

The report mappings for different k values are specified in Tables 10.1.23.3.3-1  10.1.23.3.3-6.

Table 10.1.23.3.3-1: Report mapping for k=0

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174 | Tc |
| path_00002 | -8174 path < -8173 | Tc |
|  |  | … |
| path_08175 | -1 path < 0 | Tc |
| path_08176 | 0 path < 1 | Tc |
| … | … | … |
| path_ 16349 | 8173 path < 8174 | Tc |
| path_ 16350 | 8174 path < 8175 | Tc |
| path_ 16351 | 8175 path | Tc |

Table 10.1.23.3.3-2: Report mapping for k=1

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8175 | Tc |
| path_0001 | -8175 path < -8173 | Tc |
| path_0002 | -8173 path < -8171 | Tc |
|  |  | … |
| path_4088 | -1 path < 1 | Tc |
| … | … | … |
| path_8174 | 8171 path < 8173 | Tc |
| path_8175 | 8173 path < 8175 | Tc |
| path_8176 | 8175 path | Tc |

Table 10.1.23.3.3-3: Report mapping for k=2

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8174 | Tc |
| path_0001 | -8174 path < -8170 | Tc |
| path_0002 | -8170 path < -8166 | Tc |
|  |  | … |
| path_2044 | -2 path < 2 | Tc |
| … | … | … |
| path_4086 | 8166 path < 8170 | Tc |
| path_4087 | 8170 path < 8174 | Tc |
| path_4088 | 8174 path | Tc |

Table 10.1.23.3.3-4: Report mapping for k=3

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8172 | Tc |
| path_0001 | -8172 path < -8164 | Tc |
| path_0002 | -8164 path < -8156 | Tc |
|  |  | … |
| path_1022 | -4 path < 4 | Tc |
| … | … | … |
| path_2042 | 8156 path < 8164 | Tc |
| path_2043 | 8164 path < 8172 | Tc |
| path_2044 | 8172 path | Tc |

Table 10.1.23.3.3-5: Report mapping for k=4

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8168 | Tc |
| path_0001 | -8168 path < -8152 | Tc |
| path_0002 | -8152 path < -8136 | Tc |
|  |  | … |
| path_511 | -8 path < 8 | Tc |
| … | … | … |
| path_1020 | 8136 path < 8152 | Tc |
| path_1021 | 8152 path < 8168 | Tc |
| path_1022 | 8168 path | Tc |

Table 10.1.23.3.3-6: Report mapping for k=5

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000 | path < -8160 | Tc |
| path_001 | -8160 path < -8128 | Tc |
| path_002 | -8128 path < -8096 | Tc |
|  |  | … |
| path_256 | 0 path < 32 | Tc |
| … | … | … |
| path_509 | 8096 path < 8128 | Tc |
| path_510 | 8128 path < 8160 | Tc |
| path_511 | 8160 path | Tc |

--------------------------------END OF CHANGE 2--------------------------------