3GPP TSG-RAN4 Meeting #104-e	R4-2215047

# Electronic Meeting, August 15 – August 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Cases for Scell activation and deactivation for extending NR operation to 71GHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2022-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current specification has not inlcuded test cases on Scell activation and deactivation for 71GHz |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test cases on test cases on Scell activation and deactivation for 71GHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test cases on Scell activation and deactivation for 71GHz |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revised from R4-2213348 |  |  |  |  |  |  |  |  |

# Start of Change 1

# A.14	NR standalone tests with one or more NR cells in FR2-2

## A.14.X	Signaling characteristics

### A.14.X.3	SCell Activation and Deactivation Delay

#### A.14.X.3.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX

##### A.14.X.3.1.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.6.5.3.1.1 except the PCell and SCell are in FR2-2 intra-band.

The supported test configurations are shown in table A.14.X.3.1.1-1 below. The general test parameters are the same as defined in Table A.6.5.3.1.1-2 except those described in Tables A.14.X.3.1.1-2, and cell specific test parameters are described in Tables A.14.X.3.1.1-3. OTA related test parameters are shown in table A.14.X.3.1.1-4 below.

Table A.14.X.3.1.1-1: Supported test configurations for FR2-2 SCell activation case

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | NR 480 kHz SSB SCS, 400MHz bandwidth, TDD duplex mode |
| 3 | NR 960 kHz SSB SCS, 400MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.14.X.3.1.1-2: General test parameters for FR2-2 SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channels are used for this test, cell 1 and cell2 use RF channel 1 and 2, respectively. |

Table A.14.X.3.1.1-3: Cell specific test parameters for FR2-2 SCell activation case

| ParameterNote 5 |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| SSB ARFCN |  |  | freq1 |  |  | freq2 |  |  |
| Duplex mode | Config 1, 2, 3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1, 2, 3 |  | TDDConf.3.1 |  |  |  |  |  |
| Downlink initial BWP Configuration | Config 1, 2, 3 |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP Configuration | Config 1, 2, 3 |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration | Config 1, 2, 3 |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state | Config 1, 2, 3 |  | TCI.State.0 |  |  |  |  |  |
| BWchannel | Config 1, 2, 3 | MHz | 100: NRB,c = 66 |  |  |  |  |  |
| Data RBs allocated | Config 1, 2, 3 |  | 66 |  | 66 |  | 66 |  |
| PDSCH Reference measurement channel | Config 1, 2, 3 |  | SR.3.1 TDD |  |  | - |  |  |
| RMSI CORESET Parameters | Config 1, 2, 3 |  | CR.3.1 TDD |  |  | - |  |  |
| Dedicated CORESET Parameters | Config 1, 2, 3 |  | CCR.3.1 TDD |  |  | - |  |  |
| OCNG Patterns | Config 1, 2, 3 |  | OP.1 |  |  |  |  |  |
| SSB Configuration | Config 1, 2, 3 |  | SSB.1 FR2-2 |  |  |  |  |  |
| SMTC Configuration | Config 1, 2, 3 |  | SMTC.1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting | Config 1, 2, 3 |  | CSI-RS.3.1 TDD |  |  |  |  |  |
| reportConfigType | Config 1, 2, 3 |  | periodic |  |  | N/A |  |  |
| reportQuantity | Config 1, 2, 3 |  | cri-RI-PMI-CQI |  |  | N/A |  |  |
| CSI reporting periodicity | Config 1, 2, 3 | slot | 40 |  |  | N/A |  |  |
| CSI reporting offset | Config 1, 2, 3 | slot | 4 |  |  | N/A |  |  |
| EPRE ratio of PSS to SSS | Config 1, 2, 3 | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Propagation conditions | Config 1, 2, 3 |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: Void |  |  |  |  |  |  |  |  |

Table A.14.X.3.1.1-4: OTA related test parameters for FR2-2 SCell activation case

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration | Config 1, 2 ,3 |  | Setup 1 according to table A.3.15.1 |  |  | Setup 1 according to table A.3.15.1 |  |  |
| Assumption for UE beams Note 4 | Config 1, 2 ,3 |  | Rough |  |  | Rough |  |  |
| Note1 |  | dBm/15kHzNote3 | -104.7 |  |  | -104.7 |  |  |
| Note1 | Config 1 | dBm/SCS | -95.7 |  |  | -95.7 |  |  |
|  | Config 2 |  | -95.7 |  |  | -95.7 |  |  |
|  | Config 3 |  | -92.7 |  |  | -92.7 |  |  |
|  |  | dB | 7 |  |  | 7 |  |  |
| SSB_RPNote2 | Config 1 | dBm/SCS Note4 | -88.7 |  |  | -88.7 |  |  |
|  | Config 2 |  | -88.7 |  |  | -88.7 |  |  |
|  | Config 3 |  | -85.7 |  |  | -85.7 |  |  |
|  |  | dB | 7 |  |  | 7 |  |  |
| IoNote2 | Config 1 | dBm/95.04 MHz Note3 | -58.92 |  |  | -58.92 |  |  |
|  | Config 2 | dBm/380.16 MHz | -58.92 |  |  | -58.92 |  |  |
|  | Config 3 | dBm/380.16 MHz | -58.93 |  |  | -58.93 |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

##### A.14.X.3.1.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB + 5ms as defined in clause 8.3.

#### A.14.X.3.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2

##### A.14.X.3.2.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.14.X.3.1.1 except the PCell is in FR1 and SCell is in FR2-2.

The supported test configurations are defined in Table A.14.X.3.2.1-1. The general test parameters are the same as defined in Table A.6.5.3.1.1-2 except that the length of T2 is 2s. And cell specific test parameters are described in Tables A.14.X.3.2.1-2. OTA related test parameters are defined in Table A.14.X.3.2.1-3.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on NR. During T1 the SCell is powered off and UE is not aware of SCell.

A MAC message for activation of SCell is sent by the test equipment 100ms after the RRC message, in a slot # denoted m. The point in time at which the MAC message for activation of SCell is received at the UE antenna connector defines the start of time period T2.

During T2, the test equipment monitors the L1-RSRP measurement reporting for the SCell. The time when test equipment receives a valid L1-RSRP report is denoted as slot m+TL1-RSRP. In the next DL slot after slot m+TL1-RSRP, the test equipment sends a MAC message for the activation of the TCI state of the RMC CORESET of the SCell. In the same slot, the test equipment also sends an RRC message to configure the CSI-RS resources for SCell.

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PCell during activation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell1 deactivation command is sent until CSI reporting for SCell1 is discontinued.

Table A.14.X.3.2.1-1: Supported test configurations for FR2-2 SCell activation case

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 3 | PCell: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 4 | PCell: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target SCell: 480 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 5 | PCell: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target SCell: 960 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

Table A.14.X.3.2.1-2: Cell specific test parameters for FR2-2 SCell activation case

| ParameterNote 5 |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  |  |  | T1 | T2 |
| SSB ARFCN |  |  | Freq1 |  |  |  | Freq2 |  |  |
| Duplex mode | Config 1 |  | FDD |  |  |  | TDD |  |  |
|  | Config 2,3,4,5 |  | TDD |  |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  | TDDConf.3.1 |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  | Config 3,4,5 |  | TDDConf.2.1 |  |  |  |  |  |  |
| Downlink initial BWP Configuration | Config 1,2,3,4,5 |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP Configuration | Config 1,2,3,4,5 |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration | Config 1,2,3,4,5 |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration | Config 1,2,3,4,5 |  | ULBWP.1.1 |  |  |  |  |  |  |
| TRS configuration | Config 1,2,3,4,5 |  | N/A |  |  |  | TRS.2.1 TDD |  |  |
| TCI state | Config 1,2,3,4,5 |  | TCI.State.0 |  |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |
|  | Config 3,4,5 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| Data RBs allocated | Config 1,2 |  | 52 | 66 | 52 |  | 66 | 52 | 66 |
|  | Config 3,4,5 |  | 106 |  | 106 |  |  | 106 |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  | - |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  | Config 3,4,5 |  | SR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET Parameters | Config 1 |  | CR.1.1 FDD |  |  |  | - |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  | Config 3,4,5 |  | CR.2.1 TDD |  |  |  |  |  |  |
| Dedicated CORESET Parameters | Config 1 |  | CCR.1.1 FDD |  |  |  | - |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |  |  |
|  | Config 3,4,5 |  | CCR.2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  | [SSB.x FR2-2] |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  | [SSB.x FR2-2] |  |  |
|  | Config 4 |  |  |  |  |  | [SSB.x FR2-2] |  |  |
|  | Config 5 |  |  |  |  |  | [SSB.x FR2-2] |  |  |
| CSI-RS configuration for CSI reporting | Config 1,2,3,4,5 |  | N/A |  |  |  | N/A | CSI-RS.3.1 TDD Note 6 | CSI-RS.3.1 TDD |
| reportConfigType for CSI reporting |  |  | periodic |  |  |  | N/A |  |  |
| reportConfigType for L1-RSRP |  |  | periodic |  |  |  | N/A |  |  |
| reportQuantity for CSI reporting |  |  | cri-RI-PMI-CQI |  |  |  | N/A |  |  |
| reportQuantity for L1-RSRP |  |  | ssb-Index-RSRP |  |  |  | N/A |  |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |  |  | N/A |  |  |
|  | Config 3,4,5 |  | 10 |  |  |  |  |  |  |
| L1-RSRP reporting periodicity Note 7 | Config 1,2 | slot | 5 |  |  |  | N/A |  |  |
|  | Config 3,4,5 |  | 10 |  |  |  |  |  |  |
| CSI reporting offset | Config 1,2 | slot | 2 |  |  |  | N/A |  |  |
|  | Config 3,4,5 |  | 4 |  |  |  |  |  |  |
| L1-RSRP reporting offset | Config 1,2 | slot | 2 |  |  |  | N/A |  |  |
|  | Config 3,4,5 |  | 4 |  |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Propagation conditions |  |  | N/A Link only, see clause A.3.7A |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: All parameters apply for configuration 1, 2 and 3 Note 6: CSI-RS for CSI measurement is (re)configured in the next DL slot after slot m+TL1-RSRP during T2. Note 7: L1-RSRP measurement and reporting are configured to the the UE prior to the start of time period T1. |  |  |  |  |  |  |  |  |  |

Table A.14.X.3.2.1-3: OTA related test parameters for FR1 PCell activation case with FR2-2 SCell

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 |  | T3 |  |
| Angle of arrival configuration |  |  | N/A |  |  | According to clause A.3.15.1 |  |  |  |  |
| Assumption for UE beams Note 7 |  |  | N/A |  |  | Rough |  |  |  |  |
| Note 1 | Config 1,2,3 | dBm/15kHz | Link only, see clause A.3.7A |  |  | -104.7 |  |  |  |  |
| Note 1 | Config 1,2,3 | dBm/SCS |  |  |  | -95.7 |  |  |  |  |
|  | Config 4 |  |  |  |  | -95.7 |  |  |  |  |
|  | Config 5 |  |  |  |  | -92.7 |  |  |  |  |
|  | Config 1,2,3,4,5 | dB |  |  |  | -∞ |  | 7 |  | 7 |
|  | Config 1,2,3,4,5 | dB |  |  |  | -∞ |  | 7 |  | 7 |
| SSB_RPNote 2, Note 4 | Config 1,2,3 | dBm/SCS |  |  |  | -∞ |  | -88.7 |  | -88.7 |
|  | Config 4 |  |  |  |  | -∞ |  | -88.7 |  | -88.7 |
|  | Config 5 |  |  |  |  | -∞ |  | -85.7 |  | -85.7 |
| IoNote 2, Note 4 | Config 1,2,3 | dBm/95.04 MHz |  |  |  | -66.71 |  | -58.92 |  | -58.92 |
|  |  | dBm/380.16 MHz |  |  |  | -66.71 |  | -58.92 |  | -58.92 |
|  |  | dBm/380.16 MHz |  |  |  | -66.72 |  | -58.93 |  | -58.93 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not imit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |  |

##### A.14.X.3.2.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after at least one CSI-RS transmission occasion for channel measurement and reporting after slot (m+k). UE is allowed to postpone CSI report to next available UL resource if an available uplink resource is subject to interruption.  Whether CSI report in a slot was interrupted is checked by monitoring ACK/NACK sent in PCell in the slot.

During T2 the UE shall start sending valid L1-RSRP report for the SCell in the configured slots for CSI reporting after slot (m+TL1-RSRP), where TL1-RSRP is no larger than

3ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report

as defined in clause 8.3.2. For this test case, TFirstSSB_MAX=TSMTC_MAX=Trs=20ms; TL1-RSRP, measure=240ms and TL1-RSRP, report=5ms, which allows TL1-RSRP [980] ms.

During T2 the UE shall start sending CSI reports for the SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, where

- THARQ is defined in Table A.5.5.3.1.1-2

- Tactivation_time = 3ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}, which allows [1100] ms

- TCSI_Reporting = 10ms

- NR slot length is 0.125ms for this test case.

During T3 the UE shall stop sending CSI reports for both SCells no later than slot n+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length  to m+1+THARQ+3ms+TXNR slot length, as defined in clause 8.3, where TX =20ms.

During T3 the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption of PCell due to activation of SCell shall not be more than the values specified for SA in Clause 8.2.2.2.7.

#### A.14.X.3.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX

##### A.14.X.3.3.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.14.X.3.1.1 except the PCell and SCell are in FR2-2 inter-band.

The supported test configurations are shown in table A.14.X.3.3.1-1 below. The general test parameters are described in Tables A.14.X.3.3.1-2, and cell specific test parameters are described in Tables A.14.X.3.3.1-3. OTA related test parameters are shown in table A.14.X.3.3.1-4 below.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on NR. During T1 the SCell is powered off and UE is not aware of SCell. A MAC message for activation of SCell is sent by the test equipment 100ms after the RRC message, in a slot # denoted m.

The point in time at which the MAC message for activation of SCell is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 is increased to same level as for cell 2. During T2, the test equipment monitors the L1-RSRP measurement reporting for the SCell. The time when test equipment receives a valid L1-RSRP report is denoted as slot m+TL1-RSRP. In the next DL slot after slot m+TL1-RSRP, the test equipment sends a MAC message for the activation of the TCI state of the RMC CORESET of the SCell. In the same slot, the test equipment also sends an RRC message to configure the CSI-RS resources for SCell.

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PSCell during activation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell1 deactivation command is sent until CSI reporting for SCell1 is discontinued.

Table A.14.X.3.3.1-1: Supported test configurations for FR2-2 SCell activation in FR2-2 inter-band

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | NR 480 kHz SSB SCS, 400MHz bandwidth, TDD duplex mode |
| 3 | NR 960 kHz SSB SCS, 400MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.14.X.3.3.1-2: General test parameters for FR2-2 SCell activation in FR2-2 inter-band

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1,2 | Two NR radio channels are used for this test. RF channel number 1 is in band 1 and RF channel number 2 is in band 2, where bands 1 and 2 are inter-band CA operating bands in FR2-2 as specified in Table 5.2A.2-1 in TS38.101-2. |
| Active PCell | Config 1,2,3 |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell | Config 1,2,3 |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2. |
| CP length | Config 1,2,3 |  | Normal |  |
| DRX | Config 1,2,3 |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index | Config 1,2,3 |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on NR channel number | Config 1,2,3 | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | Config 1,2,3 | ms | 160 |  |
| Cell2 timing offset to cell1 | Config 1,2,3 | s | ≤8 | A random value from 0s to 8s |
| T1 | Config 1,2,3 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | Config 1,2,3 | s | 2 | During this time the UE shall activate the SCell. |
| T3 | Config 1,2,3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | Config 1,2,3 | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | Config 1,2,3 | ms | 2 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.14.X.3.3.1-3: Cell specific test parameters for FR2-2 SCell activation in FR2-2 inter-band

| ParameterNote 5 |  | Unit | T1 |  | T2 |  | T3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode | Config 1,2,3 |  | TDD |  | TDD |  | TDD |  |
| TDD configuration | Config 1,2,3 |  | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Downlink initial BWP Configuration | Config 1,2,3 |  | DLBWP.0.1 |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Downlink dedicated BWP Configuration | Config 1,2,3 |  | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Uplink initial BWP configuration | Config 1,2,3 |  | ULBWP.0.1 |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| Uplink dedicated BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| TRS configuration | Config 1,2,3 |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| TCI state | Config 1,2,3 |  | TCI.State.0 |  | TCI.State.0 |  | TCI.State.0 |  |
| BWchannel | Config 1,2,3 | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| PDSCH Reference measurement channel | Config 1,2,3 |  | SR.3.1 TDD | - | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Parameters | Config 1,2,3 |  | CR.3.1 TDD | - | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Dedicated CORESET Parameters | Config 1,2,3 |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| CSI-RS configuration | Config 1,2,3 |  | NA | NA | NA | CSI-RS.3.1 TDD Note 2 | NA | CSI-RS.3.1 TDD |
| CSI reporting periodicity Note 3 | Config 1,2,3 |  | NA | 5 | NA | 5 | NA | 5 |
| OCNG Patterns | Config 1,2,3 |  | OP.1 |  |  |  |  |  |
| SSB Configuration | Config 1,2,3 |  | [SSB.x FR2-2] |  |  |  |  |  |
| SMTC Configuration | Config 1,2,3 |  | SMTC.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS | Config 1,2,3 | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Propagation conditions | Config 1,2,3 |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: CSI-RS for CSI measurement is (re)configured in the next DL slot after slot m+TL1-RSRP during T2. Note 3: L1-RSRP measurement and reporting are configured to the the UE prior to the start of time period T1. |  |  |  |  |  |  |  |  |

Table A.14.X.3.3.1-4: OTA related test parameters for FR2-2 SCell activation in FR2-2 inter-band

| ParameterNote 6 |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup | Config 1,2,3 |  | Setup 3 as specified in clause A.3.15 |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |
| Assumption for UE beams Note 7 | Config 1,2,3 |  | Rough |  |  | Rough |  |  |
| Note1 | Config 1,2,3 | dBm/15kHzNote4 | -92.1 |  |  | -92.1 |  |  |
| Note1 | Config 1 | dBm/SCSNote3 | -83.1 |  |  | -83.1 |  |  |
|  | Config 2 |  | -83.1 |  |  | -83.1 |  |  |
|  | Config 3 |  | -80.1 |  |  | -80.1 |  |  |
|  | Config 1,2,3 | dB | 0 |  |  | 0 |  |  |
| SS-RSRPNote2 | Config 1 | dBm/SCS Note4 | -83.1 |  |  | -83.1 |  |  |
|  | Config 2 |  | -83.1 |  |  | -83.1 |  |  |
|  | Config 3 |  | -80.1 |  |  | -80.1 |  |  |
|  | Config 1,2,3 | dB | 0 |  |  | 0 |  |  |
| IoNote2 | Config 1 | dBm/95.04 MHz Note4 | -51.10 |  |  | -51.10 |  |  |
|  | Config 2 | dBm/380.16 MHz | -51.10 |  |  | -51.10 |  |  |
|  | Config 3 | dBm/380.16 MHz | -51.12 |  |  | -51.12 |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: All parameters apply for configuration 1 Note 7: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

##### A.14.X.3.3.2	Test Requirements

During T2 the UE shall start sending CSI report for the SCell in the configured slots for CSI reporting after at least one CSI-RS transmission occasion for channel measurement and reporting after slot (m+k). UE shall send the first CSI report for SCell after receiving at least one CSI-RS transmission occasion for channel measurement and reporting after slot (m+k), or in the next available uplink resource for CSI reporting if the slot was subject to interruption. Whether CSI report in a slot was interrupted is checked by monitoring ACK/NACK sent in PCell in the slot.

During T2, the UE shall start sending valid L1-RSRP report for the SCell in the configured slots for CSI reporting after slot (m+TL1-RSRP), where TL1-RSRP is no larger than 3ms + TFirstSSB_MAX + 23*TSMTC_MAX +12*Trs + TL1-RSRP, measure + TL1-RSRP, report as defined in clause 8.3.2. For this test case, TFirstSSB_MAX=TSMTC_MAX=Trs=20ms; TL1-RSRP, measure=480ms and TL1-RSRP, report=5ms, which allows TL1-RSRP =1480ms.

During T2, the UE shall start sending CSI reports for the SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, where

-		THARQ is defined in Table A.14.X.3.3.1-2

-		Tactivation_time = 3ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}, which allows 1510ms

-		TCSI_Reporting = 10ms

-		NR slot length is 0.125ms for this test case.

During T2, the interruption of PCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length  to m+1+THARQ+3ms+TXNR slot length, where TX =20ms.

During T3, the UE shall stop sending CSI reports for SCell no later than slot n+THARQ+3msNR slot length, as defined in clause 8.3.

During T3, the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length as defined in clause 8.3.

#### A.14.X.3.4	Direct SCell activation at SCell addition of known SCell in FR2-2

##### A.14.X.3.4.1	Test Purpose and Environment

The purpose of this test is to verify that the delay and interruption for direct SCell activation delay at SCell addition are within the requirements stated in clause 8.3.4.

The supported test configurations are shown in Table A.14.X.3.4.1-1 below. The general test parameters are given in Table A.14.X.3.4.1-2 and cell-specific test parameters in Table A.14.X.3.4.1-3. OTA related test parameters are shown in Table A.14.X.3.4.1-4.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two FR2-2 carriers and two NR cells. Before the test starts the UE is connected to Cell 1 (PCell) on carrier #1, but is not aware of Cell 2 on NR carrier #2. Cell 1 and Cell 2 have constant signal levels throughout the test. The UE is monitoring the PCell. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 is monitored by the UE. During T1, Cell 2 should be detected and measured by the UE such that it meets the condition for known cell defined in clause 8.3.4 for direct SCell activation.

Time period T2 starts when the RRCReconfiguration message for the configuration and activation of Cell 2 (the SCell), which is sent from the test equipment, is received at the UE antenna connector in a slot # denoted m. The test equipment shall set the parameter sCellState to activated for the SCell, which causes Cell 2 to become configured and activated.

Time period T3 starts at (m + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during the activation of SCell. The test equipment verifies the activation time by counting the slots from the time when the SCell activation message is sent until a CQI report with other than CQI index 0 is received.

Table A.14.X.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 480 kHz SSB SCS, 400MHz bandwidth, TDD duplex mode |
| 3 | NR 960 kHz SSB SCS, 400MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.14.X.3.4.1-2: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1,2 | Two NR radio channels (1,2) in FR2-2 are used for this test |
| Active PCell | Config 1, 2 ,3 |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured and activated SCell | Config 1, 2 ,3 |  | Cell 2 | Configured and activated SCell on NR RF channel number 2. |
| CP length | Config 1, 2 ,3 |  | Normal |  |
| DRX | Config 1, 2 ,3 |  | OFF | Continuous monitoring of primary cell |
| SCell measurement cycle (measCycleSCell) | Config 1, 2 ,3 | ms | 160 |  |
| T1 | Config 1, 2 ,3 | s | 7 | During this time the measurement for Cell 2 is configured, and Cell 2 is detected. |
| T2 | Config 1, 2 ,3 | s | Ndirect | During this time the UE shall configure and activate Cell 2 as SCell. |
| T3 | Config 1, 2 ,3 | ms | 100 | During this time the UE shall report valid CQI for both PCell and SCell. |
| THARQ | Config 1, 2 ,3 | ms | k1×NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| k | Config 1, 2 ,3 | slot |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.14.X.3.4.1-3: Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| SSB ARFCN |  |  | freq1 |  | freq2 |  |  |  |
| Duplex mode | Config 1, 2, 3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1, 2, 3 |  | TDDConf.3.1 |  |  |  |  |  |
| BWchannel | Config 1, 2, 3 | MHz | 100: NRB,c = 66 |  |  |  |  |  |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |  |  |  |  |  |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |  |  |  |  |  |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |  |  |  |  |  |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |  |  |  |  |  |
| Timing offset to Cell 1 |  | ms | Not Applicable |  | 0 |  |  |  |
| PDSCH Reference measurement channel | Config 1, 2, 3 |  | SR.3.1 TDD |  | SR.3.1 TDD |  |  |  |
| RMSI CORESET Reference Channel | Config 1, 2, 3 |  | CR.3.1 TDD |  | CR.3.1 TDD |  |  |  |
| RMC CORESET Reference Channel | Config 1, 2, 3 |  | CCR.3.1 TDD |  | CCR.3.1 TDD |  |  |  |
| TRS configuration | Config 1, 2, 3 |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |  |  |
| CSI-RS configuration | Config 1, 2, 3 |  | CSI-RS.3.1 TDD |  | CSI-RS.3.1 TDD |  |  |  |
| CSI reporting periodicity | Config 1, 2, 3 | ms | 5 |  | 5 |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration | Config 1, 2, 3 |  | [SSB.x FR2-2] |  | [SSB.x FR2-2] |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |  |  |

Table A.14.X.3.4.1-4: OTA related test parameters

| ParameterNote 6 |  | Unit |  | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration | Config 1, 2, 3 |  |  | Setup 1 according to A.3.15.1 |  |  |  |  |  |
| Assumption for UE beamsNote 7 | Config 1, 2, 3 |  |  | Rough |  |  | Rough |  |  |
| Note1 | Config 1, 2, 3 |  | dBm/15kHzNote4 | -112 |  |  | -112 |  |  |
| Note1 | Config 1 |  | dBm/SCSNote3 | -102.97 |  |  | -102.97 |  |  |
|  | Config 2 |  |  | -102.97 |  |  | -102.97 |  |  |
|  | Config 3 |  |  | -99.97 |  |  | -99.97 |  |  |
|  | Config 1, 2, 3 |  | dB | 14 |  |  | 14 |  |  |
| SS-RSRPNote2 | Config 1 |  | dBm/SCS Note4 | -88.97 |  |  | -88.97 |  |  |
|  | Config 2 |  |  | -88.97 |  |  | -88.97 |  |  |
|  | Config 3 |  |  | -85.97 |  |  | -85.97 |  |  |
|  | Config 1, 2, 3 |  | dB | 14 |  |  | 14 |  |  |
| IoNote2 | Config 1 |  | dBm/95.04 MHz Note4 | -59.81 |  |  | -59.81 |  |  |
|  | Config 2 |  | dBm/380.16 MHz | -59.81 |  |  | -59.81 |  |  |
|  | Config 3 |  | dBm/380.16 MHz | -59.83 |  |  | -59.83 |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: All parameters apply for configuration 1 Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |

##### A.14.X.3.4.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after slot (m+k). UE is allowed to postpone CSI report to next available UL resource if an available uplink resource is subject to interruption.  Whether CSI report in a slot was interrupted is checked by monitoring ACK/NACK sent in PCell in the slot.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot m+NdirectNR slot length , where

Ndirect = TRRC_Process + T1 + Tactivation_time + TCSI_Reporting - 3ms,

- TRRC_Process = 16ms, which is the RRC procedure delay defined for SCell addition in clause 12 of TS 38.331 [2],

- T1 is the delay from slot m + TRRC_Process until the transmission of RRCReconfigurationComplete message,

- Tactivation_time = TFirstSSB+ 5ms = 25ms,

- TCSI_Reporting = 10ms

This gives a total of Ndirect = 16 + T1 + 25 + 10 - 3 = (48 + T1) ms, and NR slot length is 0.125ms.

During T3 the UE shall send CSI reports for SCell with non-zero CQI index and continue to send CSI reports for SCell  with non-zero CQI index until the end of T3.

During T2 interruption of PSCell during SCell activation shall not happen outside the window from slot m+1 to slot  m+1+TRRC_Process+T1+TXNR slot length as defined in clause 8.3.4, where TX =20ms.

The interruption of PCell due to activation of SCell shall not be more than the values specified for NR SA in clause 8.2.2.2.11.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot m+NdirectNR slot length as defined in clause 8.3.4 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

#### A.14.X.3.5	Direct SCell activation at handover with known SCell in FR2-2

##### A.14.X.3.5.1	Test Purpose and Environment

This test is to verify the requirements specified in sub clause 8.3.5 for the FR2-2 handover with direct SCell activation.

The test scenario comprises of three FR2-2 cells, one source PCell (Cell 1), one target PCell (Cell 2) and one SCell (Cell 3). The test consists of three successive time periods, with time durations of T1, T2, and T3 respectively.

At the start of time duration T1, the UE is in connected mode with PCell (Cell 1). Both Cell 2 and Cell 3 are known to UE and UE is reporting CQI for all Cell 1.

Time period T2 starts when UE receives a handover command that initiate handover of UE to Cell2 and also activates Cell 3. This is done using an RRCConnectionReconfiguration message with parameter sCellState set to activated for the Cell 3. The message is sent from the test equipment to the UE and is received in a slot number n at the UE antenna connector. The UE shall accomplish the handover, addition and activation of the SCell no later than slot (n +NdirectNR slot length).

Time period T3 starts at (n +NdirectNR slot length), at which point UE shall be reporting a valid CSI for both Cell 2 and Cell 3 as given in tables A.14.X.3.5.1-1 and A.14.X.3.5.1-2.

Table A.14.X.3.5.1-1: Supported test configurations for FR2-2 handover with direct SCell activation case

| Configuration | Description |
| --- | --- |
| 1 | SCell: NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | SCell: NR 480 kHz SSB SCS, 400MHz bandwidth, TDD duplex mode Source cell: NR 480 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode Target cell: NR 480 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode |
| 3 | SCell: NR 960 kHz SSB SCS, 400MHz bandwidth, TDD duplex mode Source cell: NR 960 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode Target cell: NR 960 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.14.X.3.5.1-2: General test parameters for FR2-2 handover with direct SCell activation case

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1,2,3 |  | 1, 2, 3 | Three NR radio channels are used for this test, Cell 1, Cell2 and Cell 3 use RF channel 1, 2 and 3 respectively. |
| A4-Offset |  | Config 1,2,3 | dBm | -120 |  |
| Time offset between cells |  | Config 1,2,3 |  | 3 s | Synchronous cells |
| Initial conditions | Source cell | Config 1,2,3 |  | Cell 1 | Source Cell |
|  | Target cell |  |  | Cell 2 | Neighbour cell |
|  | SCell |  |  | Cell 3 | SCell is not added and activated |
| Final condition | Source cell | Config 1,2,3 |  | Cell 2 | Cell 2 is Source cell after handover |
|  | Neighbour cell |  |  | Cell 1 | Neighbour cell |
|  | SCell |  |  | Cell 3 | SCell is added and activated |

Table A.14.X.3.5.1-3: Cell specific test parameters for FR2-2 SCell activation case

| ParameterNote 5 |  | Unit | T1 |  |  | T2 |  |  | T3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |
| SSB ARFCN | Config 1,2,3 |  | freq1 | freq2 | freq 3 | freq1 | freq2 | freq 3 | freq1 | freq2 | freq3 |
| Duplex mode | Config 1,2,3 |  | TDD |  |  | TDD |  |  | TDD |  |  |
| TDD configuration | Config 1,2,3 |  | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| Downlink initial BWP Configuration | Config 1,2,3 |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Downlink dedicated BWP Configuration | Config 1,2,3 |  | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  |
| Uplink initial BWP configuration | Config 1,2,3 |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Uplink dedicated BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  |
| TRS configuration | Config 1,2,3 |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  |
| TCI state | Config 1,2,3 |  | TCI.State.0 |  |  | TCI.State.0 |  |  | TCI.State.0 |  |  |
| BWchannel | Config 1,2,3 | MHz | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| PDSCH Reference measurement channel | Config 1,2,3 |  | SR.3.1 TDD |  | - | SR.3.1 TDD |  | - | SR.3.1 TDD |  |  |
| RMSI CORESET Parameters | Config 1,2,3 |  | CR.3.1 TDD |  | - | CR.3.1 TDD |  | - | CR.3.1 TDD |  |  |
| Dedicated CORESET Parameters | Config 1,2,3 |  | CCR.3.1 TDD |  | - | CCR.3.1 TDD |  | - | CCR.3.1 TDD |  |  |
| OCNG Patterns | Config 1,2,3 |  | OP.1 |  |  |  |  |  |  |  |  |
| SSB Configuration | Config 1,2,3 |  | [SSB.xFR2-2] |  |  |  |  |  |  |  |  |
| SMTC Configuration | Config 1,2,3 |  | SMTC.1 |  |  |  |  |  |  |  |  |
| PRACH configuration | Config 1,2,3 |  | FR2-2 PRACH configuration 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |  |
| Propagation conditions |  |  | AWGN |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Void |  |  |  |  |  |  |  |  |  |  |  |

Table A.14.X.3.5.1-4: OTA related test parameters for FR2-2 SCell activation case

| ParameterNote 6 |  | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration | Config 1,2,3 |  | Setup 1 according to table A.3.15.1 |  |  | Setup 1 according to table A.3.15.1 |  |  | Setup 1 according to table A.3.15.1 |  |  |
| Assumption for UE beams Note 7 | Config 1,2,3 |  | Rough |  |  | Rough |  |  | Rough |  |  |
| Note1 | Config 1,2,3 | dBm/15kHzNote4 | -112 |  |  | -112 |  |  | -112 |  |  |
| Note1 | Config 1 | dBm/SCSNote3 | -102.97 |  |  | -102.97 |  |  | -102.97 |  |  |
|  | Config 2 |  | -102.97 |  |  | -102.97 |  |  | -102.97 |  |  |
|  | Config 3 |  | -99.97 |  |  | -99.97 |  |  | -99.97 |  |  |
|  | Config 1,2,3 | dB | 14 |  |  | 14 |  |  | 14 |  |  |
| SS-RSRPNote2 | Config 1 | dBm/SCS Note4 | -88.97 |  |  | -88.97 |  |  | -88.97 |  |  |
|  | Config 2 |  | -88.97 |  |  | -88.97 |  |  | -88.97 |  |  |
|  | Config 3 |  | -85.97 |  |  | -85.97 |  |  | -85.97 |  |  |
|  | Config 1,2,3 | dB | 14 |  |  | 14 |  |  | 14 |  |  |
| IoNote2 | Config 1 | dBm/95.04 MHz Note4 | -59.81 |  |  | -59.81 |  |  | -59.81 |  |  |
|  | Config 2 |  | -59.81 |  |  | -59.81 |  |  | -59.81 |  |  |
|  | Config 3 |  | -59.84 |  |  | -59.84 |  |  | -59.84 |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |  |  |

##### A.14.X.3.5.2	Test Requirements

The UE shall be capable to transmit valid CSI report for PCell (Cell 2) and to the directly activated SCell1 no later than in slot n+ Ndirect.

The SCell activation delay, Ndirect, can be expressed as: Ndirect = TRRC_process + Tinterrupt + T2 + T3 + Tactivation_time + TCSI_Reporting - 3ms; Where:

-	TRRC_Process: RRC procedure delay defined in clause 12 of TS 38.331 and it is equal to 16ms,

-	Tinterrupt: Interruption time during handover as specified in clause 6.1.1. The value to be verified in the test is 52 ms (Tinterrupt = 0 ms for Tsearch + 10ms for TIU + 20 ms for Tprocessing + 20ms for T∆ + 2 ms for Tmargin ms) by assuming known SCell and SMTC.1 configuration.

-	T2: Delay from slot n+TRRC_Process+TinterruptNR slot length  until UE has obtained a valid TA command for the target PCell,

-	T3: Delay for applying the received TA for uplink transmission in the target PCell, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213,

-	Tactivation_time and TCSI_Reporting are specified in clause 8.3.2, where the following definitions of TFirstSSB and TFirstSSB_MAX as defined in section 8.3.5 shall apply:

During time period T2 of the test, the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TSMTC_SCell + 5ms, as defined in clause 8.3.

During time period T3 of the test, the UE shall stop sending CSI reports for SCell at latest in a slot m+THARQ+3msNR slot length, as defined in clause 8.3.

During time period T2 of the test, interruption of PCell / PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.

During time period T3 of the test, the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During time period T2 of the test, if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

# End of Change 1