3GPP TSG-RAN4 Meeting #104-e		R4-2215088

# Electronic Meeting, August 15th - 26th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on NR UE Transmit Timing Test for FR1 for 1 and 2 Rx UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Perf |  |  |  |  |  | Date: |  |  | 2022-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Provides new test cases for RedCap rel-17 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Skeleton structure contains high level sections for: - Timing - UE transmit timing - A.6.4.1.x1 NR UE Transmit Timing Test for FR1 for 1 Rx UE - A.6.4.1.x2 NR UE Transmit Timing Test for FR1 for 2 Rx UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap test cases requirements will be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections: A.6.4.1.x1, and A.6.4.1.x2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE 1----------------------------

#### A.6.4.1.x1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE

##### A.6.4.1.x1.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1A.2.

Supported test configurations are shown in Table A.6.4.1.x1.1-1.

Table A.6.4.1.x1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz |
| 4 | NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

For this test a single NR cell is used. Table A.6.4.1.x1.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.6.4.1.1.1-3.

Table A.6.4.1.x1.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2,3 | 1 |  | 1 |  |
| TDD configuration |  | 1 | Not Applicable |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |
|  |  | 4 | Not Applicable |  |  |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |  |  |
|  |  | 2 | 10: NRB,c = 52 |  |  |  |
|  |  | 3 | 20: NRB,c = 52 |  |  |  |
|  |  | 4 | 10: NRB,c = 52 |  |  |  |
| Initial BWP Configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3,4 | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| DRx Cycle | ms | 1,2,3,4 | N/A |  |  | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
|  |  | 4 | SR.x.x HD-FDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
|  |  | 4 | CR.x.x HD-FDD |  |  |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
|  |  | 4 | CCR.x.x HD-FDD |  |  |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  |  |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |  |  |
|  |  | 3 | SSB.x1 FR1 |  |  |  |
| SMTC Configuration |  | 1,2,4 | SMTC.1 |  |  |  |
|  |  | 3 | SMTC.2 |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
|  |  | 4 | TRS.x.x HD-FDD |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3,4 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4 | -98 |  | -98 |  |
| Note2 | dBm/SCS | 1,2,4 | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  |
|  |  | 1,2,3,4 | 3 |  | 3 |  |
|  |  | 1,2,3,4 | 3 |  | 3 |  |
| SS-RSRPNote3 | dBm/SCS | 1,2,4 | -95 |  | -95 |  |
|  |  | 3 | -92 |  | -92 |  |
| IoNote3 | dBm/9.36MHz | 1,2,4 | -65.2 |  | -65.2 |  |
|  | dBm/18.72MHz | 3 | -62.28 |  | -62.28 |  |
| Propagation condition |  | 1,2,3,4 | AWGN |  |  |  |
| SRS Config |  | 1,2,4 | SRSConf.1Note6 | SRSConf.3Note6 |  |  |
|  |  | 3 | SRSConf.1Note6 | SRSConf.2Note6 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRx related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.6.4.1.1.1-3 |  |  |  |  |  |  |

##### A.6.4.1.x1.2	Test requirements

The test requirements are the same as in clause A.6.4.1.1.2.

----------------------END OF CHANGES 1----------------------------

----------------------START OF CHANGE 2----------------------------

#### A.6.4.1.x2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE

##### A.6.4.1.x2.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1A.2.

Supported test configurations are shown in Table A.6.4.1.x2.1-1.

Table A.6.4.1.x2.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz |
| 4 | NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

For this test a single NR cell is used. Table A.6.4.1.x2.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.6.4.1.1.1-3.

Table A.6.4.1.x2.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2,3 | 1 |  | 1 |  |
| TDD configuration |  | 1 | Not Applicable |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |
|  |  | 4 | Not Applicable |  |  |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |  |  |
|  |  | 2 | 10: NRB,c = 52 |  |  |  |
|  |  | 3 | 20: NRB,c = 52 |  |  |  |
|  |  | 4 | 10: NRB,c = 52 |  |  |  |
| Initial BWP Configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3,4 | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| DRx Cycle | ms | 1,2,3,4 | N/A |  |  | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
|  |  | 4 | SR.x.x HD-FDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
|  |  | 4 | CR.x.x HD-FDD |  |  |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
|  |  | 4 | CCR.x.x HD-FDD |  |  |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  |  |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |  |  |
|  |  | 3 | SSB.x1 FR1 |  |  |  |
| SMTC Configuration |  | 1,2,4 | SMTC.1 |  |  |  |
|  |  | 3 | SMTC.2 |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
|  |  | 4 | TRS.x.x HD-FDD |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3,4 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4 | -98 |  | -98 |  |
| Note2 | dBm/SCS | 1,2,4 | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  |
|  |  | 1,2,3,4 | 3 |  | 3 |  |
|  |  | 1,2,3,4 | 3 |  | 3 |  |
| SS-RSRPNote3 | dBm/SCS | 1,2,4 | -95 |  | -95 |  |
|  |  | 3 | -92 |  | -92 |  |
| IoNote3 | dBm/9.36MHz | 1,2,4 | -65.2 |  | -65.2 |  |
|  | dBm/18.72MHz | 3 | -62.28 |  | -62.28 |  |
| Propagation condition |  | 1,2,3,4 | AWGN |  |  |  |
| SRS Config |  | 1,2,4 | SRSConf.1Note6 | SRSConf.3Note6 |  |  |
|  |  | 3 | SRSConf.1Note6 | SRSConf.2Note6 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRx related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.6.4.1.1.1-3 |  |  |  |  |  |  |

##### A.6.4.1.x2.2	Test requirements

The test requirements are the same as in clause A.6.4.1.1.2.

----------------------END OF CHANGES 2----------------------------