3GPP TSG-RAN WG4 Meeting #104-e                                         R4-2215091

Electronic Meeting, Aug 15 – Aug 26, 2022

Title:	draft Reply LS on ModifiedMPR-Behaviour clarification for different power classes

Response to:	R5-223635

Release:	Rel-16

Work Item:	NR_RF_FR2_req_enh-UEConTest

Source:	RAN4

To:	RAN5

Cc:

Contact Person:

Name: 	Liu Ye

E-mail Address: 	leo.liuye@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

RAN4 would like to thank RAN5 for clarification questions on ModifiedMPR-Behaviour for different power classes.

With regard to the questions raised by RAN5, the answers from RAN4 are as follows:

Question a) For Rel-15 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 applicable if the UE supports modifiedMPR-Behaviour bit 0 UE capability?

Answer: Yes, for Rel-15 PC3 UE, MPR defined in 38.101-2 v16.2.0 is also applicable if the UE reports modifiedMPR-Behaviour bit 0 UE capability, but this modified MPR is optional for a Rel-15 UE.

Question b) For Rel-15 PC2 and PC4 UEs, is modifiedMPR-Behaviour bit 0 capability applicable?

Answer: No, modifiedMPR-Behaviour bit 0 capability is not applicable for Rel-15 PC2 and PC4 UEs.

Question c) For Rel-16 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 mandatory or optional? In case it is mandatory then is the Rel-16 UE expected to signal modifiedMPR-Behaviour bit 0=true?

Answer: No consensus reached in RAN4 yet. RAN4 will keep RAN5 updated for the conclusion of the question.

Question d) For Rel-16 PC3 UE, which version of specification is taken as default MPR requirement, 38.101-2 v16.2.0 or latest version (v16.11.0 released in Apr 2022)? What are the Rel-16 MPR requirements if the UE signals respectively modifiedMPR-Behaviour bit 0=false and modifiedMPR-Behaviour bit 0=true?

Answer: No consensus reached in RAN4 yet. RAN4 will keep RAN5 updated for the conclusion of the question.

Question e) For Rel-16 PC2, PC4 and PC5 UEs is modifiedMPR-Behaviour bit 0 capability applicable?

Answer: No, modifiedMPR-Behaviour bit 0 capability is not applicable for Rel-16 PC2, PC4 and PC5 UEs.

Question f) For Rel-17 PC3 UE, what are the MPR requirements if the UE signals respectively modifiedMPR-Behaviour bit 0=false and modifiedMPR-Behaviour bit 0=true?

Answer: For a Rel-17 PC3 UE, it shall comply with the improved MPR defined in the latest Rel-17 version of 38.101-2 and may not set this bit 0 to false.

2. Actions:

To RAN5:

ACTION: RAN4 respectfully asks RAN5 to take the above feedback into account for the future work.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#104-bis-e				10th – 19th October 2022									Online

TSG-RAN4 Meeting#105 	 					14th – 18th November 2022								Canada