3GPP TSG-RAN4 Meeting #104-e		R4-2215103

# Electronic Meeting, 15-26 August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Set 1-2: PRP and PRS Ês/Iot conditions for NR PRS-based measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The PRP and PRS Ês/Iot conditions for NR PRS-based measurements are defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draft CR is according to the work split in R4-2211060: - Set 1-2: PRP and PRS Ês/Iot conditions for NR PRS-based measurements. The conditons in terms of minimum PRS_RP and applicable PRS Ês/Iot are specified for the following PRS measurements: - PRS-RSRPP measurements and - Following PRS measurements when they are performed with reduced number of samples: - RSTD for reference and neighbour cell PRS resources, - PRS-RSRP, - PRS-RSRPP and - UE Rx-Tx time difference measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conditions for applicability of requirements for PRS-RSRPP and PRS measurements with reduced number of samples will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

## B.2.14	Conditions for NR PRS-based measurements

This clause defines the following conditions for NR PRS-based measurements and corresponding procedures performed based on PRS: PRP and PRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.14-1 for FR1 NR cells.

The conditions are defined in Table B.2.14-2 for FR2 NR cells.

Table B.2.14-1: Conditions for NR PRS-based measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum PRP1,2 |  |  | PRS Ês/Iot |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSPRS |  |  | dB |  |
|  |  | SCSPRS = 15 kHz | SCSPRS = 30 kHz | SCSPRS = 60 kHz |  |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -6 Note2 -13 Note3 -3 Note4 | -6 Note5 -3 Note6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 |  |  |
|  | NR_TDD_FR1_C | -126 | -123 | -120 |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 |  |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 |  |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 |  |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 |  |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. NOTE 2: PRS Ês/Iot for RSTD measurement reference cell PRS resource. NOTE 3: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 4: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 5: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement when performed with reduced number of samples. NOTE 6: PRS Ês/Iot for RSTD measurement reference cell PRS resource when performed with reduced number of samples. |  |  |  |  |  |  |

Table B.2.14-2: Conditions for NR PRS-based measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum PRP1,2 Note 2, Note 3 |  |  |  |  | PRS Ês/Iot |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSPRS |  |  |  |  | dB |  |
|  |  |  | SCSPRS = 120 kHz |  |  |  | SCSPRS = 60 kHz |  |  |
|  |  |  | UE power class |  |  |  | UE power class |  |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSPRS = 120 kHz) - 3dB | -6 Note4 -13 Note5 -3 Note6 | -6 Note7 -3 Note8 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |  |
|  |  | n259 |  |  | -108.5 | -124.7+Y4 |  |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSPRS = 120 kHz) - 3dB | -6 Note4 -13 Note5 -3 Note6 | -6 Note7 -3 Note8 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |  |
|  |  | n259 |  |  | -95.7 | -115.7+Z4 |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum PRS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. NOTE 4: PRS Ês/Iot for RSTD measurement reference cell PRS resource. NOTE 5: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 6: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 7: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement when performed with reduced number of samples. NOTE 8: PRS Ês/Iot for RSTD measurement reference cell PRS resource when performed with reduced number of samples. |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.14-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

----------------------END OF CHANGE----------------------------