3GPP TSG-RAN WG4 Meeting # 104-e					R4-2215140

Electronic Meeting, August 15 – August 26, 2022

3GPP TSG-RAN WG2 Meeting #119 electronic		                              R2-2209002

Online, August 17th- August 29th, 2022

Title:	LS to RAN4 on DC location

Response to:	R2-2206951 (R4-2210782)

Release:	Release 17

Work Items:	NR_RF_FR2_req_enh2-Core

Source:	RAN WG2

To:	RAN WG4

CC:

Contact Person:

Name:		Xiaodong Yang

E-mail Address:	yangxiaodong5g@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 would like to thank RAN4 for their LS on DC location for intra-band CA. RAN2 discussed the LS and has the following questions for clarification.

Question 1:

RAN4 has concluded that frequency component type is same for both default DC locations for different CC groups. RAN2  also understands that the frequency component type is same for default DC locations for a given CC group independing on the different activated CC/BWP combination in a CC group RAN2 asks RAN4 to confirm our understanding.

Question 2:

In the LS R2-2206951 (R4-2210782), regarding offset range, RAN4 indicates that the granularity is the lowest subcarrier spacing used in the CA configuration. But considering that the offset is signaled per CC group, RAN2’s understanding is that lowest SCS in the CC group is used as the offset granularity. RAN2 asks RAN4 to confirm our understanding.

# 2	Actions

To RAN4

ACTION: RAN2 would like to ask RAN4 to answer the questions in this LS.

# 3	Dates of next TSG RAN WG2 meetings

TSG-RAN WG2 Meeting #119bis-e	10th October – 19th October 2022	E-meeting

TSG-RAN WG2 Meeting #120	14th November – 18th November 2022	Canada, CA