3GPP TSG-RAN WG4 Meeting # 104-e	R4-2215154

Electronic Meeting, 15 ‒ 26 August 2022

Title: 	WF on HST FR2 RRM Core Requirement Maintenance

Agenda Item:	9.7.5

Source: 	Nokia, Nokia Shanghai Bell

Document for:	Approval

# UL timing

## Large one-step UL timing adjustment

Way forward on large one-step UL timing adjustment

Open issue needs further discussion:

- Option 1: Clarify the requirement if target TCI state is not in the active TCI state list and the DL timing difference is larger than [CP/4]

  - Option 1a: If target TCI state is not in the active TCI state list and the DL timing difference is larger than [CP/4], limit the time needed for the UE to follow again clause 7.1.2.1 requirements and to adjust its UL timing within ±Te. It should happen not later than Trs + 2ms after the TCI state switch.

  - Option 1b: Same as above, but Tssb is used instead of Trs

  - Option 1c: Tq requirement in 7.1.2.1 applicable to UL slots except the first after TCI state switch.

  - Option 1d: The gradual timing adjustment step of Tq shall be applied after the one shot uplink timing adjustment after TCI state switch.

  - Option 1e: The gradual timing adjustment in 7.1.2.1 with reference timing T_new - (N_TA + N_TA_offset) + 2*(T_old - T_new) is applicable to UL slots except the first after TCI state switch.

  - Other options are not precluded

- Option 2: Keep current specification as it is.

Way forward on the value of relaxed UL transmit timing accuracy

Open issue needs further discussion:

- Option 1: Remove square brackets and use ±7Ts as relaxed UL transmit timing accuracy in the current requirement.

- Option 1a: Remove square brackets and use ±7*64*Tc as relaxed UL transmit timing accuracy in the current requirement.

- Option 2: Do not distinguish the case “new target TCI state is not in the active TCI state list” and “in the active TCI state” and use ±7Ts accuracy for both.

## Other remaining UL transmit timing issues

Way forward on UL timing requirement when large one-step mechanism is disabled:

Open issue needs further discussion:

- Option 2: No impact on UE behavior (no additional requirements)

- Option 4: After the TCI state switch, the UE shall not transmit except for RACH preamble in the new target TCI before one of the following conditions is fulfilled:

  - the new timing advance is acquired and applied in the target TCI state according to the requirements in clause 7.3;

  - the UL transmission is scheduled by the gNB.
In this case, the requirements in clause 7.1.2.1 apply.

- Option 6: Power Class 6 UE initial transmission timing error after the TCI state switch shall be less than or equal to ±Te. Enhance the requirement in 7.1.2.

- Option 7: Introduce a DL timing difference threshold when highSpeedLargeOneStepUL-TimingFR2 is disabled.

- Other options are not precluded

# Other remaining issues in RRM CORE maintenance

Way forward on L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

Open issue needs further discussion:

- Option 1: For L1-SINR measurements with SSB-based CMR and dedicated IMR configured for FR2 HST, the same enhancements as SSB-based L1-RSRP measurements should be applied.

- Option 2: Do not define enhancement for L1-SINR measurements with SSB-based CMR and dedicated IMR

- Option 2a: For FR2 PC6 UE which support Rel-16 L1-SINR measurement, it can rely on legacy Rel-16 test cases to verify the performance.

Way forward on SMTC length in HST FR2 enhanced requirements:

For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TPSS/SSS_sync_intra is given in Table 9.2.5.1-11; [otherwise, TPSS/SSS_sync_intra is given in Table 9.2.5.1-2.]

For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TSSB_measurement_period_intra is given in Table 9.2.5.2-7; [otherwise, TSSB_measurement_period_intra is given in Table 9.2.5.2-2.]

The addition of the clarification note needs further discussion:

- Option 2: Add Notes in the TS:
Note: Operation with TPSS/SSS_sync_intra in Table 9.2.5.1-2 may not be guaranteed for the maximum speed under high-speed deployment scenarios.

Note: Operation with TSSB_measurement_period_intra in Table 9.2.5.2-2 may not be guaranteed for the maximum speed under high-speed deployment scenarios.

- Option 3: Add Notes in the WF:

Note: Operation with TPSS/SSS_sync_intra in Table 9.2.5.1-2 may not be guaranteed for the maximum speed under high-speed deployment scenarios.

Note: Operation with TSSB_measurement_period_intra in Table 9.2.5.2-2 may not be guaranteed for the maximum speed under high-speed deployment scenarios.

- Option 4: Do not add any notes.

Agreement on Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps for power class 6 UEs:

Define Mpss/sss_sync_w/o_gaps = 24 and Mmeas_period_w/o_gaps = 24 for PC 6 UEs in Clause 9.2.5.

NOTE 3 from tables Table 9.2.5.1-11 and Table 9.2.5.2-7 can be removed.

Way forward on applicability of enhanced requirements for other PCs:

Open issue needs further discussion:

When HST FR2 flags are configured for other power classes other than PC6, whether legacy requirements should be used.