3GPP TSG-RAN4 Meeting #104-e	R4-2215239

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1190 | rev | - | Current version: | 15.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for 38.101-1 maintenance part1 (Rel-15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15, NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-08-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | R4-2215101: In Table 6.2.3.1-1: Additional maximum power reduction (A-MPR), the frequency range for which no A-MPR is applied for NS_05 in Note 2 is incorrectly listed and inconsistent with Table 6.2.3.4-1: A-MPR regions for NS_05 and NS_05U. R4-2215024: For the simultaneousRxTx clarification for CA and SUL in clause 5.2A.2 and 5.2C, there are two kinds of band combinations, i.e. lower order and higher order band combinations. But it’s very hard to distinguish them in these clarification. That’s why the wording for simultaneousRxTx clarification need to be improved. R4-2212361: This CR aims to introduce missing harmonic exceptions and correct certain errors in the UE coexistence tables. Furthermore, duplicate band entries are removed. The UE coexistence lists specify relaxed emission requirements in case a harmonic falls into a protected band. The relaxation is defined in Note 2 of the UE coexistence table. Cases exist where a harmonic can fall very close to a protected band so that the 1MHz extension of the exception interval (as defined in Note 2) overlaps with the protected band. Those cases are currently handled inconsistently as Note 2 is not always specified. For example, Note 2 is provided for the case where the second harmonic of n7 affects the protected band n79 or in case of second harmonic of n85 affecting the protected band 50. On the other side it is missing for second harmonic of band n2 affecting the protected band 48. Another aspect is to harmonize LTE and NR specs. For example, E-UTRA Band 12 protects band 50 with -50dBm/MHz but allows relaxation for harmonics. This is required as the second harmonic affect the protected band. However, the harmonic relaxation is missing for n12 in 38.101-1 spec. To remove inconsistencies the Note 2 needs to be introduced for single bands if harmonic relaxation is missing. A full list of changes is provided in ‘Summary of change’. R4-2215110: To correct n41 MSD test frequencies to follow the channel raster rule R4-2214973: Due to some NR bands support 4 antenna connector implementation, in case of these bands are implemented as the constitute band in inter-band NR CA band combination, the MSD caused by harmonic/Rx harmonic mixing/cross band isolation/IMD for the impacted NR band who supports 4 antenna connector for some band combination were defined, but no 4Rx IMD MSD values defined for the others band combintion, see more details in R4-2212708. R4-2212563: There exists some incorrect reference clause numbers in the specification. R4-2214070: Hanging paras R4-2214881: The requirements for UL MIMO transmit quality are incorrect with respect to which requirements are defined per layer or per connector. Frequency error, carrier leakage, EVM and EVM equalizer flatness are measured on a per layer basis. As an alternative, the requirements may be verified on a per connector basis, however only using a specific UL MIMO configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | R4-2215101: The lower channel edge cutoff is replaced by the upper channel edge. R4-2215024: The wording for simultaneousRxTx clarification is improved to distinguish lower and higher order band combinations. R4-2212361: The following modifications are made for : 1. Band n2: Second harmonic with its overall exception interval overlaps with band 48. 2. Band n12: Second harmonic with its overall exception interval overlaps with band 50. 3. Band n25: Second harmonic with its overall exception interval overlaps with band 48. R4-2215110: To add general note in 7.3A.1: “For reference sensitivity exception test points where the specified carrier frequency does not correspond to a valid NR-ARFCN, the closest NR-ARFCN as specified in clause 5.4.2 applies” R4-2214973: A new text related to 4Rx is added, meanwhile delete the 4Rx MSD values for some band combination R4-2212563: Correct the incorrect reference clause numbers. R4-2214070: Change hanging paras to .0 sub clause R4-2214881: Define frequency error and carrier leakage per layer. Add general clause for UL MIMO transmit signal quality |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains unclear and incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.2, 5.2C, 6.2.3.1, 6.3,6.3.4.3, 6.3.4.4, 6.4.2, 6.4D.0, 6.4D.1, 6.4D.2.2, 6.5.3.2, 6.5A, 6.5A.0, 7.3A.1, 7.3A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a lower order band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.

<<<  End of changes  >>>

<<<  Start of changes  >>>

## 5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a lower order band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.

<<<  End of changes  >>>

<<<  Start of changes  >>>

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same AMPR as Outer RB allocations. In absense of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK  is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{10 log10(1 + NRB_gap / NRB_alloc), 0.5} dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 4) | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.7 |
| NS_04 | 6.5.2.3.2, 6.5.3.3.1 | n41 | 10, 15, 20, 40, 50, 60 80, 90, 100 |  | Clause 6.2.3.2 |
| NS_05 | 6.5.3.3.4 | n1, n84 | 5, 10, 15, 20 (NOTE 2) |  | Clause 6.2.3.4 |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n84 (NOTE 1) | 5, 10, 15, 20 |  | Clause 6.2.3.4 |
| NS_06 | 6.5.2.3.4 | n12 | 5, 10, 15 |  | N/A |
| NS_10 |  | n20, n82 | 15, 20 | Table 6.2.3.3-1 | Table 6.2.3.3-1 |
| NS_17 | 6.5.3.3.2 | n28, n83 | 5,10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 |
| NS_35 | 6.5.2.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.6 | n74 (NOTE 3) | 10, 15 | Table 6.2.3.8-1 | Table 6.2.3.8-1 |
| NS_38 | 6.5.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.9-1 | Table 6.2.3.9-1 |
| NS_39 | 6.5.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.10-1 | Table 6.2.3.10-1 |
| NS_40 | 6.5.3.3.9 | n51 | 5 |  | Table 6.2.3.5-1 |
| NS_41 | 6.5.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.11-1 |
| NS_42 | 6.5.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.12-1 |
| NS_43 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_43U | 6.5.3.3.5, 6.5.2.4.2 | n8, n81 (NOTE 1) | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_50 | 6.5.3.3.16 | n39 | 25, 30, 40 |  | Clause 6.2.3.19 |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n25, n66, n80, n81, n84, n86 (NOTE 1) |  |  | Table 6.2.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the upper channel edge is ≥ 1930 MHz,10 MHz BWChannel where the upper channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the upper channel edge is ≥ 1955 MHz and 20 MHz BWChannel where the upper channel edge is ≥ 1970 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: The NS_01 label with the field additionalPmax [7] absent is default for all NR bands. |  |  |  |  |  |

Table 6.2.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 |  |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 |  |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 |  |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |

<<<  End of changes  >>>

<<<  Start of changes  >>>

#### 6.3.4.3	Relative power tolerance

The relative power tolerance is the ability of the UE transmitter to set its output power in a target sub-frame (1 ms) relatively to the power of the most recently transmitted reference sub-frame (1 ms) if the transmission gap between these sub-frames is less than or equal to 20 ms.

The minimum requirements specified in Table 6.3.4.3-1 apply when the power of the target and reference sub-frames are within the power range bounded by the minimum output power as defined in clause 6.3.1 and the measured PUMAX as defined in clause 6.2.4.

To account for RF Power amplifier mode changes, 2 exceptions are allowed for each of two test patterns. The test patterns are a monotonically increasing power sweep and a monotonically decreasing power sweep over a range bounded by the requirements of minimum power and maximum power specified in clauses 6.3.1 and 6.2.1, respectively. For those exceptions, the power tolerance limit is a maximum of ± 6.0 dB in Table 6.3.4.3-1.

<<<  End of changes  >>>

<<<  Start of changes  >>>

#### 6.3.4.4	Aggregate power tolerance

The aggregate power control tolerance is the ability of the UE transmitter to maintain its power in a sub-frame (1 ms) during non-contiguous transmissions within 21 ms in response to 0 dB commands with respect to the first UE transmission and all other power control parameters as specified in TS 38.213 [8] kept constant.

The minimum requirement specified in Table 6.3.4.4-1 apply in the power range bounded by the minimum output power as specified in clause 6.3.1 and the maximum output power as specified in clause 6.2.1.

<<<  End of changes  >>>

<<<  Start of changes  >>>

### 6.4.2	Transmit modulation quality

### 6.4.2.0	General

Transmit modulation quality defines the modulation quality for expected in-channel RF transmissions from the UE. The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage

-	In-band emissions for the non-allocated RB

All the parameters defined in clause 6.4.2 are defined using the measurement methodology specified in Annex F.

In case the parameter 3300 or 3301 is reported from UE via the parameter txDirectCurrentLocation in UplinkTxDirectCurrentList IE (as defined in TS 38.331 [7]), carrier leakage measurement requirement in clause 6.4.2.2 and 6.4.2.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

<<<  End of changes  >>>

<<<  Start of changes  >>>

## 6.4D	Transmit signal quality for UL MIMO

### 6.4D.0	General

For a UE supporting UL MIMO, the requirements in this section are defined per layer or as the sum of emissions from both antennas to account for the UL MIMO scheme.

Alternatively, when applicable, requirements may be verified per antenna connector using an UL MIMO transmission with codebook of and a configuration defined in Table 6.4D.0-1.

Table 6.4D.0-1: UL MIMO configuration for per connector measurements

| Transmission scheme | DCI format | Codebook Index |
| --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | Codebook index 0 |

### 6.4D.1	Frequency error for UL MIMO

For UE(s) supporting UL MIMO, the basic measurement interval of modulated carrier frequency is 1 UL slot.  The mean value of basic measurements of UE modulated carrier frequency per layer shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the NR Node B.

### 6.4D.2	Transmit modulation quality for UL MIMO

### 6.4D.2.0	General

For UE supporting UL MIMO, the transmit modulation quality requirements are specified based on measurements made at each transmit antenna connector.

If UE is configured for transmission on single-antenna port, the requirements specified for single carrier apply.

The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage (caused by IQ offset)

-	In-band emissions for the non-allocated RB

In case the parameter 3300 or 3301 is reported from UE via the parameter txDirectCurrentLocation in UplinkTxDirectCurrentList IE (as defined in TS 38.331 [7]), carrier leakage measurement requirement in clause 6.4D.2.2 and 6.4D.2.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

#### 6.4D.2.1	Error Vector Magnitude

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Error Vector Magnitude requirements specified in clause 6.4.2.1 apply per layer. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2

#### 6.4D.2.2	Carrier leakage

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Relative Carrier Leakage Power requirements specified in Table 6.4.2.2-1 which is defined in clause 6.4.2.2 apply per layer. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2

#### 6.4D.2.3	In-band emissions

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the In-band Emission requirements specified in Table 6.4.2.3-1 which is defined in clause 6.4.2.3 apply at each transmit antenna connector. The requirements shall be met with the uplink MIMO configurations specified in Table 6.2D.1-2

#### 6.4D.2.4	EVM equalizer spectrum flatness for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the EVM Equalizer Spectrum Flatness requirements specified in clause 6.4.2.4 apply per layer. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2

<<<  End of changes  >>>

<<<  Start of changes  >>>

### 6.5.3	Spurious emissions

### 6.5.3.0	General

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 [9] and NR operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<<<  End of changes  >>>

<<<  Start of changes  >>>

#### 6.5.3.2	Spurious emissions for UE co-existence

This clause specifies the requirements for NR bands for coexistence with protected bands.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n1, n84 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| n2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n3, n80 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76. NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, 52, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 18, 19, 24, 25, 26, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 52, 53 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| n8, n81 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| n20, n82 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| n25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n28, n83 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 73 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| n34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 74, 75, 76, NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| n39 | E-UTRA Band 1, 8, 22, 26, 34, 40, 41, 42, 44, 45, 50, 51, 52, 74, NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 15, 26, 33 |
| n40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8 |
| n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8 |
| n50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| n51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 65, 66, 67, 68 | FDL_low | - | FDL_high | -50 | 1 |  |
| n66, n86 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| n74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 42 |
|  | Frequency range | 1475 | - | 1488 | -28 | 1 | 15, 42 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 15, 45 |
|  | Frequency range | 1475.9 | - | 1510.9 | -35 | 1 | 15, 46 |
| n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 32, 34, 39, 40, 41, 65, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: 15 kHz SCS is assumed when RB is mentioned in the note when channel bandwidth is less than or equal to 50MHz, lowest SCS is assumed when channel bandwidth is larger than 50 MHz. The transmission bandwidth in terms of RB position and range is not limited to 15 kHz SCS and shall scale with SCS accordingly. NOTE 4: Void NOTE 5: For non-synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: N/A NOTE 7: Void NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 9: Void NOTE 10: Void NOTE 11: Void NOTE 12: Void NOTE 13: Void NOTE 14: Void NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 16: Void NOTE 17: Void NOTE 18: Void NOTE 19: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: Void NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: Void NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: Void NOTE 29: Void NOTE 30: Void NOTE 31: Void NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth up to 20MHz and confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718-728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 35: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: Void NOTE 37: Void NOTE 38: Void NOTE 39: Void NOTE 40: Void NOTE 41: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power of 15 dBm. NOTE 42: Applicable when upper edge of the assigned NR UL channel bandwidth frequency is more than 1460 MHz and less than or equal to 1470 MHz for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is more than 1460 MHz and less than or equal to 1465 MHz for 10 MHz bandwidth. NOTE 43: Void NOTE 44: Void NOTE 45: Applicable when upper edge of the assigned NR UL channel bandwidth frequency is equal to or less than 1460 MHz. NOTE 46: Applicable for 5 MHz bandwidth and when the NR carrier is within 1447.9 – 1462.9 MHz. |  |  |  |  |  |  |  |

<<<  End of changes  >>>

<<<  Start of changes  >>>

## 6.5A	Output RF spectrum emissions for CA

### 6.5A.0	General

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the output RF spectrum emissions requirements in clause 6.5 apply.

<<<  End of changes  >>>

<<<  Start of changes  >>>

### 7.3A.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. For operations with 4 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,4R in Table 7.3.2-2 when MSD > 0.

For reference sensitivity exception test points where the specified carrier frequency does not correspond to a valid NR-ARFCN, the closest NR-ARFCN as specified in clause 5.4.2 applies.

<<<  End of changes  >>>

<<<  Start of changes  >>>

### 7.3A.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands given in Table 7.3A.5-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.5-1. For these test points the reference sensitivity requirement specified in Table 7.3.2-1 and Table 7.3.2-2 are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.5-1.

Table 7.3A.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | [26] | FDD | IMD24 |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1765 | 5 | 25 | 1860 | [8.0] | FDD | IMD44 |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 2: RBSTART = 0, 15 kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. |  |  |  |  |  |  |  |  |

<<<  End of changes  >>>