3GPP TSG-RAN WG4 Meeting # 104b-e	R4-2215333

# Electronic Meeting, October 10 - 19, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Band 54 protection from bands n24, n66, n70 and n255

Agenda Item:	7.3.2	UE RF requirements	[LTE_TDD_1670_1675MHz -Core]

Document for:	Approval

# Introduction

In RAN4#104e, the issues of low rejection of existing UL filters for bands in the neighborhood of the 1670-1675MHz frequency range of band 54 resulting in poor protection levels in some cases, was discussed in [2] and initial agreements were captured in [1]. In this contribution, we discuss and define the achievable protection level of band 54 from band 24/n24/n255 filter and band 66/n66 and 70/n70 combined filter versus UL channel bandwidth.

# Discussion

## Existing filters performance and need for relaxed protection level

In way forward [1], it was agreed that a -50dBm/MHz protection level for band 54 could not be achieved by neighbor bands 24/n24/n99/n255, 70/n70 and 66/n66/n86.

This was due to the poor rejection achievable by the UL filters especially for legacy filters:

- Band 24/n24/n99/n255 UL filters did not have any DL directly above the UL band and the UL filter was optimized for the stringent protection of the GNSS system just below the UL band and thus provide almost no rejection of band 54.

- Band 66 and band 70 use a common UL filter in some implementations and thus should be the basis for the band 66 rejection which is limited due to the 25MHz downward extension when adding Band 70. Due to this limitation, only moderate rejection is achieved for band 54.

We have looked at our legacy filters as well as filters that are close to reaching production to make the following rejection proposal.

Proposal 1:

- Bands 24/n24/n99/n255 UL filter rejection of the LTE band 54 is 0dB

- Bands 70/n70 UL filter rejection of the LTE band 54 is 10dB based on a combined band 66 and band 70 UL filter

- Bands 66/n66/n86 UL filter rejection of the LTE band 54 is 10dB based on a combined band 66 and band 70 UL filter

## Achievable protection level per channel bandwidth

Based on the filter rejection of 0dB for n24 and 10dB for n70+n66 and the NR SEM mask, it is possible to derive the dBm/MHz UL PSD in band 24 for the worst UL interfering channel; in addition, it is possible to deduct at which carrier frequency offset the default -50dBm/MHz can be achieved. These derivations are presented in Table 1 for bands 24, 70 and 66 UL with 0dB filter rejection for the band 24 UL filter and 10dB rejection for bands 66+70 UL filter.

Table reading:

- Cyan separates the different interfering UL bands

- Grey rows are for NR only

- For ACLR and IMD overlap, orange cells are for partial overlap (>1MHz) and red cells are for full overlap

- The PSD in dBm/MHz is colored in function of the protection level achieved for band 54

- For the last column, the carrier frequency at which the -50dBm/MHz protection level is achievable is in red if the channel cannot fit in the band.

Table 1: Calculated protection level in B54 and carrier frequency at which -50dBm/MHz is achievable

Observations for bands 24/n24/n99/n255:

- 5MHz channel: The protection level is -40dBm/MHz while channels with Fc<1652.4MHz can achieve -50dBm/MHz

- 10MHz channel: The achievable protection level is -25dBm/MHz while channels with Fc<16535 MHz can achieve -50dBm/MHz.

Observations for bands 70/n70:

- -50dBm/MHz is achieved by all 5 and 10MHz channels (also valid for LTE)

- -35dBm/MHz is needed for all 15 and 20MHz channels (also valid for LTE)

- -20dBm/MHz is needed for all 25MHz channel (NR only).

Observations for bands 66/n66/n86:

- -50dBm/MHz is achievable by:

  - 5, 10 and 15MHz channels (also valid for LTE)

  - 20MHz channel for Fc > 1725MHz (also valid for LTE)

  - 25, 30, 35MHz channels for FC > 1735.5, 1750 and 1662.5MHZ respectively (NR only)

- -35dBm/MHz is needed for:

  - Lower 20MHz channel (also valid for LTE)

  - Lower 25, 30, 35MHz channel (also valid for LTE)

- -20dBm/MHz is needed for all 40 and 45MHz channels.

Based on the above, band protection requirement can be split into 3 levels when feasible:

- Channels BW and positions achieving -50dBm/MHz

- Channels BW and positions achieving -35dBm/MHz

- Channels BW and positions achieving -20dBm/MHz, -25dBm/MHz can be used instead for n24 as only two levels are needed

- All channel bandwidths for NR bands

- Channel bandwidths up to 20MHz for LTE bands.

## Band 54 protection level from bands 24/n25/n99/n255

Proposal 2: The following spurious coexistence limits are proposed for bands 24/n24/n99/n255 and related CA combinations to protect the new LTE band 54 (bold part).

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n24, n99 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 54 | FDL_low | - | FDL_high | -50 | 1 | X |
|  |  |  |  |  | -40 | 1 | Y |
|  |  |  |  |  | -25 | 1 | Z |
| NOTE X: Valid for 5 MHz channels with Fc < 1652.5 MHz and 10 MHz channels with Fc < 1635 MHz. NOTE Y: Valid for 5 MHz channels with Fc ≥ 1652.5 MHz. NOTE Z: Valid for 10 MHz channels with Fc ≥ 1635 MHz. |  |  |  |  |  |  |  |

## Band 54 protection level from bands 70/n70

Proposal 3: The following spurious coexistence limits are proposed for bands 70/n70 and related CA combinations to protect the new LTE band 54 (bold part) with Cyan part only for NR bands.

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 47, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 54 | FDL_low | - | FDL_high | -50 | 1 | X |
|  |  |  |  |  | -35 | 1 | Y |
|  |  |  |  |  | -20 | 1 | Z |
| NOTE X: Valid for all 5 and 10 MHz channels. NOTE Y: Valid for all 15 and 20MHz channels. NOTE Z: Valid for all 25 MHz channels. |  |  |  |  |  |  |  |

## Band 54 protection level from bands 66/n66/n86

Proposal 4: The following spurious coexistence limits are proposed for bands 66/n66/n86 and related CA combinations to protect the new band 54 (bold part) with Cyan part only for NR bands.

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n66, n86 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 54 | FDL_low | - | FDL_high | -50 | 1 | X |
|  |  |  |  |  | -35 | 1 | Y |
|  |  |  |  |  | -20 | 1 | Z |
| NOTE X: Valid for all 5, 10 and 15 MHz channels and 20, 25, 30 and 35 MHz channels with Fc > 1725, 1737.5, 1750 and 1762.5 MHz respectively. NOTE Y: Valid for 20, 25, 30 and 35 MHz channels with Fc ≤ 1725, 1737.5, 1750 and 1762.5 MHz respectively. NOTE Z: Valid for all 40 and 45MHz channels |  |  |  |  |  |  |  |

# Conclusion

In this contribution we provided calculation for the protection level for band 54 for coexistence with neighbor UL bands for different channel bandwidths and came to the following proposals.

Proposal 1:

- Bands 24/n24/n99/n255 UL filter rejection of the LTE band 54 is 0dB

- Bands 70/n70 UL filter rejection of the LTE band 54 is 10dB based on a combined band 66 and band 70 UL filter

- Bands 66/n66/n86 UL filter rejection of the LTE band 54 is 10dB based on a combined band 66 and band 70 UL filter

Proposal 2: The following spurious coexistence limits are proposed for bands 24/n24/n99/n255 and related CA combinations to protect the new LTE band 54 (bold part).

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n24, n99 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 54 | FDL_low | - | FDL_high | -50 | 1 | X |
|  |  |  |  |  | -40 | 1 | Y |
|  |  |  |  |  | -25 | 1 | Z |
| NOTE X: Valid for 5 MHz channels with Fc < 1652.5 MHz and 10 MHz channels with Fc < 1635 MHz. NOTE Y: Valid for 5 MHz channels with Fc ≥ 1652.5 MHz. NOTE Z: Valid for 10 MHz channels with Fc ≥ 1635 MHz. |  |  |  |  |  |  |  |

Proposal 3: The following spurious coexistence limits are proposed for bands 70/n70 and related CA combinations to protect the new LTE band 54 (bold part) with Cyan part only for NR bands.

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 47, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 54 | FDL_low | - | FDL_high | -50 | 1 | X |
|  |  |  |  |  | -35 | 1 | Y |
|  |  |  |  |  | -20 | 1 | Z |
| NOTE X: Valid for all 5 and 10 MHz channels. NOTE Y: Valid for all 15 and 20MHz channels. NOTE Z: Valid for all 25 MHz channels. |  |  |  |  |  |  |  |

Proposal 4: The following spurious coexistence limits are proposed for bands 66/n66/n86 and related CA combinations to protect the new band 54 (bold part) with Cyan part only for NR bands.

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n66, n86 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 54 | FDL_low | - | FDL_high | -50 | 1 | X |
|  |  |  |  |  | -35 | 1 | Y |
|  |  |  |  |  | -20 | 1 | Z |
| NOTE X: Valid for all 5, 10 and 15 MHz channels and 20, 25, 30 and 35 MHz channels with Fc > 1725, 1737.5, 1750 and 1762.5 MHz respectively. NOTE Y: Valid for 20, 25, 30 and 35 MHz channels with Fc ≤ 1725, 1737.5, 1750 and 1762.5 MHz respectively. NOTE Z: Valid for all 40 and 45MHz channels |  |  |  |  |  |  |  |

# References

- R4-2214440	WF on Introduction of LTE TDD band in 1670 – 1675 MHz, RAN4#104e

- R4-2212164 Coexistence aspects for the new 1670-1675MHz TDD band, Skyworks Solutions, Inc., RAN4#104e