3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215334

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	9.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Impact of NS mapping between RAN2 and RAN4 specs on n77 issue

Document for:	Approval

# 1	Introduction

RAN4#104-e approved WF on clarification of the NS mapping from RAN2 to RAN4 [1] where a following WF is captured.

In this contribution, we discuss which option we should take.

# 2	Clarification of the issue

## 2.1	Relation b/w RAN4 and RAN2 spec

Generally, CA_NS_01 and CA_NC_NS_01 are defined for all UL CA cases to indicate A-MPR is not used and only MPR applies. For FR1, currently there are intra-band CA_NS requirements in TS38.101-1 for bands n7, n41 and n48 as shown below:

Since the whole concept of NS-values was copy-pasted from LTE, it’s good to check also how it was handled there. And indeed, LTE RRC makes the CA case clearer: It is explicitly said that the same NS-value is used for all SCells, but that value is then applicable for all serving cells including PCell when UL CA is configured, and the HO command PCell NS-value is only used in non-UL CA cases, as shown in below excerpts from LTE RRC:

RadioResourceConfigCommon field descriptions

additionalSpectrumEmissionSCell The UE requirements related to additionalSpectrumEmissionSCell are defined in TS 36.101 [42]. E-UTRAN configures the same value in additionalSpectrumEmissionSCell for all SCell(s) of the same band with UL configured. The additionalSpectrumEmissionSCell is applicable for all serving cells (including PCell) of the same band with UL configured.

PhysicalConfigDedicated field descriptions

additionalSpectrumEmissionPCell E-UTRAN does not configure this field in this release of the specification.

MobilityControlInfo field descriptions

additionalSpectrumEmission For a UE with no SCells configured for UL in the same band as the PCell, the UE shall apply the value for the PCell instead of the corresponding value from SystemInformationBlockType2 or SystemInformationBlockType1. For a UE with SCell(s) configured for UL in the same band as the PCell, the UE shall, in case all SCells configured for UL in that band are released after handover completion, apply the value for the PCell instead of the corresponding value from SystemInformationBlockType2 or SystemInformationBlockType1. The UE requirements related to IE AdditionalSpectrumEmission are defined in TS 36.101 [42], table 6.2.4-1, for UEs neither in CE nor BL UEs and TS 36.101 [42], table 6.2.4E-1, for UEs in CE or BL UEs.

This means that the LTE specifications are written so that RRC uses the CA NS-values already, which was presumably the intention also for NR, it just wasn’t captured very clearly when the NR field description in RAN2 specification, i.e., TS38.331, was created.

Observation 1: In LTE RRC, the NS-value for PCell which is obtained in single carrier or after HO is not used when UL CA is configured and the NS-value is configured only for SCell and the NS-value for SCell is applicable to PCell instead. In other words, an NS-value for PCell follows an NS-value configured for SCells when UL CA is configured.

In our view, the NR intent should have been the same as in LTE, i.e. the UL CA requirement in the additionalSpectrumEmission only relates to the NS-value for SCells, and PCell value just follows the value configured for SCells when UL CA is configured.

Observation 2: NS value handling in NR specifications should follow the same way as LTE spec

## 2.2	Whether RAN2 or RAN4 spec should be fixed?

In our view, it’s better and easier to fix RAN2 specification in following reasons. Firstly, in RAN4 specifications, CA_NS and CA_NC_NS are referred to by several sections, 6.5A.2.3(Additional spectrum emission mask for CA) or  6.5A.3.3(Additional spurious emissions for CA), and corresponding A-MPR table in 6.2A.3. Stopping using CA_NS and CA_NC_NS and somehow tying NS for single carrier with them would require huge changes in RAN4 specifications.  On the other hand, normally RAN2 specifications are band agnostic so that the required changes are not that much. It’s noted that a specific example for required RAN2 spec changes is shown in Annex.

Proposal 1: Adopt option 2 and send an LS to RAN2 to share RAN4’s preference.

The last, but not the least, in order to avoid re-discussion on n77 US and Canada issues again, it must be better for RAN4 to clarify that RAN4 has no intention to turn over the agreement [2, 3] on n77 US and Canada issues made in RAN2 in RAN2#119-e in the above LS since strictly following LTE specification leads to exclude the agreement.

Proposal 2: Clarify that RAN4 has no intention to turn over the agreement on n77 US and Canada issues made in RAN2 in RAN2#119-e in the LS.

Accordingly, we don’t see any reasons to fix the newly inserted text in the agreed CRs in [2-4] in terms of NS mapping issue.

Proposal 3: No RAN4 spec changes are needed due to RAN2 spec changes to fix NS mapping issue.

# 3	Conclusion

This document has made the following observations:

Observation 1: In LTE RRC, the NS-value for PCell which is obtained in single carrier or after HO is not used when UL CA is configured and the NS-value is configured only for SCell value and the NS-value for SCell is applicable to PCell instead. In other words, an NS-value for PCell follows an NS-value configured for SCells when UL CA is configured.

Observation 2: NS value handling in NR specifications should follow the same way as LTE spec

And proposed the following:

Proposal 1: Adopt option 2 and send an LS to RAN2 to share RAN4’s preference.

Proposal 2: Clarify that RAN4 has no intention to turn over the agreement on n77 US and Canada issues made in RAN2 in RAN2#119-e in the LS.

Proposal 3: No RAN4 spec changes are needed due to RAN2 spec changes to fix NS mapping issue.

# 4	References

[1] R4-2214409, WF on clarification of the NS mapping from RAN2 to RAN4, Qualcomm Incorporated, RAN4#104-e

[2] R2-2209137, Correction to additionalSpectrumEmission for UL CA in n77 for the US, Ericsson, Nokia, Nokia Shanghai Bell, T-Mobile USA, TSG RAN WG2 #119-e

[3] R2-2209139, Correction to additionalSpectrumEmission for UL CA in n77 for Canada, Ericsson, Nokia, Nokia Shanghai Bell, T-Mobile USA, TSG RAN WG2 #119-e

# Annex A: RRC CR to clarify the UL CA NS-value usage

#### –	AdditionalSpectrumEmission

The IE AdditionalSpectrumEmission is used to indicate emission requirements to be fulfilled by the UE (see TS 38.101-1 [15], clause 6.2.3/6.2A.3, and TS 38.101-2 [39], clause 6.2.3/6.2A.3).

AdditionalSpectrumEmission information element

-- ASN1START

-- TAG-ADDITIONALSPECTRUMEMISSION-START

AdditionalSpectrumEmission ::=              INTEGER (0..7)

-- TAG-ADDITIONALSPECTRUMEMISSION-STOP

-- ASN1STOP

<UNNECESSARY PARTS OMITTED>

#### –	FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-START

FrequencyInfoUL ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR                                OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission          AdditionalSpectrumEmission                              OPTIONAL,   -- Need S

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...

}

-- TAG-FREQUENCYINFOUL-STOP

-- ASN1STOP

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList (see TS 38.211 [16], clause 4.4.4.2).

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If the field is absent, the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15], table 6.2.3.1-1A, 6.2.3.1.1-2 and 6.2A.3.1.2-2, and TS 38.101-2 [39], table 6.2.3.1-2 and 6.2A.3.1-2). Network configures the same value in additionalSpectrumEmission for all SCells of the same band that have UL configured, except for additionalSpectrumEmission value corresponding to NS_55/NS_57. If NS_55/NS_57 (see TS 38.101-1 [15], table 6.2.3.1-1) is applicable for at least one uplink carrier, the network may configure either NS_55/NS_57 or NS_01 for these uplink carriers, and NS_01 for the remaining uplink carrier(s) of band n77. When SCell with uplink is configured, the additionalSpectrumEmission (as defined in TS 38.101-1 [15], 6.2.3.1.1-2 and 6.2A.3.1.2-2 and TS 38.101-2 [39], 6.2A.3.1-2) is applicable for all serving cells (including PCell) with uplink of the same band .

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Maximum transmit power allowed in this serving cell. The maximum transmit power that the UE may use on this serving cell may be additionally limited by p-NR-FR1 (configured for the cell group) and by p-UE-FR1 (configured total for all serving cells operating on FR1). If absent, the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]. Value in dBm. This field is ignored by IAB-MT, the IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise (if this FrequencyInfoUL is for an unpaired UL (TDD). |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL), or if this FrequencyInfoUL is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise. |