3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215339

# Electronic Meeting, October 10th-19th, 2022

# Title:	TP for TS 38.181 - Clause 6.5.3 EVM

# Source:	THALES, CATT

# Type:	pCR

# Document for:	Decision

# Agenda Item:	4.2.3.2.1

Release	Rel-17

# Introduction

The following Satellite 5G NR (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

The conformance testing in TS 38.181 is currently under work. This contribution therefore provides a Text Proposal for SAN EVM conformance testing. For further information see TS 38.181 and agreed requirements in TS 38.108 as part of NTN Rel-17 WI.

In this proposal, the previous contribution from TP R4-2214834 (THALES) is updated accordingly taking into account current TM.

# Text Proposal

Conformance testing is an important part of 3GPP work, and is currently under work in TS 38.181. This section is to propose a TP for Satellite Access Node (SAN) with the requirements described in TS 38.108.

In this proposal, the previous contribution from TP R4-2214834 (THALES) is updated accordingly taking into account current TM.

>>>>>>>>>>>>>>>>>>>>>>>------- Start Text Proposal --------<<<<<<<<<<<<<<<<<<<<<

## 6.5	Transmitted signal quality

### 6.5.3	Modulation quality

#### 6.5.3.1	Definition and applicability

Modulation quality is defined by the difference between the measured carrier signal and an ideal signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector.

For SAN type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

#### 6.5.3.2	Minimum Requirement

The minimum requirement is in TS 38.108 [x], clause 6.5.2.2.

#### 6.5.3.3	Test purpose

The test purpose is to verify that modulation quality is within the limit specified by the minimum requirement.

#### 6.5.3.4	Method of test

##### 6.5.3.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: B, M and T; see clause 4.9.1.

RF bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single-band operation, see clause 4.9.1.

##### 6.5.3.4.2	Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.3.1 for SAN type 1-H. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	For a TAB connector declared to be capable of single carrier operation only ([D.16]), set the TAB connector under test to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models:

-	NR-FR1-TM3.1 if 64QAM is supported by BS without power back off, or

-	NR-FR1-TM3.1 at manufacturer's declared rated output power if 64QAM is supported by BS with power back off, and NR-FR1-TM3.2 at maximum power, or

- 	NR-FR1-TM3.2 if highest modulation order supported by SAN is 16QAM, or

-	NR-FR1-TM3.3 if highest modulation order supported by SAN is QPSK.

For TAB connector declared to be capable of multi-carrier operation, set the TAB connector under test to transmit according to the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models on all carriers configured:

-	NR-FR1-TM3.1 if 64QAM is supported by BS without power back off, or

-	NR-FR1-TM3.1 at manufacturer's declared rated output power if 64QAM is supported by BS with power back off, and NR-FR1-TM3.2 at maximum power, or

-	NR-FR1-TM3.2 if highest modulation order supported by SAN is 16QAM, or

-	NR-FR1-TM3.3 if highest modulation order supported by SAN is QPSK.

For NR-FR1-TM3.1, power back-off shall be applied if it is declared.

2)	Measure the EVM and frequency error as defined in [annex H].

#### 6.5.3.5	Test requirements

The EVM of each NR carrier for different modulation schemes on PDSCH shall be less than the limits in table 6.5.3.5-1.

Table 6.5.3.5-1 EVM requirements for SAN type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM (NOTE) | 8 % |
| NOTE: EVM requirement for 64QAM is optional. |  |

EVM shall be evaluated for each NR carrier over all allocated resource blocks and downlink slots. Different modulation schemes listed in table 6.5.3.5-1 shall be considered for rank 1.

For all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and downlink slots within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

Table 6.5.3.5-2, 6.5.3.5-3, 6.5.3.5-4 below specify the EVM window length (W) for normal CP for SAN type 1-H.

Table 6.5.3.5-2 EVM window length for normal CP for NR, FR1, 15 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | 14 | 40 |
| 10 | 1024 | 72 | 28 | 40 |
| 15 | 1536 | 108 | 44 | 40 |
| 20 | 2048 | 144 | 58 | 40 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 to 6 and 8 to 13. Symbols 0 and 7 have a longer CP and therefore a lower percentage. |  |  |  |  |

Table 6.5.3.5-3 EVM window length for normal CP for NR, FR1, 30 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 256 | 18 | 8 | 40 |
| 10 | 512 | 36 | 14 | 40 |
| 15 | 768 | 54 | 22 | 40 |
| 20 | 1024 | 72 | 28 | 40 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 through 13. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |  |

Table 6.5.3.5-4 EVM window length for normal CP for NR, FR1, 60 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) |
| --- | --- | --- | --- | --- |
| 10 | 256 | 18 | 8 | 40 |
| 15 | 384 | 27 | 11 | 40 |
| 20 | 512 | 36 | 14 | 40 |
| NOTE: These percentages are informative and apply to all OFDM symbols within subframe except for symbol 0 of slot 0 and slot 2. Symbol 0 of slot 0 and slot 2 may have a longer CP and therefore a lower percentage. |  |  |  |  |

>>>>>>>>>>>>>>>>>>>>>>>------- End Text Proposal --------<<<<<<<<<<<<<<<<<<<<<

# Conclusions

TP for TS 38.181, Clause 6.5.3 EVM.

# References

- TS 38.108, Rel-17

- TS 38.181, Rel-17

- R4-2214834 (THALES)

-END-