3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215348

# Electronic Meeting, October 10th-19th, 2022

# Title:	VSAT UE Characteristics and Initial Simulation Parameters

# Source:	THALES

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	6.22.2

Release	Rel-18

# Introduction

The following Satellite 5G NR (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

From Article 5 of the ITU Radio Regulations (RR) the mobile operation of ESIMs is also described in the following footnotes,

- 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15). (WRC-15)

- 5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). (WRC-19)

This contribution therefore provides material for discussion with respect to Ka-band coexistence simulations and VSAT antenna parameters.

# Discussion

## 2.1 General aspects

All VSAT UE antenna parameters have been adapted from TR 38.821 by considering the following parameters adapted to Ka satellite band:

- Downlink frequency: 17.7 - 20.2 GHz

- Uplink frequency: 27.5 GHz - 30 GHz

- 60 cm antenna aperture diameter (2*a=60cm)

The Technical Report TR 38.811 provides typical RF parameters shown in the table below:

Table 1. Typical minimum RF characteristics of NTN VSAT UE

|  | Very Small Aperture Terminal (VSAT) (fixed or mounted on moving platforms) |
| --- | --- |
| Transmit Power | 2 W (33 dBm) |
| Antenna type | 60 cm equivalent aperture diameter (circular polarisation) |
| Antenna gain | Tx: 43.2 dBi Rx: 39.7 dB |
| Noise figure | 1.2 dB |
| EIRP | 45.75 dBW |
| G/T (NOTE 1) | 18.5 dB/K |
| Polarisation (NOTE 2) | Circular |
| NOTE 1: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta - Noise Figure: NF in dB including feeder loss |  |

## 2.2 VSAT UE antenna parameters

The following normalised antenna pattern corresponding to a theoretical reflector antenna with circular aperture can be considered for coexistence analysis:

Fθ=2J1uu

Where:

- J1x  is the Bessel function of first type and first order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Figure 1  shows the normalised antenna pattern of a VSAT transmit antenna operating at  28 750 MHz.

Figure 1. Normalised antenna pattern of a VSAT transmit antenna operating at 29 750 MHz

The VSAT transmit and receive performances are summarised in the Table 2.

Table 2. VSAT UE Parameters

| VSAT UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power | (W) | 2 |  |
| Output power | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| EIRP |  | 44,9 |  |
| Receiver noise figure | (dB) |  | 1,2 |
| Feeder loss | (dB) |  | -0,50 |
| Sky temperature | (K) |  | 30 |
| Ground temperature | (K) |  | 10 |
| Antenna temperature | (K) |  | 40 |
| G/T figure of merit | (dB/K) |  | 16,5 |

NOTE1: T_a = T_Sky + T_Ground

NOTE2: The antenna temperatures are based on e.g. ITU-R Rec. P372 and Rec. P618.

NOTE3: T_sky is computed using [ITU-R Rec. P.618-12] as expressed below

With respect to the adjacent antenna lobes (i.e. adjacent antenna lobes to the main directional lobes, or secondary antenna lobes from the antenna radiation pattern), the following recommendation can be used (see [ITU-R S.465-5]):

## 2.3 Link Budget Parameters

With respect to link budget parameters, is proposed that RAN4 should follow similar methodology as in TR 38.863, with the adaptations required for Ka band with channel models/link budget parameters described in TR 38.811 and TR 38.821.

Proposal 3: With respect to link budget analysis for Ka band, RAN4 to use explained methodology from TR 38.821, TR 38.811.

# Conclusions

Proposal 1: For the coexistence analysis in Ka band, RAN4 to use the following VSAT UE parameters

Table x. VSAT UE Parameters

| VSAT UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power | (W) | 2 |  |
| Output power | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| EIRP |  | 44,9 |  |
| Receiver noise figure | (dB) |  | 1,2 |
| Feeder loss | (dB) |  | -0,50 |
| Sky temperature | (K) |  | 30 |
| Ground temperature | (K) |  | 10 |
| Antenna temperature | (K) |  | 40 |
| G/T figure of merit | (dB/K) |  | 16,5 |

Proposal 2: With respect to VSAT UE secondary lobes and related coexistence analysis, RAN4 to use the recommendation from ITU-R S.465-5:

Proposal 3: With respect to link budget analysis for Ka band, RAN4 to use explained methodology from TR 38.821, TR 38.811.

# References

- R4-2213361, “Discussion on Ka adjacent band NTN-TN NR coexistence scenarios”

- TR 38.811

- TR 38.821

- ITU-R Rec. P.618-12

- ITU-R Rec. S.465-5

-END-