3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215352

# Electronic Meeting, October 10th-19th, 2022

# Title:	Discussion on Ka-band NTN-TN NR adjacent band coexistence scenarios

# Source:	THALES, Lockheed Martin, Hispasat, Intelsat, Magister Solutions Ltd, Satellite Applications Catapult, ESA, Avanti, Hughes/EchoStar, Inmarsat, Eutelsat, Sateliot

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	6.22.2

Release	Rel-18

# Introduction

The following Satellite 5G NR (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

From Article 5 of the ITU Radio Regulations (RR) the mobile operation of ESIMs is also described in the following footnotes,

- 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15). (WRC-15)

- 5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). (WRC-19)

Based on WID for NR NTN (Non-Terrestrial Networks) enhancements NR-NTN deployment in above 10 GHz bands (RP-222654), the following assumptions are taken a baseline for this work:

- GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered

- ESIM scenarios for NGSO in Ka band are not considered in this WI.

- Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified

- Regarding mobile VSAT, three types of terminal and scenario exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study.

- FDD mode is assumed for satellite operation above 10 GHz, while TDD mode is assumed for terrestrial operation in FR2

- The ITU-R harmonized Ka band will serve as reference

- Co-existence between overlapping NTN and TN band portions is out of scope of this work item. This aspect will be captured in the specification.

This contribution therefore provides material for discussion on handling of satellite FR2 bands as part of NTN Rel-18 WI.

# Discussion

## 2.1 Allocated satellite frequency bands in between FR1, FR2

The current 3GPP NR Band range is defined as following:

- FR1 with the 410 MHz – 7125 MHz frequency range;

- FR2 with the 24250 MHz – 52600 MHz (24.25 GHz – 52.6 GHz) frequency range.

The following bands are commonly referred to as “Ka-band” and identified by ITU Radio Regulation for use by Geo-stationary Orbit (GEO) and Non-Geostationary Orbit (NGSO) based satellite services on a worldwide basis, historically denominated as “FSS” and “MSS” (see RP-210439). The duplex mode for all satellite bands is FDD and therefore the bands are distinctly assigned to UL and DL ranges. These are allocated by ITU and are subject to the provisions of ITU Radio Regulation Article 5 of the ITU Radio Regulation. These frequencies will henceforth be collectively referred to as “satellite Ka-band” throughout the rest of this document.

Table 0.1 - “Ka-band” spectrum for GEO based Satellite services

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space-earth) | 17.3 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth-space) | 27.5 – 30.0 GHz | 27.0 – 30.0 GHz | 27.0 – 30.0 GHz |

Table 0.2 - “Ka-band” spectrum for NGSO Satellite services

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space-earth) | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth-space) | 27.5 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz |

Observation 1: Based on the current range, some part of the satellite service allocated frequency bands for both GEO and NGSO fall outside of the FR1 and FR2 range. As shown in the diagram below, these include:

- Ka-Band DL

- Ku-Band UL and DL

Figure 1. Illustration of FR1, FR2, bands

While an exemplary Ka-band has been agreed for co-existence studies, other satellite bands such as the Ku-band and Q/V-bands will follow a similar development in the future.

## 2.2 NTN-TN Ka band coexistence analysis for FR2

The harmonized Satellite Ka-band refers to [17.7 – 20.2 GHz] on the downlink and [27.5 – 30.0 GHz] on the uplink.

This harmonized Ka satellite band is further proposed for NTN coexistence analysis with TN n258.

Observation 2: The harmonized Satellite Ka-band refers to [17.7 – 20.2 GHz] on the downlink and [27.5 – 30.0 GHz] on the uplink.

Proposal 1: RAN4 shall consider FDD harmonized Satellite Ka-band (17.7 – 20.2 GHz on the downlink, and 27.5 – 30.0 GHz on the uplink) for coexistence analysis.

Proposal 2: RAN4 shall focus on FDD UL 27.5 – 30.0 GHz NTN coexistence analysis with TDD TN n258 (24.250 - 27.5 GHz).

RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands [RAN4] (see RP-222654). Therefore, for co-existence studies in Ka-band the following Table 1 shall be considered:

Table 1 - TN-NTN coexistence scenarios in adjacent bands for Ka-band

| No. | NTN Frequency | TN Frequency | TN scenario | NTN |
| --- | --- | --- | --- | --- |
| 1 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Urban macro | GEO |
| 2 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Urban macro | LEO@600km |
| 3 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Rural macro | GEO |
| 4 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Rural macro | LEO@600km |

Similarly, in order to leverage the work in FR1 (see RP-222654), FR2 studies related to NTN-NTN coexistence should be deprioritized.

Proposal 3: Leveraging the work in FR1, RAN4 shall focus on Ka-band coexistence analysis using urban and rural macro scenarios.

Proposal 4: Leveraging the work in FR1, FR2 studies related to NTN-NTN coexistence should be deprioritized.

Proposal 5: RAN4 shall focus on Ka-band coexistence analysis with LEO 600km and GEO constellations.

Proposal 6: RAN4 shall consider the following table for co-existence studies in Ka-band:

Table x - TN-NTN coexistence scenarios in adjacent bands for Ka-band

| No. | NTN Frequency | TN Frequency | TN scenario | NTN |
| --- | --- | --- | --- | --- |
| 1 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Urban macro | GEO |
| 2 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Urban macro | LEO@600km |
| 3 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Rural macro | GEO |
| 4 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Rural macro | LEO@600km |

The co-existence examples are further illustrated by the following figure,

Figure 2. Different interference scenarios in Ka adjacent bands between NTN 5G NR and TN 5G NR

As illustrated in Figure 2 (see also [1]), the interference scenarios in adjacent bands are identified as follows:

- i1: DL TN 5G NR to UL NTN 5G NR

- i2: UL NTN 5G NR to DL TN 5G NR

- i3: UL NTN 5G NR to UL TN 5G NR

- i4: UL TN 5G NR to UL NTN 5G NR

Please also note that interference types i1 and i2 in adjacent bands are UL-DL (since NTN networks are FDD while TN networks are TDD), while interference types i3 and i4 in adjacent bands are UL-UL. Moreover, interference type i4 can be further down-scoped from coexistence cases.

Proposal 7: RAN4 shall consider the following figure for co-existence studies in Ka-band:

Figure x. Different interference scenarios in Ka adjacent bands between NTN 5G NR and TN 5G NR

Observation 3: It can be noted that for FR2, coexistence scenarios will be much simpler than for FR1, because the NTN-TN scenarios for FR2 are limited only to i1 (DL TN in UL NTN), i2 (UL NTN in DL TN), i3 (UL NTN in UL TN).

# Conclusions

Observation 1: Based on the current range, some part of the satellite service allocated frequency bands for both GEO and NGSO fall outside of the FR1 and FR2 range. As shown in the diagram below, these include:

- Ka-Band DL

- Ku-Band UL and DL

Figure 1. Illustration of FR1, FR2, bands

Observation 2: The Harmonized Satellite Ka-band refers to [17.7 – 20.2 GHz] on the downlink and [27.5 – 30.0 GHz] on the uplink.

Proposal 1: RAN4 shall consider FDD harmonized Satellite Ka-band (17.7 – 20.2 GHz on the downlink, and 27.5 – 30.0 GHz on the uplink) for coexistence analysis.

Proposal 2: RAN4 shall focus on FDD UL 27.5 – 30.0 GHz NTN coexistence analysis with TDD TN n258 (24.250 - 27.5 GHz).

Proposal 3: Leveraging the work in FR1, RAN4 shall focus on Ka-band coexistence analysis using urban and rural macro scenarios.

Proposal 4: Leveraging the work in FR1, FR2 studies related to NTN-NTN coexistence should be deprioritized.

Proposal 5: RAN4 shall focus on Ka-band coexistence analysis with LEO 600km and GEO constellations.

Proposal 6: RAN4 shall consider the following table for co-existence studies in Ka-band:

Table x - TN-NTN coexistence scenarios in adjacent bands for Ka-band

| No. | NTN Frequency | TN Frequency | TN scenario | NTN |
| --- | --- | --- | --- | --- |
| 1 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Urban macro | GEO |
| 2 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Urban macro | LEO@600km |
| 3 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Rural macro | GEO |
| 4 | 27.5 GHz-30 GHz | NR n258 (24.250 - 27.5 GHz) | Rural macro | LEO@600km |

Proposal 7: RAN4 shall consider the following figure for co-existence studies in Ka-band:

Figure x. Different interference scenarios in Ka adjacent bands between NTN 5G NR and TN 5G NR

Observation 3: It can be noted that for FR2, coexistence scenarios will be much simpler than for FR1, because the NTN-TN scenarios for FR2 are limited only to i1 (DL TN in UL NTN), i2 (UL NTN in DL TN), i3 (UL NTN in UL TN).

# References

- R4-2107217, “On the FR2 NTN coexistence scenarios”

- R4-2213361, “Discussion on Ka adjacent band NTN-TN NR coexistence scenarios”

- RP-210439, “Ka-band proposed as exemplary band for NR-NTN in Rel-17”

- RP-222654, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements

-END-