3GPP TSG-RAN WG4 Meeting # 104b-e	R4-2215353

# Electronic meeting, 10th-19th, Oct., 2022

Agenda Item:	4.5.1.1

Source:	Intel Corporation

Title:	Discussion about remaining issues about Unified TCI state in FeMIMO

Document for:	Discussion

# Introduction

In this meeting, we will provide our view regarding to the unified TCI state for DL and UL, there are still some open issues in WF[1] in last meeting are as follows:

- Issue1-1-1a Whether UE need to track UL time/frequency for UL TCI state activation when DL-RS is associated with serving cell/non-serving cell - Issue 1-2-1 Joint TCI switching delay requirement for DL TCI state switching - Issue 1-2-2 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2 - Issue 1-4-1 Whether to consider unknown TCI state in the TCI state list

# Discussion

## 2.1 UL time/frequency for UL TCI state activation

Issue1-1-1a Whether UE need to track UL time/frequency for UL TCI state activation when DL-RS is associated with serving cell - Proposal: - Option 1: - No, UL timing for cell with different PCI if derived from DL timing of serving cell - Option 2: - Depends on whether source RS in active UL TCI state is a subset of source RS in DL active TCI list Issue1-1-1b Whether UE need to track UL time/frequency for UL TCI state activation when DL-RS is associated with cell with different PCI - Proposal: - Option 1: - No, UL timing for cell with different PCI if derived from DL timing of serving cell in Rel-17 - Option 2: - Depends on whether source RS in active UL TCI state is a subset of source RS in DL active TCI list

For UL TCI state activation of serving cell, no timing/frequency tracking is needed since the timing will be dependent on the serving cell DL timing.

For UL TCI state activation for non-serving cell, the UL timing will be dependent on the DL timing of cell with additional PCI. In general case, if the DL RS in the activated UL TCI state belong to the active DL TCI list, then no further timing tracking is needed since UE will maintain the timing for the active DL TCI state. if DL RS is not in  the active DL TCI list, UE may not maintain the timing. However, for unified TCI state switch, the requirement is defined that timing offset between serving cell and non-serving cell is smaller than CP. It seems that serving cell DL timing may still be applied for UL timing of non-serving cell. therefore, we prefer that UE don’t need to perform timing/frequency tracking for UL TCI state activation for both serving cell and cell with additional PCI.

Proposal 1: UE don’t need to perform timing/frequency tracking for UL TCI state activation for both serving cell and cell with additional PCI.

## 2.2 Joint TCI state switching requirement

Issue 1-2-1 Joint TCI switching delay requirement - Proposal: - For DL TCI state switching, - [In case of joint TCI state switch, UE is not expected to receive on DL before UE completes the DL and UL TCI state switch.]

For the discussion for Joint TCI state switch delay, the DL/UL TCI state switch can be performed independently. However, for the test, UL transmission and DL transmission will rely on each other. It’s hard to verify the DL reception with new TCI state without uplink feedback. Therefore, we prefer to keep the current statement in the specification.

Proposal 2: Keep the current clarification for DL TCI state switching in Joint TCI state switch in the specification.

2.3 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2

Issue 1-2-2 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2 - Proposals - Option 1: - Longer delay is expected. - Option 2: - Reuse the existing delay requirement of MAC CE based UL TCI state switch.

The issue has been discussed for several meetings. If sweeping factor N=8 is used, the total delay will be 5*8=40 samples. If SSB periodicity is 40ms, the total delay will exceed 1s which is quite long. Some company think that SSB is used as source RS and  will continue to perform RX beam sweeping for continuously update the pathloss. From our understanding, for TCI activation, the main purpose is to switch the beam fast. However, pathloss calculation will not only be used for TCI activation but also for some normal TX power calculation. Here, we think that maybe it’s not proper to consider the total length of pathloss calculation for UL TCI activation delay.

Therefore, we prefer to develop compromise solution for the purpose of fast TCI activation. There are two possible options. For option 1, since the SNR condition is above -2dB for TCI activation, less samples can be used for pathloss calculation compared with L3 measurement where SNR is -6dB. For Option 2, less sweeping factor can be used. Both option 1 and option 2 are candidate implementation solution and we don’t need to differentiate them. We can define the final extended SSB resource number and it’s up to UE implementation how to perform pathloss calculation with these samples.

- The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + NM* (Tfirst_target-PL-RS + Q*Ttarget_PL-RS + 2ms).

Where Q is the extended number of SSB resource number (Q>4), Q can be further discussed, e.g. Q=7.

Proposal 3: When SSB is indicated as PL-RS in UL TCI state for FR2, the total delay is:

- n+THARQ + 3ms + NM* (Tfirst_target-PL-RS + Q*Ttarget_PL-RS + 2ms)

Where Q is the extended number of SSB resource number(Q>5), Q is FFS.

## 2.4 TCI state list update delay

Issue 1-4-1 Whether to consider unknown TCI state in the TCI state list - Proposals - If one or more TCI states in the active TCI state list is unknown, active DL TCI state list update delay is FFS.

In GTW session, the following agreement is achieved:

- [Longer delay applies if any TCI state is unknown in TCI state list update]. Active TCI state list can contains known and unknown TCI states.

It’s agreed that both known and unknown TCI states can be included in the TCI state list. We prefer to remove the bracket directly. Since we already have unknown TCI state list update delay. we can so compromise to the delay for unknown TCI state update.

# Conclusion

In this contribution, we provide our views regarding Unified TCI state in FeMIMO:

Proposal 1: UE don’t need to perform timing/frequency tracking for UL TCI state activation for both serving cell and cell with additional PCI.

Proposal 2: Keep the current clarification for DL TCI state switching in Joint TCI state switch in the specification.

Proposal 3: When SSB is indicated as PL-RS in UL TCI state for FR2, the total delay is:

- n+THARQ + 3ms + NM* (Tfirst_target-PL-RS + Q*Ttarget_PL-RS + 2ms)

Where Q is the extended number of SSB resource number, Q is FFS.

# Reference

[1] R4-2214481, WF on FeMIMO RRM impact for unified TCI, Intel