3GPP TSG-RAN WG4 Meeting # 104b-e	R4-2215356

# Electronic meeting, 10th-19th, Aug., 2022

Agenda Item:	6.9.2.1

Source:	Intel Corporation

Title:	Discussion on L3 part enhancement for FR2 SCell activation

Document for:	Discussion

# Introduction

In last meeting , the WF is agreed[1] and we will discuss the following aspects related to L3 measurement:

- Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation - Beam sweeping factor enhancement related with WI of FR2 multi-Rx chain DL reception - Beam reporting

# Discussion

In unknown SCell activation case, UE needs to perform the following steps:

- Application of SCell activation MAC CE

- Cell detection, in order to find coarse timing of the SCell, and (L3 part)

- AGC, in order to settle the gain setting for the SCell, and  (L3 part)

- L1-RSRP measurement and reporting, in order to find Rx beam for receiving and help the network to select Tx beam (L1 part)

- Application of the PDCCH TCI activation MAC CE (L1 part)

- Fine time tracking (L1 part)

- CQI measurement and reporting (L1 part)

In this contribution, we will analyse the possible enhancement for L3 part.

Issue 2-2-1: Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception) - FFS on beam sweeping factor reduction during unknown FR2 SCell activation. - Option 1a (CMCC): The agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or positioning enhancement can be considered as baseline. - Option 1b (vivo): Introduce the UE capability to support Rx beam sweeping factor can be less than 8 for FR2 - Option 1c (Ericsson): - RAN4 to define RX beam constant time. Where, RX beam constant time is a time duration or window within which the RX beams are assumed to be constant or non changing. Rx beam constant time to be agreed as [X=1280ms]. - If UE performed a full RX beam sweeping for a procedure, next procedures or steps fall within RX beam constant time do not need to perform RX beam sweeping. - UE can speed up the remaining steps in SCell activation with a shorter beam scaling factor based on prior Rx beam information from the step before - Other option is not precluded

Two steps are involved in L3 related procedure.

- Cell detection

- AGC

1 SSB for cell search and 2 SSB for AGC are assumed. During L3 procedure, Rx beam sweeping will be performed. Since the RX beam sweeping factor is 8, therefore total 3*8=24 SMTC is assumed, where reduced Rx beam sweeping factor can be investigated. Similar with positioning WI, a new UE capability can be introduced to further reduce the FR2 RX beam sweeping factor to be less than 8, e.g. {1,2,4,6}.

Proposal 1: A new UE capability can be introduced to further reduce the FR2 RX beam sweeping factor to be less than 8.

Issue 2-2-2: Beam sweeping factor enhancement related with WI of FR2 multi-Rx chain DL reception - FFS whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction.

From our understanding,  beam sweeping factor may also be discussed in FR2 multi-RX chain DL reception WI. However, the beam sweeping factor is designed based on multi-panel, i.e. simultaneous measurement for multiple TRPs. It has no impact to single SCell activation. For single SCell activation delay enhancement, L1-RSRP measurement is performed on one SCell where there seems to be no multi-panel issue. Besides, whether L1-RSRP measurement can be skipped is FFS. The impact to multiple SCell activation may be discussed later.

As agreed in last meeting, single SCell activation will be considered in 1st phase. Multiple SCell activation will be considered in 2nd phase.

- 1st phase: RAN4 starts the discussion of the R18 unknown FR2 SCell activation enhancement based on baseline FR2 SCell activation requirement in TS38.133 section 8.3.2 - Discuss the feasibility and method of SCell activation enhancement based on AP-CSI-RS and/or A-TRS - 2nd phase: the other SCell activation cases, e.g., multiple SCell activation, direct SCell activation, and PUCCH SCell activation, to be discussed after baseline case (i.e. single FR2 SCell activation) is completed. - 2nd phase will start from RAN4 #107 meeting.

Therefore, we prefer that don’t need to consider multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI at least in 1st phase.

Proposal 2: Don’t need to consider to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI at least in 1st phase.

Issue 2-2-3: Cell measurement/synchronization sample number related enhancement of FR2 unknown SCell activation - FFS to align the understanding of cell measurement/synchronization/AGC in the existing FR2 unknown SCell activation delay requirement. - FFS on all options for issue 2-2-3 - Option 1 (Apple, OPPO, CMCC, LGE, MTK, NTT DCM): RAN4 to study sample number reduction for L3 measurement/synchronization during unknown FR2 SCell activation (with -2dB SINR side condition) - Option 1a(Apple, CMCC): RAN4 to study if we can remove or reduce L3 T/F tracking time (8 Trs) - Option 2 (ZTE): Based on the assumption of scaling factor 8, whether the processing latency of AGC can be reduced, should be further discussed.

In legacy unknown SCell activation, 1 SSB for cell search and 2 SSB for AGC are assumed. For cell search part, 1*8 samples will be used for FR2. However, rough timing corresponding to different RX beam may not differ quite a lot. Therefore, maybe total M samples is enough(M<8) and M is independent of Rx beam sweeping factor.

For AGC part, the total delay will still be scaled by Rx beam sweeping factor since the beam power may be much different. Besides, we are open to further discussed whether 2 samples can be reduced to 1 sample.

Proposal 3: For cell search, 1*8 samples will be reduced to M samples(M<Rx beam sweeping factor) where M is fixed number and M is independent of Rx beam sweeping factor.

# Conclusion

In this contribution, we provide our views regarding SCell activation delay reduction:

Proposal 1: A new UE capability can be introduced to further reduce the FR2 RX beam sweeping factor to be less than 8.

Proposal 2: Don’t need to consider to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI at least in 1st phase.

Proposal 3: For cell search, 1*8 samples will be reduced to M samples(M<Rx beam sweeping factor) where M is fixed number and M is independent of Rx beam sweeping factor.

# Reference

[1] R4-2214345, WF on R18 eFeRRM, Apple