3GPP TSG-RAN WG4 Meeting # 104b-e	R4-2215359

# Electronic meeting, 10th-19th, Oct., 2022

Agenda Item:	6.20.3

Source:	Intel Corporation

Title:	Discussion on RRM impacts from R18 L1/L2 mobility

Document for:	Discussion

# Introduction

In last meeting , the WF is agreed[1] and we share our view regarding the following topics:

- Whether to consider simultaneous multi-panel in FR2 - Definition of intra-frequency/inter-frequency in inter-cell operation - Whether to cover inter-frequency - synchronous scenarios definition - Known condition for L1/L2 inter-cell mobility - TCI activation delay

# Discussion

Issue 3-2-2: Whether to consider simultaneous multi-panel in FR2 - Option 1 (MTK, Apple, vivo, QC, HW, LGE): Not consider simultaneous multi-panel in FR2 - Option 2 (Ericsson, Nokia): Consider simultaneous multi-panel in FR2

Simultaneous multi-panel reception mainly focus about increasing DL MIMO layers and related beam management. This WID focus about L1/L2 mobility, even through it may involve L1-RSRP measurement. We prefer not to discuss parallel in two WIs.

Proposal 1: Not consider simultaneous multi-panel in FR2 for L1/L2 inter-cell mobility.

Issue 3-2-3: Definition of intra-frequency/inter-frequency in inter-cell operation - Option 1 (Apple, Ericsson, vivo, HW, Nokia): RAN4 to discuss the definition of intra-frequency/inter-frequency in inter-cell operation - Option 1a (MTK): For SSB L1-RSRP measurement, follow the definition of L3 measurement. - Option 1b: For SSB L1-RSRP measurement, intra-frequency is defined as long as the SSB-based L1 measurement is performed within active BWP(s) of the UE. - Option 2 (QC): discuss later Issue 3-2-4: Whether to cover inter-frequency - Option 1 (MTK, vivo, QC, apple): Further discuss the necessity, feasibility, and pros/cons of specifying inter-frequency L1-RSRP measurement. - Option 2 (CMCC, HW, Ericsson, Nokia): Yes

For legacy L3 measurement, the intra-frequency and inter-frequency definition is clear. For Rel-17 L1-RSRP measurement for cell with different PCI, in the spec, the requirement is defined when SSB resources of the cell with different PCI in csi-SSB-ResourceSet within the CSI-ResourceConfig settings for the active BWP. There is no clear definition of intra-frequency L1-RSRP measurement yet. For L1/L2 inter-cell mobility, we prefer to keep  the intra/inter frequency definition as legacy L3 measurement to avoid confusion.

For inter-frequency L1-RSRP measurement, there are two scenarios. Similar as L3 measurement, there are two scenarios:

- Inter-frequency L1-RSRP measurement with gap

- Inter-frequency L1-RSRP measurement without gap

When gap is needed, the L1-RSRP measurement will be shared with L3 measurement, then the delay will be extended for both of them. In Rel-17 inter-cell L1-RSRP measurement, the requirement is defined that no L3 measurement impact is expected. Similarly, here, we should focus about inter-frequency L1-RSRP measurement without gap first. Then Rel-17 inter-cell L1-RSRP measurement requirement can be re-used as baseline.

Proposal 2: For L1/L2 inter-cell mobility, we prefer to keep the intra/inter frequency definition as legacy L3 measurement.

Proposal 3: For inter-frequency L1/L2 mobility, focus about inter-frequency L1-RSRP measurement without gap first.

Whether to cover non-synchronous scenarios - Option 1: Not consider non-synchronous scenarios. - Option 2 (Ericsson, Nokia): Consider non-synchronous scenarios. - Option 3 (Ericsson, HW): FFS - Option 3a (Apple, MTK, vivo): discuss the definition of synchronous and non-synchronous - Option 3b (Apple, QC, vivo, Nokia): wait for RAN2’s progress

In Rel-17 inter-cell L1-RSRP measurement requirement, the timing offset is defined within CP. Therefore, for L1/L2 inter-cell mobility, the synchronous definition will also refer to timing offset smaller than CP, which is similar with synchronous definition in DAPS.

Proposal 4: Synchronous scenario will refer to timing offset smaller than CP between source cell and target cell.

From the LS from RAN2, For L1/L2 inter-cell mobility,

Assume that we rely on L1 measurements to trigger L1L2 mobility (still measurement for preparation could be L3, FFS)

It seems that both L3 and L1 measurement will be candidate options before HO command is triggered. If SSB based L3 measurement report is still used, there is no timing offset limitation. However, if L1 measurement is configured for mobility preparation. It only supports that timing offset is smaller than CP. We are open to further discuss whether non-synchronous scenarios need to be considered according to the RAN2 progress.

Proposal 5: Further discuss whether non-synchronous scenarios need to be considered according to the RAN2 progress.

Similar with legacy L3 HO, known and unknown cell condition needs to be defined as well. If L3 measurement report is still used, the known and unknown cell condition of legacy can be re-used.

When L1 measurement is configured to be performed before HO command, the known and unknown cell condition will be based on whether L1-RSRP result is reported recently. However, Rel-17 inter-cell L1-RSRP is only defined under known cell condition where the UE has sent a valid L3 measurement report during the last 5 seconds. It seems that L3 measurement is still required before L1 measurement. Therefore, L1 measurement will depend on L3 measurement as well. It not, extra cell search and time tracking for L1 measurement are needed.

Observation 1: Rel-17 inter-cell L1-RSRP is only defined under known cell condition where the UE has sent a valid L3 measurement report during the last 5 seconds.

RAN4 may need to further discuss whether to support inter-cell L1-RSRP measurement when L3 measurement is not available recently.

Proposal 6: Further discuss whether to support inter-cell L1-RSRP measurement when L3 measurement is not available recently.

L1/L2 inter-cell mobility delay requirements

The component of L1/L2 inter-cell mobility delay will depend on the design of RAN2, e.g. whether L1-RSRP measurement is included in the delay, whether MAC CE is used to trigger the HO command. Here, we want to raise another issue that whether TCI activation is considered in L1/L2 mobility delay. It’s possible that TCI configuration will be included in RRC or MAC CE HO command. In Rel-17 TCI activation delay for cell with different PCI, TCI state may be known or unknown. If TCI state is unknown, UE needs to perform extra Rx beam sweeping. Besides, the Rel-17 TCI activation requirement is defined with the assumption that timing offset is smaller than CP and cell is known. In Rel-18 L1/L2 mobility, it’s FFS whether such condition is satisfied or not. We can further discuss TCI activation delay related issue based on RAN2 design.

Proposal 7: Further discuss whether TCI activation is considered in L1/L2 mobility delay requirement.

# Conclusion

In this contribution, we provide our views regarding SCell activation delay reduction:

Proposal 1: Not consider simultaneous multi-panel in FR2 for L1/L2 inter-cell mobility.

Proposal 2: For L1/L2 inter-cell mobility, we prefer to keep the intra/inter frequency definition as legacy L3 measurement.

Proposal 3: For inter-frequency L1/L2 mobility, focus about inter-frequency L1-RSRP measurement without gap first.

Proposal 4: Synchronous scenario will refer to timing offset smaller than CP between source cell and target cell.

Proposal 5: Further discuss whether non-synchronous scenarios need to be considered according to the RAN2 progress.

Observation 1: Rel-17 inter-cell L1-RSRP is only defined under known cell condition where the UE has sent a valid L3 measurement report during the last 5 seconds.

Proposal 6: Further discuss whether to support inter-cell L1-RSRP measurement when L3 measurement is not available recently.

Proposal 7: Further discuss whether TCI activation is considered in L1/L2 mobility delay requirement.

# Reference

[1] R4-2214348, WF on Further NR Mobility Enhancements, MTK

[2] R2-2209257, LS on L1/L2-based inter-cell mobility, MTK