3GPP TSG-RAN WG4 Meeting #104-bis-e		R4-2215366

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.1.2

Source:	Intel Corporation

Title:	Views on 100kHz channel raster and the carrier resource grid

Document for:	Discussion

# 1	Introduction

FFS:

- RAN4 common understanding for channel raster for Rel-15/16/17 on the band with 100KHz channel raster specified

  - UE channel bandwidth is aligned with 100KHz channel raster

    - Negative: Intel, Ericsson, China Telecom

  - UE BWP is allowed not to be aligned with 100KHz channel raster

    - BWP is subset of channel bandwidth.

  - SIB 1 channelBandwidth is aligned with 100KHz channel raster for RAN4 requirements for both BS and UE

    - Negative: ZTE, Huawei

In the previous RAN4 #104 meeting, the topic of SIB1 carrierBandwidth, BWP and 100kHz channel raster were discussed.  A RAN4 common understanding was that SIB1 carrierBandwidth corresponds to BS transmit bandwidth configurations, which is not mandated to be the maximum BS transmission bandwidth configuration specified in TS38.104 and can be any values in number of PRBs.  Also, that the dedicated channel (UE) BWP has to be configured within resource grid (refer to Clause 4.4 and Clause 4 in 38.211).   The following open items were left for future study for the RAN #104-bis-e meeting.

# Discussion

The three bullets for discussion all deal with clarifying which aspects of channel bandwidth must be aligned with 100kHz channel raster.  There are three components involved, the SIB1 carrier bandwidth (also called resource grid) which is signalled as common to the cell, the UE channel bandwidth (also called the dedicated carrier bandwidth), and the bandwidth part (BWP).  The starting location of the SIB1 carrier bandwidth is signalled as a frequency (ARFCN), all other signalling for the location of these entities are counted as an integer number of RBs, none are signalled in MHz or kHz.  The RAN4 specs describe how to translate the number of RBs to frequencies.

### 2.1 SIB1 Carrier Signaling and UE Channel Signaling

To better understand the differences between the SIB1 carrier bandwidth and the UE channel bandwidth, it is worth looking at what is currently implemented in the RAN2 signaling spec, to see the limitations of the range and type of values that can be conveyed about SIB1 carrier bandwidth and UE channel bandwidth. In TS 38.331, the beginning of the carrier resource grid is given by:

SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> frequencyInfoDL-> absoluteFrequencyPointA = ARFCN-ValueNR

which gives an ARFCN value for the location of the zeroth RB.  Point-A can also be similarly defined with offsetToPointA in relation to the CD-SSB.  In both cases, Point-A defines the start of the carrier resource grid and is on channel raster.  The width is carrier resource grid is given by:

SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> frequencyInfoDL-> scs-SpecificCarrierList-> carrierBandwidth = X PRBs

In RRC connected, a dedicated UE channel bandwidth can be set as:

ServingCellConfig-> downlinkChannelBW-PerSCS-List-> offsetToCarrier = X PRBs

and

ServingCellConfig-> downlinkChannelBW-PerSCS-List-> carrierBandwidth = X PRBs

in TS 38.331 this is described as:

downlinkChannelBW-PerSCS-List
A set of UE specific channel bandwidth and location configurations for different subcarrier spacings (numerologies). Defined in relation to Point A. The UE uses the configuration provided in this field only for the purpose of channel bandwidth and location determination. If absent, UE uses the configuration indicated in scs-SpecificCarrierList in DownlinkConfigCommon / DownlinkConfigCommonSIB. Network only configures channel bandwidth that corresponds to the channel bandwidth values defined in TS 38.101-1 [15] and TS 38.101-2 [39].

In the case of UE channel bandwidth, the location is defined again in reference to Point A.  The UE dedicated channel bandwidth is not directly defined with its own ARFCN value.  The only time the SIB1 carrier bandwidth and the UE channel bandwidth, directly reference a frequency is through Point A, all other references are offset by a number of RB.

Observation 1: The carrier bandwidth and the UE channel bandwidth, directly reference the ARFCN frequency through Point A.

Observation 2: There is no RAN2 signaling that directly defines a dedicated UE channel with an ARFCN value or other frequency value.  The RAN2 signaling defines dedicated UE channel as a number of RB offset in relation to Point A.

Proposal 1: It is proposed to clarify the RAN4 spec, that the ARFCN value of Point A is used in signaling, to describe the location of the carrier resource grid, which is more accurate than saying that there is signaling for the actual RF channels.  This text should be added to clarify TS 38.101-1,2 and TS 38.104

If RAN4 chooses to limit UE channels to 100kHz raster in addition to the RF reference frequency which is already on a 100kHz raster, then the specification will be doubly constrained, and only small set of UE channel locations can be used.  This would mean that the carrier resource grid would be 100kHz raster, but then the offset to carrier or the offset to RF reference frequency would also be on a 100kHz raster.  This would result in the least common multiple of the two constraints becoming the new effective raster, e.g., for SCS 15kHz with 180kHz RBs, lcm(100,180) = 900kHz.

Observation 3: If RAN4 constrains the dedicated UE channels to 100kHz raster, and since signaling limits the location of UE channels to an integer number of PRBs in relation to the RF reference frequency, then the least common multiple of the two constraints becomes the effective raster, e.g., 900kHz raster for SCS 15kHz which limits potential UE channel locations and will result in inefficient spectrum usage.

### 2.2 Clarification of Channel Raster in the RAN4 Spec

Channel raster is described in TS 38.101-1,2 and TS 38.104 in section 5.4.2 of both specs.  The RF reference frequency or Fref, is used to identify the positions of RF channels and is given by NR Absolute Radio Frequency Channel Number (ARFCN).  RAN4 chooses Fref, to describe channels through their center frequency, which is typical in RF engineering.  RAN1,2 choose to describe channels through the Point A location which is clearer for integer math based on the number of RBs.  For clarity, the RAN4 spec should provide translation between these two methods of channel description.

### 5.4.2	Channel raster

#### 5.4.2.1	NR-ARFCN and channel raster

The global frequency raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of the carrier resource grid of the RF channels, SS blocks and other elements. The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

In section 5.4.2.1, the spec states that “the RF reference frequency is used in signaling to identify the position of the RF channels, SS blocks and other elements”.  As we discussed in our previous section, this is not entirely accurate.  There is no direct ARFCN signaling for RF channels, what actually occurs is that the ARFCN is used to describe Point A of the carrier resource grid, and then RB offsets from Point A are used to describe the location of the RF channels.

Proposal 2: It is proposed to clarify the RAN4 spec, section 5.4.2.1 that the RF reference frequency is used in signaling, to describe the location of the carrier resource grid, whereas UE channel bandwidths are not signaled with an ARFCN number but rather an offset location in the carrier resource grid.  This text should be added to clarify TS 38.101-1,2 and TS 38.104

5.4.2.2 Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the corresponding resource element of the carrier resource grid of the RF channel is given in Table 5.4.2.2-1 and can be used to identify the RF channel position, signalled by Point A, the beginning of the carrier resource grid [9]. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the UE.

Table 5.4.2.2-1: Channel Raster to Resource Element Mapping

|  | NRBmod2=0 | NRBmod2=1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

k, nPRB, NRB are as defined in TS 38.211[9], nPRB is the PRB index within the carrier resource grid, k is the resource element index within the PRB, aligned with FREF. nPRB and k are not signalled.  NRB is signalled by carrierBandwidth [11].

In section 5.4.2.2, the RAN4 specs state how the RF reference frequency Fref, corresponds to the center resource element (sub-carrier).  However, again in this case, the RAN4 spec is unclear and needs additional text to help readers make the link between the RF frequency and what is actually being signaled, the Point A ARFCN, and NRB.  It is the resource element of the carrier resource grid that is being calculated in this table.  Furthermore, the values of k and are not signalled values but are only used to calculate the position of for calculation of the relationship between the RF reference frequency and the center resource element.  The reference to TS 38.211 should be removed, since in that document k has a different meaning and PRBs mean RBs that exist within an BWP while CRBs are the common resource blocks of the resource grid. Within the RAN4 spec, PRB has a more general meaning and should not be changed to align with the RAN1 spec.

Proposal 2: It is proposed to clarify the RAN4 spec section 5.4.2.2 that k and nPRB are used to identify the RF channel position but not signaled and are only used for calculation the RF reference frequency.  Point A and carrierBandwidth and offset are used to signal channel position. This text should be added to clarify TS 38.101-1,2 and TS 38.104

### 2.3 BWP does not need to be on raster

BWPs are clearly described in TS 38.211 section 4.4.5 to exist within the carrier resource grid.  BWP operate on RB have no precise frequency definition.  There is no need for BWP to be on raster.

Observation 4: Since BWP operate on RB have no precise frequency definition, there is no need for BWP to be on raster.  There is no need for any spec clarification on BWP.

# 3	Conclusions

In summary, we have discussed several aspects of SIB1 carrier bandwidth and the UE channel bandwidth and proposed changes to the RAN4 spec to clarify: that ARFCN, defining Point A, is the only way frequency is directly signaled, whereas SIB1 carrier bandwidth and location and UE channel bandwidth and location are signaled as an integer number of RBs.

Observation 1: The carrier bandwidth and the UE channel bandwidth, directly reference the ARFCN frequency through Point A.

Observation 2: There is no RAN2 signaling that directly defines a dedicated UE channel with an ARFCN value or other frequency value.  The RAN2 signaling defines dedicated UE channel as a number of RB offset in relation to Point A.

Proposal 1: It is proposed to clarify the RAN4 spec section 5.4.2.1 that the ARFCN value of Point A is used in signaling, to describe the location of the carrier resource grid, which is more accurate than stating that there is signaling for the actual RF channels.  This text should be added to clarify TS 38.101-1,2 and TS 38.104

Observation 3: If RAN4 constrains the dedicated UE channels to 100kHz raster, and since signaling limits the location of UE channels to an integer number of PRBs in relation to the RF reference frequency, then the least common multiple of the two constraints becomes the effective raster, e.g., 900kHz raster for SCS 15kHz which limits potential UE channel locations and will result in inefficient spectrum usage.

Proposal 2: It is proposed to clarify the RAN4 spec, section 5.4.2.1 that the RF reference frequency is used in signaling, to describe the location of the carrier resource grid, whereas UE channel bandwidths are not signaled with an ARFCN number but rather an offset location in the carrier resource grid.  This text should be added to clarify TS 38.101-1,2 and TS 38.104

Proposal 3: It is proposed to clarify the RAN4 spec section 5.4.2.2 that k and nPRB are used to identify the RF channel position but not signaled and are only used for calculation the RF reference frequency.  Point A and carrierBandwidth and offset are used to signal channel position. This text should be added to clarify TS 38.101-1,2 and TS 38.104

Observation 4: Since BWP operate on RB have no precise frequency definition, there is no need for BWP to be on raster.  There is no need for any spec clarification on BWP.

# 4	References

Note: the reference indexes are staggered to match those in TS 38.104

[9] 3GPP TS 38.211: "NR; Physical channels and modulation"

[11] 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".