3GPP TSG-RAN WG4 Meeting # 104bis-e	                                  R4-2215367

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	6.10.2

Source:	Intel Corporation

Title:	Discussion on RRM requirements for pre-configured MGs, multiple concurrent MGs and NCSG

Document for:		Discussion

# Introduction

In the last RAN4 meeting, Rel18 NR measurement gap enhance WI was initially discussed. In this paper, the objective 1 in [1][3] was discussed.

The following objectives are considered in this WI: - Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or concurrent MGs and/or NCSG [RAN4] - Prioritize at least joint requirements for UE configured with - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - Note 1: Gaps that are configured for NTN are precluded in Case 1 and Case 2 - Note 2: The requirement discussions on the scenarios that NCSG is considered in Case 1 and that Pre-configured MG is considered in Case 2 will be started after RAN#99. - Note 3: Prioritization among other possible combinations of pre-configured MG, concurrent MG, NTN gaps and NCSG can be discussed after RAN#99 - Note 4: This WID does not include any inter-working with MUSIM gaps - Define RRM requirements for measurement without gaps for the following cases - NR SSB-based inter-frequency and intra-frequency measurements without gaps for UEs reporting NeedForGapsInfoNR IE [RAN4] - Study whether the additional interruption is allowed when UE reporting ‘NeedForGapsInfoNR'. Further define the interruption length, occasion and ratio, if the interruption is allowed - Define related requirements, such as CSSF, measurement period, scheduling restriction etc. - Inter-RAT measurements without gaps [RAN4] - Inter-RAT NR measurements - Inter-RAT LTE measurement

# Discussion

# Clarifications on using scenario

In the last RAN4 meeting, companies seem have quite diverse understandings on the case 1 and case 2 WID. The open issues on these using scenarios in [2] was also self-contained below.

### Issue 2-1: Whether to define requirements for one Pre-configured MG and one NCSG < Way forward >: - RAN4 to clarify whether one Pre-configured MG and one NCSG is included in Case 1 and/or Case 2 - FFS to explicitly set the priority of different combinations

From the technical perspective, the mixed configuration among the pre-configured MG , NCSG and other legacy MGs will potentially increase the max number of supported concurrent gaps in Rel17. It will impact the UE complexity significantly.

Additionally, with the clarifications in last RANP meeting [3], the combination of Pre-MG and NCSG will be not included in Case 1 or Case 2 in the current stage of this WI.

Observation 1: The cases when both Pre-configured MG and NCSG are configured together within the concurrent MGs were exclude in the current stage of this WI [3].

Proposal 1: RAN4 can confirm that the Case 1 and Case 2 below shall be prioritized before RANP#99.

- Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG)

- Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG)

Regarding to the more detailed combinations for Case1, the several scenarios (e.g. the combinations below) can be considered were proposed in the last RAN4 meeting.

- Legacy per-UE gap + Pre-configured per-UE gap

- Legacy per-UE gap + Pre-configured per-FR gap

- Legacy per-FR gap + Pre-configured per-UE gap

- Legacy per-FR gap + Pre-configured per-FR gap

Given the current requirements on the maximum number of UE supported concurrent gaps is 3 as the more than one per-FR measurement gaps is not supported, we can propose that:

Proposal 2: The combinations in Case 1 can be:

- Legacy per-UE gap + Pre-configured per-UE gap

- Legacy per-UE gap + Pre-configured per-FR gap

- Legacy per-FR gap + Pre-configured per-UE gap

For the more detailed combinations in Case2, as illustrated in Figure 1 below, there are one NCSG and other legacy gap (Rel16) within concurrent multiple measurement gaps. However, if UE support one NCSG already, there is absolutely a vacant RF chain available for UE. That is when UE needs to measure the other carriers beside the one based on NCSG(e.g. f2 in Figure 1a.), it can be operated in both RF chains ( f0 and f1/2) simultaneously during the MG R16. As a result, the simultaneous serving data reception and measurements on the neighbour carriers within this legacy measurement gap can be allowed.

Observation 2: The vacant RF chain can be shared by the capable UE during the different measurement gaps (e.g. NCSG #1, NCSG #2 or legacy MG#1). Thus, the interruptions due to all of them can be same as that for NCSG.

(a)

(b)

Figure 1. Examples of NCSG within concurrent MGs

Therefore, from UE perspective, if UE can support at least one NCSG as one of induvial gap instance within the concurrent measurement gaps the interruption on the UE’s data reception can be vanished during the UE measuring the other carriers with the vacant RF chain. From the network perspective, it is also beneficial to configure more than one NCSGs to UE when it can support both NCSG and concurrent MG capabilities instead of one NCSG the other non-NCSG.

Proposal 3: Network shall configure all measurement gaps within the concurrent MGs as NCSG when UE can support NCSG capability.

Then, we can also propose the detailed combinations for Case 2 as:

Proposal 4: The combinations of NCSG, concurrent MGs in case 2 can be listed below.

- More than one NCSG per-UE gaps

- NCSG per-UE gaps + NCSG per FR gap

Nevertheless, in case of NW configured both NCSG and non-NCSG gaps within the concurrent MGs, from UE requirements perspective, we can suggest that:

Proposal 4a: If both NCSG and other non-NCSG measurement gaps are within the concurrent MGs, the interruption duration due to the non-NCSG measurement gaps can be same as that defined in 9.1.9.1 of TS38.133[3] for NCSG.

### Issue 2-2: Whether to define requirements for one Pre-configured NCSG < Agreement >: - RAN4 focuses on Case 1 and Case 2 first. RAN4 can continue discuss whether and when to discuss requirements for Pre-configured NCSG

On the other hand, whether the NCSG can be pre-configured is also under debating. With the clarifications in RANP above[3],we can conclude that

Observation 3: The NCSG which can be pre-configured is out of the current stage scope of WID.

However, in comparison with the legacy gaps which can be pre-configured, the similar benefits can be achieved if the gap is NCSG. For an example, when UE needs the activated NCSG when it needs measurement other objects, it can shorten the configuration time for NSCG(e.g. RRCReconfiguration). At same time, such aspect may not increase any UE capability in terms of maximum gap number supported.

Observation 4: There is obvious benefits when NCSG is pre-configured.

It is also feasible to pre-configure NCSG and activate them when needed. That is one of gap instance of concurrent gaps can be pre-configured and activated/deactivated when UE needs to perform the measurements within the gap.

Proposal 5: The pre-configured NCSG shall be also study in this WI after RANP#99.

### Issue 2-8: Positioning measurement gaps < Agreement >: - Rel-17 MGE Pre-MG for RRC-based POS is in the scope of this WI < Way forward >: - FFS Rel-17 MAC-CE based ePOS gap is in or out of scope of this WI

In Rel17, there are two types of pre-configured gap for positioning: one is the pre-configured MG which was configured by “GapConfig-r17” and can be activated/deactivated by DCI/timer-based/RRC; the other is pre-configured MG for ePos which can be configured by “PosGapConfig-r17” and activated by MAC message.

MeasGapConfig ::= SEQUENCE { gapFR2 SetupRelease { GapConfig } OPTIONAL, -- Need M ..., [[ gapFR1 SetupRelease { GapConfig } OPTIONAL, -- Need M gapUE SetupRelease { GapConfig } OPTIONAL -- Need M ]], [[ gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 OPTIONAL, -- Need N gapToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17 OPTIONAL, -- Need N posMeasGapPreConfigToAddModList-r17 PosMeasGapPreConfigToAddModList-r17 OPTIONAL, -- Need N posMeasGapPreConfigToReleaseList-r17 PosMeasGapPreConfigToReleaseList-r17 OPTIONAL -- Need N ]] } … GapConfig-r17 ::= SEQUENCE { measGapId-r17 MeasGapId-r17, gapType-r17 ENUMERATED {perUE, perFR1, perFR2}, gapOffset-r17 INTEGER (0..159), mgl-r17 ENUMERATED {ms1, ms1dot5, ms2, ms3, ms3dot5, ms4, ms5, ms5dot5, ms6, ms10, ms20}, mgrp-r17 ENUMERATED {ms20, ms40, ms80, ms160}, mgta-r17 ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot75}, refServCellIndicator-r17 ENUMERATED {pCell, pSCell, mcg-FR2} OPTIONAL, -- Cond NEDCorNRDC refFR2-ServCellAsyncCA-r17 ServCellIndex OPTIONAL, -- Cond AsyncCA preConfigInd-r17 ENUMERATED {true} OPTIONAL, -- Need R ncsgInd-r17 ENUMERATED {true} OPTIONAL, -- Need R gapAssociationPRS-r17 ENUMERATED {true} OPTIONAL, -- Need R gapSharing-r17 MeasGapSharingScheme OPTIONAL, -- Need R gapPriority-r17 GapPriority-r17 OPTIONAL, -- Need R ... } PosMeasGapPreConfigToAddModList-r17 ::= SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF PosGapConfig-r17 PosMeasGapPreConfigToReleaseList-r17 ::= SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF MeasPosPreConfigGapId-r17 PosGapConfig-r17 ::= SEQUENCE { measPosPreConfigGapId-r17 MeasPosPreConfigGapId-r17, gapOffset-r17 INTEGER (0..159), mgl-r17 ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20}, mgrp-r17 ENUMERATED {ms20, ms40, ms80, ms160}, mgta-r17 ENUMERATED {ms0, ms0dot25, ms0dot5}, gapType-r17 ENUMERATED {perUE, perFR1, perFR2}, ... } MeasPosPreConfigGapId-r17 ::= INTEGER (1..maxNrofPreConfigPosGapId-r17)

Also in [3], the similar conclusion was agreed in RANP. Therefore, we can conclude that:

Proposal 6: In order to avoid any confusions, we prefer to discuss the pre-MG for Pos by the GapConfig-r17 only.

# Inherited principles to define RRM requirements in Case 1(Combination of Pre-configured MG and Concurrent MG)

In Rel17, these two aspects of MG enhancement were discussed orthogonally because of the time limited. However, it is feasible to include the pre-configured gap as one of instance of multiple concurrent gap pattern if UE supported as illustrated in Figure 2 below. That is one of gap instance of concurrent gaps can be pre-configured and activated/deactivated when UE needs to perform the measurements within the gap.

Figure 2. Joint configurations of pre-MG and concurrent MGs

Observation 5: For the individual gap instance which is pre-configured, how to configure and (de)activated is same as the rules in Rel17 because the other instance are completely independent with the gap activation rules.

Proposal 7: The concurrent MGs can be pre-configured, but part of them can be activated depending on the rules defined in TS38.133[4] for the individual pre-configured MG.

On the other hand, only the activated pre-configured MG needs to be counted as an effective concurrent gap instance. Hence, there are following impacts on RRM requirements defined in Rel17 for concurrent gap[4,TS38.133].

Firstly, UE behavior when concurrent MGs were pre-configured need to be redefined.

Proposal 8: Only the activated pre-MGs will be counted into the instances which could be overlapped with others when UE supports concurrent measurement gaps.

Secondly, the other fundamental RRM impacts is the max number of concurrent MG to be supported can be same as these in Rel17 with some clarifications.

Proposal 9: RRM requirements on the maximum number of UE supported concurrent gaps below need to be defined as:

Table 9.1.8-1: The number of Gap Combination Configurations by UE supporting both concurrent measurement gap patterns and independent measurement gap patterns [3]

| Gap Combination Configuration Id | The number of simultaneous configured activated measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap | Per-FR2 measurement gap | Per-UE measurement gap |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. Note 2: For gap instances which are pre-configured they shall be activated. |  |  |  |

The additional RRM requirements impacted includes the Pre-MG activation delay in 8.19 of TS38.133[3]. In case of multiple activation procedures happened overlap, if the simultaneous activation is not allowed the extension on the delay requirements may be needed.

Figure 3. Concurrent gaps which can be simultaneously activated

Proposal 10: In case of the activation procedures of multiple pre-configured gaps being overlapped, the pre-configured gap activation delay requirements in [4] need to be extended.

The other RRM requirements defined in [3] for MG enhancement in Rel17 (e.g. trigger conditions, measurement reporting delay) can be independent.

In short summary, the potential RRM requirements impacted due to joint pre-MG and concurrent MGs in [3] can be listed in the table below.

| Clause in [3] | RRM requirements | Impacted because of pre-MG + concurrent MG |
| --- | --- | --- |
| 8.19 | Pre-configured measurement gap activation/deactivation delay | Yes |
| 9.1.8.2 | The number of Gap Combination Configurations by UE supporting both concurrent measurement gap patterns and independent measurement gap patterns | Yes |
| 9.2.6 | Intra-frequency measurements with measurement gaps | yes |
| 9.3.4 | Inter-frequency measurement with measurement gaps | Yes |

# Inherited principles to define RRM requirements in Case 2(Combination of NCSG and Concurrent MG)

Firstly, RRM requirements on the maximum number below need to be updated also.

Proposal 11: RRM requirements on the maximum number of UE supported concurrent gaps below when NCSG being configured as one of them can be defined as:

Table 9.1.8-1: The number of Gap Combination Configurations by UE supporting both concurrent measurement gap patterns and independent measurement gap patterns

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap Note x | Per-FR2 measurement gap Note x | Per-UE measurement gap Note x |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. Note x: these measurement gaps can be the legacy gap and/or NCSG. |  |  |  |

The other important issue related to RRM requirements for the concurrent MGs is the collision overlapping handling.

### Issue 2-15: Required changes for NCSG on collision < Agreement >: - When determining if a collision occurs between a NCSG instance and another gap instance, the baseline requirement considers the total NCSG duration, including both the VILs and the ML. < Way forward >: - FFS other potential enhancements regarding spare RF chains, interruption due to collision on RRT, revised proximity condition, … etc.

As illustrated in the figure below, there are two typical cases when we consider the interruption when both NCSG and other legacy measurement gaps are configured within the concurrent measurement gaps. One is there is no overlapping among NCSG and other legacy gaps used for the measurements on the different carriers as that of measurements by NCSG.

Figure 4. Two cases to define the interruption in Case 2

Observation 6-1: If UE support the NSCG within the concurrent measurement gaps, the interruption due to the individual gap instances which are not overlapped can be completely independent.

That is the interruptions due to individual legacy gap and NCSG which are not overlapped shall follow the requirements in clause 9.1.2 and 9.1.9.1 respectively.

On the other hand, for the cases when the multiple measurement gaps(NCSG or other measurement gaps) was overlapped, the interruption on UE due to both of the different measurement gaps can be overlapped also.

Observation 6-2: The total interruption to UE due to the multiple measurement gaps which are overlapped within the concurrent gaps can be coupled together.

Observation 6-3: For the window between NCSG VIL1 and VIL2(e.g. [t1,t2] in the figure above), during the interval in which there is no any overlapped interruptions UE can received the data as normal.

Therefore, the total interruption length due to both NCSG and other measurement gaps can be equal to

Tint_ncsg+Tint_other-Toverlap               (1)

Wherein, Tint_ncsg,Tint_other represented the allowed interruption due to NCSG and legacy measurements defined in clause 9.1.2 and 9.1.9.1 of TS38.133[4] respectively. And Tint_other is the overlapped time duration in slot among NCSG RTT time and legacy measurement gap length.

Proposal 12:  The interruption requirements for the multiple measurement gaps when NCSG being included in the concurrent measurement gaps can be defined as:

Tint_ncsg+Tint_other-Toverlap

Wherein, Tint_ncsg,Tint_other represented the allowed interruption due to NCSG and legacy measurements defined in clause 9.1.2 and 9.1.9.1 of TS38.133[4] respectively. And Toverlap is the overlapped time duration in slot among NCSG RTT time and legacy measurement gap length.

However, in order to simplify the requirements from RAN4 perspective, we prefer to define the interruption requirements for the individual measurement gaps independently as these in case of non-overlapping.

Proposal 12a:  The interruption requirements for the multiple measurement gaps when NCSG and other legacy measurement gaps can be defined as :

Tint_ncsg+Tint_other

Wherein, Tint_ncsg,Tint_other represented the allowed interruption due to NCSG and legacy measurements defined in clause 9.1.2 and 9.1.9.1 of TS38.133[4] respectively.

And for the collision proximity conditions, since RAN4 agreed to take both VILS and ML counted into as the gap occasion, in our views, the current requirements below can be reused.

When UE is configured with concurrent measurement gaps, two measurement gap occasions are considered colliding if at least one of the following conditions is met: - the two occasions are fully or partially overlapping in time domain, or - the distance between the two occasions is equal to or smaller than [4]ms.

Proposal 13: The proximity conditions in TS38.133[4] can be reused as the start point to define the collision between NCSG and other measurement gap instances within the concurrent measurement gaps.

# Conclusion

In this contribution, serval issues related to the measurement gap enhancement WI in Rel18 are discussed. The proposals can be summarized as:

Observation 1: The cases when both Pre-configured MG and NCSG are configured together within the concurrent MGs were exclude in the current stage of this WI [3].

Proposal 1: RAN4 can confirm that the Case 1 and Case 2 below shall be prioritized before RANP#99.

- Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG)

- Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG)

Proposal 2: The combinations in Case 1 can be:

- Legacy per-UE gap + Pre-configured per-UE gap

- Legacy per-UE gap + Pre-configured per-FR gap

- Legacy per-FR gap + Pre-configured per-UE gap

Observation 2: The vacant RF chain can be shared by the capable UE during the different measurement gaps (e.g. NCSG #1, NCSG #2 or legacy MG#1). Thus, the interruptions due to all of them can be same as that for NCSG.

Proposal 3: Network shall configure all measurement gaps within the concurrent MGs as NCSG when UE can support NCSG capability.

Proposal 4: The combinations of NCSG, concurrent MGs in Case 2 can be listed below.

- More than one NCSG per-UE gaps

- NCSG per-UE gaps + NCSG per FR gap

Proposal 4a: If both NCSG and other non-NCSG measurement gaps are within the concurrent MGs, the interruption duration due to the non-NCSG measurement gaps can be same as that defined in 9.1.9.1 of TS38.133[3] for NCSG.

Observation 3: The NCSG which can be pre-configured is out of the current stage scope of WID.

Observation 4: There is obvious benefits when NCSG is pre-configured.

Proposal 5: The pre-configured NCSG shall be also study in this WI after RANP#99

Proposal 6: In order to avoid any confusions, we prefer to discuss the pre-MG for Pos by the GapConfig-r17 only.

Observation 5: For the individual gap instance which is pre-configured, how to configure and (de)activated is same as the rules in Rel17 because the other instance are completely independent with the gap activation rules.

Proposal 7: The concurrent MGs can be pre-configured, but part of them can be activated depending on the rules defined in TS38.133[4] for the individual pre-configured MG.

Proposal 8: Only the activated pre-MGs will be counted into the instances which could be overlapped with others when UE supports concurrent measurement gaps.

Proposal 9: RRM requirements on the maximum number of UE supported concurrent gaps below need to be defined as:

Table 9.1.8-1: The number of Gap Combination Configurations by UE supporting both concurrent measurement gap patterns and independent measurement gap patterns [3]

| Gap Combination Configuration Id | The number of simultaneous configured activated measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap | Per-FR2 measurement gap | Per-UE measurement gap |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. Note 2: For gap instances which are pre-configured they shall be activated. |  |  |  |

Proposal 10: In case of the activation procedures of multiple pre-configured gaps being overlapped, the pre-configured gap activation delay requirements in [4] need to be extended.

Proposal 11: RRM requirements on the maximum number of UE supported concurrent gaps below when NCSG being configured as one of them can be defined as:

Table 9.1.8-1: The number of Gap Combination Configurations by UE supporting both concurrent measurement gap patterns and independent measurement gap patterns

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap Note x | Per-FR2 measurement gap Note x | Per-UE measurement gap Note x |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. Note x: these measurement gaps can be the legacy gap and/or NCSG. |  |  |  |

Observation 6-1: If UE support the NSCG within the concurrent measurement gaps, the interruption due to the individual gap instances which are not overlapped can be completely independent.

Observation 6-2: The total interruption to UE due to the multiple measurement gaps which are overlapped within the concurrent gaps can be coupled together.

Observation 6-3: For the window between NCSG VIL1 and VIL2(e.g. [t1,t2] in the figure above), during the interval in which there is no any overlapped interruptions UE can received the data as normal.

Proposal 12:  The interruption requirements for the multiple measurement gaps when NCSG being included in the concurrent measurement gaps can be defined as:

Tint_ncsg+Tint_other-Toverlap

Wherein, Tint_ncsg,Tint_other represented the allowed interruption due to NCSG and legacy measurements defined in clause 9.1.2 and 9.1.9.1 of TS38.133[4] respectively. And Toverlap is the overlapped time duration in slot among NCSG RTT time and legacy measurement gap length.

Proposal 12a:  The interruption requirements for the multiple measurement gaps when NCSG and other legacy measurement gaps can be defined as :

Tint_ncsg+Tint_other

Wherein, Tint_ncsg,Tint_other represented the allowed interruption due to NCSG and legacy measurements defined in clause 9.1.2 and 9.1.9.1 of TS38.133[4] respectively.

Proposal 13: The proximity conditions in TS38.133[4] can be reused as the start point to define the collision between NCSG and other measurement gap instances within the concurrent measurement gaps.

# References

- RP-221018 New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps

- R4-14346, WF on further enhancements on measurement gaps and measurements without gaps

- RP-222337

- TS38.133 v17.6.0

- TS38.331 v17.1.0