3GPP TSG-RAN WG4 Meeting # 104b-e	R4-2215374

# Electronic Meeting, October 10 - 19, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Enabling 2Rx option for n104

Agenda Item:	9.2 Study of 2Rx exception for U6GHz

Document for:	Approval

# Introduction

In RP#97e, the support for 4Rx in band n104 was discussed and the proposed way forward in RP-222684 was endorsed making 4RX the baseline, asking RAN4 to investigate enabling 2Rx option. In this contribution we discuss why and how 2Rx for n104 should also be enabled.

# Discussion

The following way forward in [1] was captured in RP#97e meeting minutes.

Moderator Way Forward:

- Define band n104 requirements with 4Rx as baseline. RAN4 to perform a study on any possible 2Rx exception (as done for 2Rx exception for automotive) and, if confirmed, identify the necessary specification impact (e.g. requirements, signalling) for the 2Rx exception to be used.

Observation: Since the 2RX and 4Rx features are already defines for n104 there is no impact on the 38.101-1 specification other than possibly a note describing the solution for 4Rx baseline with 2Rx exception.

## Why enable a 2Rx option for n104

Currently, there is only one regulation body (RCC) that has provided requirements for n104 and the related volumes and deployments are uncertain. Mandating 4Rx instead of 2Rx is a further deterrent to implement the band at all in a UE.

On the other hand, most of the high-end UEs support the WiFi6E (6GHz band) and/or n96 but only with two antennas in the majority of cases. As such, when n96 is supported, the two 6GHz receive paths are already available and, in order to support n104, only an additional PC3 PA is needed. This is far less effort/investment than adding two receive paths and two more antennas that may not be used by any other band or reusing other antennas that may have very poor performance above 6.4GHz.

It is clear that; until a sufficient market develops for n104, it is much better to enable UEs that already support n96 with 2Rx and to declare support for n104 with 2Rx rather than not support the band at all.

Observation:

- Until sufficient market is available for n104, UEs that support n96 with 2Rx should be allowed to declare support for n104 with 2Rx rather than n104 not supported

- Since n96 is not supported in the same region as n104 and support of n102 does not guarantee that the n104 frequency range is supported, some specific signalling might be required.

- Valid power classes for NRU are PC5 (default) and PC3 while it PC3 (default) and PC2 for NR

## How 2Rx option for n104 can be enabled

As discussed above, since n96 is not supported in the same region as n104, it is unclear if the EU is allowed to enumerate its support for n96 and whether it supports 2Rx or 4Rx in this band. One solution is to allow UEs to declared band n96 support in RCC countries with 2Rx or 4Rx capability. If the UE then declares support of n96 with 2Rx it can optionally declare 2Rx support for n104.

Proposal:

- If the UE declares 2Rx support for n96 it is also allowed to declare 2Rx support for n104 in RCC countries

- FFS if regions or countries beyond RCC should be added

- Details of the signalling can be left to RAN2.

# Conclusions

In this contribution, we discuss the reasons to allow 2Rx support for n104 and provide options on how this can be declared. The following proposals are made.

Proposal:

- If the UE declares 2Rx support for n96 it is also allowed to declare 2Rx support for n104 in RCC countries

- FFS if regions or countries beyond RCC should be added

- Details of the signalling can be left to RAN2.

# References

- RP-222684 Moderator’s summary for discussion [97e-29-4Rx-6GHz], RAN Vice-Chair (AT&T) , RP#97e