3GPP TSG RAN WG4 Meeting #104b-e	R4-2215381

# Electronic Meeting, 10 - 19 Oct., 2022

Agenda item:	6.6.1 (FR1_enh2)

Source:	SoftBank Corp.

Title:	On international roaming possibility of CPE/FWA/vehicle/industrial devices

Document for:	Approval

1.  Introduction

In the RAN#96 meeting, the revised WID on FR1-RF enhancement including the introduction of 29dBm CPE/FWA/vehicle/industrial type of UEs was approved [1] and a set of clarification questions was provided in [2] in RAN4#104-e.

This paper is to response questions raised during RAN4#104-e and explaining the background, on the relation between international roaming possibility and RF requirements.

2.  Relation to regulations

If there is no international roaming-in possibility for a certain type of UE, we can assume that all the UEs of the type in a country, if any, are subject to UE authentication/type approval of the country. Thus, before the type of UEs are used in a country, the national regulation or procedure is to be discussed/set and an operator simply follows the rules made.

But if international roaming is possible for the UE of this type and none of domestic operators/service providers shows an interest (= no motivation for domestic rule making), only a roaming-in UE could be operated without corresponding domestic rules specific to the UE.

[Observation-1] There is a case that roaming-in UEs could be operated in a country before a national regulation specific to the type of UEs is defined.

Even in this case, it is difficult to expect that the UE is free from any RF regulations: it is likely that the existing rules are to be applied to the roaming-in UEs regardless of its types/characteristics, i.e., HP-UE or specific purposes. In Japan, typically MOP of such UEs would be limited to the “default” value for the band and as a result, operators in Japan will be requested to limit the power for some bands.

This is the background of our asking clarifications on international roaming possibility.

[Observation-2] For the case mentioned in Observation-1, conventional requirements are likely to be applied to the type of UEs and to comply with this, operators might have to take an appropriate action such as MOP or unwanted emission limitation.

Currently we expect/hope that existing tools such as P-max/NS could be sufficient, but depending on the forthcoming discussion about RF assumptions, we might need to come back with a new proposal. Clarifying types of UEs unlike to roam could help mitigating a pain for the study.

Thus, we’d like to propose to make clear whether four types of devices under this WID are subject to international roaming, to take necessary actions against aspects in [Observation-2].

[Proposal-1] A clarification is requested whether four types of devices under this WID are subject to international roaming.

3.  Conclusion

This paper is intended for explaining the background to clarify whether a certain type of UE is subject to international roaming. The proposal is:

[Proposal-1] A clarification is requested whether four types of devices under this WID are subject to international roaming.

Reference

RP-221496		Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 
　　　　　　　(FR1), Huawei, HiSilicon

R4-2211749		Assumptions on CPE/FWA/vehicle/industrial devices, SoftBank corp.