3GPP TSG-RAN WG4 Meeting # 104bis-e                                                       R4-2215385

# Electronic Meeting, October 10 – October 19, 2022

Title: 	Further discussion on adjacent channel co-existence simulation assumption

Source: 	CATT

Agenda item:	6.17.2.1

Document for:	Discussion

- Background

Some preliminary co-existence simulation assumptions were agreed in last meeting [1]. This contribution provides our further consideration of remaining open items.

- Discussion

2.1 Scenarios

For the scenarios, the following scenarios are copied from [1].

Table 1: Scenarios for SBFD co-ex study (copied from [1])

| Victim | Aggressor | Aggressor baseline | Priority |
| --- | --- | --- | --- |
| NR TDD DL | SBFD | NR TDD DL | High priority |
| NR TDD UL | SBFD | NR TDD UL | Low priority |
| SBFD | NR TDD DL | FFS | High priority |
|  | NR TDD UL |  | Low priority |
|  | SBFD |  | Low priority |

There’re some details which need further discussion and decisions. First is the DU configuration for SBFD. According to the WF [2], DUD and DU can be the configuration. Cases for the simulation are summarized in Table 2. We also listed the priority we proposed. We proposed case 3 as low priority although it’s SBFD co-existence with TDD DL because the interference for the U SB adjacent with TDD DL should be severe, it should be avoided.

Table 2: Scenarios for NR TDD – SBFD co-existence

| Victim | Aggressor | Figures | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- |
| NR TDD DL | SBFD (DUD) | Case 1 | NR TDD DL | High |
|  | SBFD (DU) | Case 2 | NR TDD DL | High |
|  |  | Case 3 | NR TDD DL | Low |
| NR TDD UL | SBFD(DUD) | Case 4 | NR TDD UL | Low |
|  | SBFD(DU) | Case 5 | NR TDD UL | Low |
|  |  | Case 6 | NR TDD UL | Low |
| SBFD (DUD) | NR TDD DL | Case 1 | No system in adjacent channel | High |
| SBFD (DU) | NR TDD DL | Case 2 |  | High |
|  |  | Case 3 |  | Low |
| SBFD(DUD) | NR TDD UL | Case 4 |  | Low |
| SBFD(DU) | NR TDD UL | Case 5 |  | Low |
|  |  | Case 6 |  | Low |

Proposal 1: Case 1 and case 2 in Table 2 for SBFD     TDD DL are in high priority for the adjacent channel co-exist simulation.

2.2 Other simulation assumptions

For the remaining simulation assumptions, the following

Table 3: Remaining simulation assumptions

| Parameters | Assumption | Comments |
| --- | --- | --- |
| SBFD sub-band configurations | DUD: [40MHz, 20MHz, 40MHz] DU: [80MHz, 20MHz] | Whether the ratio of U impacts the results may need some discussion. |
| Grid shift | Option 1: 100% |  |
| Indoor/Outdoor UE ratio | Option 1-2: For Macro-to-Macro cases of both FR1 and FR2, assume 80% indoor and 20% outdoor ratio. |  |
| UE distribution mechanism | Option 2-1: Evenly random dropping in service area; |  |
| BS antenna configurations | Option 1-2: Utilize an extra panel for subband UL operation. In this case, the TRP and element number for DL and UL in SBFD BS will be the same as TDD BS configuration. | If only half of the TRP is used, the compare with the baseline may not be correct. |
| BS Tx power | Option 2, the same with legacy TDD |  |
| UE antenna and Tx power for FR2 | Option 1: Re-use TR 38.828 assumptions. i.e. FR2 max Tx 13.4dBm (peak eirp 22.4dBm), min Tx -40dBm, with antenna configuration in the table of annex 2 |  |
| BS mechanical down-tilt angle | Option 1: Use 6 degrees for Macro BS for FR1 and FR2 as provided in TR 38.803, and 90-deg (point to ground) for indoor. |  |
| Traffic model | Reuse 38.828 assumption |  |
| UE-UE interference model | Tx: UE in-band emission Rx: Rx ICS (image) |  |

2.4 Timing issue

In [4], we provide detail analysis on the timing issue. Figure 1 is copied from [4].

Figure 1: Different timing for BS side and UEs’ side

The analysis shows that the FFT calculation brings extra interference when the timing for aggressor UE and victim UE may not be aligned for inter-subband and adjacent channel. Whether co-existence study should take the timing impact into account should be discussed and decided.

Proposal 2: Whether co-existence study should take the timing impact into account should be discussed and decided.

- Summary

This contribution provides our view on the adjacent channel co-existence simulation assumption. We have the following proposals.

Proposal 1: Case 1 and case 2 in Table 2 for SBFD     TDD DL are in high priority for the adjacent channel co-exist simulation.

Proposal 2: Whether co-existence study should take the timing impact into account should be discussed and decided.

The views for the remaining detail assumptions are provided in Table 3.

Reference

[1] R4-2214378, “WF on adjacent channel co-existence study”, Samsung, RAN4#104e

[2] R4-2214379, “WF on Simulation assumption for adjacent co-existence study”, CMCC, RAN4#104e

[3] R4-2214304, “Email Discussion Summary for [104-e][315] FS_NR_duplex_evo”, Samsung, RAN4#104e

[4] R4-2215384, “Further discussion of the interference modelling for duplex evolution SLS”, CATT, RAN4#104be