3GPP TSG-RAN WG4 Meeting # 104bis-e                                                       R4-2215389

# Electronic Meeting, October 10 – October 19, 2022

Title: 	Discussion of remaining issues for FR2 MU

Source: 	CATT

Agenda item:	4.1.5.3.3

Document for:	Discussion

- Background

One for the remaining issues for the RF repeater conformance test is FR2 MU. This contribution provides our consideration based on the WF in RAN4#103e [1].

- Discussion

According to the WF [1], the followings are the open items for FR2 MU.

Table 1: Remaining open MUs for FR2

| Subclause | Maximum Test System Uncertainty and Range over which Test System Uncertainty applies |  |
| --- | --- | --- |
| 7.3 OTA Frequency stability | ±12 Hz Measurement results of TBD Hz |  |
| 6.4 OTA Out of band gain | FFS | This is not a BS requirement so no available value. Also it includes the input and output uncertainties |
| 7.7 OTA Input intermodulation | FFS | NO equivalent BS requirement but can be calculated using same approach as FR1 |
| 7.8 OTA Adjacent channel rejection ratio | FFS | No equivalent in BS spec. Relative measurement of 2 powers. |

For OTA frequency stability, we suggest following FR1 approach, i.e. reusing ±12 Hz and the range applies within the measurement results of  0.1ppm which is UE frequency error compared with BS. Then for FR2, it can be ±12 Hz within the measurement results of  5000Hz (0.1ppm of 50GHz).

For OTA out of band gain, MU of OTA output power Pmax,out,TRP can be reused as the requirement is TRP based.

For OTA input intermodulation, the MU can refer BS receiver output modulation and plus the MU of the output port. According to the analysis in [2], BS receiver intermodulation MU can be reused or + [0.1] dB. So we propose to reuse BS receiver intermodulation MU.

OTA ACRR, our understanding is that OTA ACLR MU can be reused.

So the following is our proposal for the remaining MU.

Table 2: Proposals for remaining open MUs for FR2

| Subclause | Maximum Test System Uncertainty and Range over which Test System Uncertainty applies | Comments |
| --- | --- | --- |
| 7.3 OTA Frequency stability | ±12 Hz Measurement results of 5000 Hz | 5000 Hz equals 0.1ppm of 50 GHz |
| 6.4 OTA Out of band gain | ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) ±2.2 dB (43.5 GHz < f ≤ 48.2 GHz) | Reuse output power MU |
| 7.7 OTA Input intermodulation | ±2.0 dB, f ≤ 3.0 GHz ±2.6 dB, 3.0 GHz < f ≤ 4.2 GHz ±3.2 dB, 4.2 GHz < f ≤ 6.0 GHz | Reuse BS receiver intermodulation MU |
| 7.8 OTA Adjacent channel rejection ratio | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) | Reuse absolute ACLR MU |

- Summary

This contribution provides our proposal for the remaining MUs for FR2. Proposals are provided in Table 2.

Reference

[1] R4-2211150, “WF on NR FR1/FR2 repeater MU and TT”, Huawei, RAN4#103e.

[2] R4-2208141, “Discussion of radiated test setup, MU and TT for NR repeater”, CATT, RAN4#103e.