3GPP TSG-RAN WG4 Meeting # 104bis-e                                                     R4-2215390			R4-22xxxxx

# Electronic Meeting, October 10 – October 19, 2022

Title: 	Further discussion on feasibility study for duplex evolution

Source: 	CATT

Agenda item:	6.17.2.2.1

Document for:	Discussion

- Background

In the last RAN4 meeting, a WF [1] on feasibility was approved with some preliminary understanding on how to model the RF performance. However, the details for feasibility were not discussed. This contribution provides further analysis and proposals from our company’s understanding.

- Discussion

  - gNB Self-interference

For the gNB self-interference, we analysed the 1dB degradation requirement for FR1. The followings are some assumptions for the analysis.

- BWchannel=100MHz

- Subband assignment：｛DUD｝， 40MHz DL+20MHz UL+40MHz DL

- Tx power over whole channel  PTx =46dBm

- ACLR：45dB

- Noise Figure: 5dB

- UL receiver sensitivity degradation due to self-interference: 1dB

- ACS：46dB

- Rx in-band blocking interfering signal mean power: -43dBm

Figure1: subband assignment

Figure 2: gNB Tx antenna output and Rx antenna input of self-interference

Interfering signal power can be calculated as in Table 1 according to the concept shown in Figure 2.

Table 1: Interfering signal mean powers of receiver

| Parameters | Value |
| --- | --- |
| Rx subband BWUL (MHZ) | 20 |
| Noise figure (dB) | 5 |
| Noise power (dBm/ BWUL) | -96 |
| Receiver sensitivity degradate (dB) | 1 |
| Receiver total noise power (dBm/ BWUL) | -95 |
| Interfering signal mean power (dBm/ BWUL) | -102 |

Both Tx leakage and ACS contribute to the noise. Assuming each of them contributes half of the noise,  so -105dBm is used as the noise power  in Tx leakage and ACS analysis.

2.1.1 Tx leakage contribution to the SI

Isolation or receiver performance improvement requirement:

Table 2: Tx leakage isolation or receiver performance improvement requirement of SI

| Parameters | Value |
| --- | --- |
| Tx subband BWDL (MHZ) | 40 |
| Rx subband BWUL (MHZ) | 20 |
| Tx power over whole channel PTx（dBm) | 46 |
| Interfering signal mean power (dBm/ BWUL) | -105 |
| Isolation or receiver performance improvement requirement (dB) | 148 |

2.1.2 Adjacent Channel Selectivity (ACS)

Isolation or receiver performance improved requirement:

Table 3: ACS isolation or receiver performance improvement requirement  of SI

| Parameters | Value |
| --- | --- |
| Tx subband BWDL (MHZ) | 40 |
| Rx subband BWUL (MHZ) | 20 |
| Tx power over whole channel PTx（dBm) | 46 |
| Interfering signal mean power (dBm/ BWUL) | -105 |
| Isolation or receiver performance improved improvement requirement (dB) | 151 |

2.1.3 Rx blocking consideration

Isolation or receiver performance improved requirement:

Table 4: Rx in-band blocking isolation or receiver performance improvement requirement

| Parameters | Value |
| --- | --- |
| Tx subband BWDL (MHZ) | 40 |
| Rx subband BWUL (MHZ) | 20 |
| Tx power over whole channel PTx（dBm) | 46 |
| BS in-band blocking requirement (dBm) | -43 |
| Isolation or receiver performance improved improvement requirement (dB) | 89 |

2.1.4 Summary

From the above analysis, if 1 dB is the SI target, the isolation to avoid blocking is not the most difficult performance. The dynamic range for ACS+small wanted signal and the ACS+Tx leakage contribution to the Rx sub band are the aspects should be taken into account. Table 5 shows the possible technologies and the performance requirement to reach the 1 dB target.

Table 5: Isolation or receiver performance improved requirements of SI

| Parameters | Tx leakage | ACS | Rx In-band blocking |
| --- | --- | --- | --- |
| Isolation or receiver performance improved requirements (dB) | 148 | 151 | 89 |
| Assume frequency isolation ACLR(dB) | 45 |  |  |
| Assume ACS (dB) |  | 46 |  |
| High isolation Antenna (dB) | √ | √ | √ |
| Subband RF filter (dB) |  | √ | √ |
| Digital filter (dB) |  | √ |  |
| Digital IC (dB) | √ |  |  |

It should be noted that the performance requirements are based on the specific DUD configuration and other assumptions including BS Tx power, etc. Other assumptions should be analysed case by case.

Observation 1: Both Tx leakage and ACS contribution to SI should be considered for the feasibility analysis. Different sub-band configurations should be analyzed case by case.

  - Co-channel co-site inter-sector gNB-to-gNB intra-subband CLI

For the two sub-band configurations shown in Figure 4,

Figure 4:  Co-channel co-site inter-sector gNB-to-gNB intra-subband CLI

Assuming gNB2:

- Subband assignment｛DU｝， 80MHz DL +20MHz UL

- Tx power over whole channel  PTx-gNB2 =46dBm.

The interference is shown in Figure 5.

Figure 5: Intra-subband CLI between inter-sector BS

2.2.1 Aggressor tx power directly falling in victim Rx SB

gNB2 DL sub-band output signal will fall into gNB1 UL subband, the isolation or receiver performance improvement needed are shown in Table 6.

Table 6: Tx power to Rx subband isolation or receiver performance improvement requirement of CLI

| Parameters | Value |
| --- | --- |
| Tx subband BWDL-gNB2 (MHZ) | 80 |
| Rx subband BWUL-gNB1 (MHZ) | 20 |
| Tx power over whole channel PTx-gNB2（dBm) | 46 |
| Interfering signal mean power (dBm/ BWUL-gNB1) | -105 |
| Isolation or receiver performance improvement requirement (dB) | 145 |

2.2.2 Adjacent Channel Selectivity (ACS)

For the adjacent channel aspect, the analysis is shown in Table 7.

Table 7: ACS isolation or receiver performance improvement requirement of CLI

| Parameters | Value |
| --- | --- |
| Tx subband BWDL-gNB2 (MHZ) | 80 |
| Rx subband BWUL-gNB1 (MHZ) | 20 |
| Tx power over whole channel PTx-gNB2（dBm) | 46 |
| Interfering signal mean power (dBm/ BWUL-gNB1) | -105 |
| Isolation or receiver performance improvement requirement (dB) | 151 |

2.2.3 Rx blocking consideration

Assume gNB1 in-band blocking interfering signal mean power is -43dBm, isolation or receiver performance improved requirement are shown in Table 8.

Table 8: Rx blocking isolation or receiver performance improvement requirement of CLI

| Parameters | Value |
| --- | --- |
| Tx subband BWDL-gNB2 (MHZ) | 80 |
| Rx subband BWUL-gNB1 (MHZ) | 20 |
| Tx power over whole channel PTx-gNB2（dBm) | 46 |
| Current BS in-band blocking requirement (dBm) | -43 |
| Isolation or receiver performance improved improvement requirement (dB) | 89 |

2.2.4 Summary

The summary is shown in Table 9.

Table 9: Isolation or receiver performance improved requirements of intra-subband CLI

| Parameters | Tx power to Rx subband | ACS | In-band blocking |
| --- | --- | --- | --- |
| Isolation or receiver performance improved requirement (dB) | 145 | 151 | 89 |
| High spatial isolation of different sectors (dB) | √ | √ | √ |
| Assume ACS (dB) | —— | 46 | —— |
| Subband RF filter (dB) |  | √ | √ |
| Digital filter (dB) |  | √ |  |

It can be seen that if 1 dB is still the target, without Tx ACLR and interference cancellation help, it’s very challenging. So the reference degradation target should be discussed and decided first. And the blocking issue should also be analyzed.

Observation 2: Interference cancellation between inter-sector gNBs is challenging, it can’t be assumed for the analysis.

Observation 3: For co-site inter-sector intra-subband gNB-to-gNB interference, blocking issue should be analyzed.

Observation 4: The target for the co-site inter-sector intra-subband BS interference should be discussed and decided first. The contribution from Tx power falling in overlapped spectrum and ACS contribution should be analyzed for different subband configurations.

  - Co-channel inter-cell co-site inter-sector gNB-to-gNB inter-subband CLI

For co-site inter-sector co-channel inter-subband CLI, the analysis is similar with intra-subband CLI with the difference that ACLR and ACS can do some help. The analysis is shown in the following figures and Table 10.

Figure 6:  Co-channel co-site inter-sector gNB-to-gNB inter-subband CLI

Figure 7: gNB2 Tx antenna output and gNB1 Rx antenna input of inter-subband CLI

Table 10: Isolation or receiver performance improved requirements of inter-subband CLI

| Parameters | Tx leakage | ACS | Rx In-band blocking |
| --- | --- | --- | --- |
| Isolation or receiver performance improved requirement (dB) | 148 | 151 | 89 |
| Assume frequency isolation ACLR(dB) | 45 |  |  |
| Assume ACS (dB) |  | 46 |  |
| High spatial isolation of different sectors (dB) | √ | √ | √ |
| Subband RF filter (dB) |  | √ | √ |
| Digital filter (dB) |  | √ |  |

Observation 5: For co-site inter-sector inter-subband interference, the analysis approach is similar with co-site inter-sector intra-subband gNB-to-gNB. ACLR and ACS can be assumed compared with intra-subband.

- Summary

Observation 1: Both Tx leakage and ACS contribution to SI should be considered for the feasibility analysis. Different sub-band configurations should be analyzed case by case.

Observation 2: Interference cancellation between inter-sector gNBs is challenging, it can’t be assumed for the analysis.

Observation 3: For co-site inter-sector intra-subband gNB-to-gNB interference, blocking issue should be analyzed.

Observation 4: The target for the co-site inter-sector intra-subband BS interference should be discussed and decided first. The contribution from Tx power falling in overlapped spectrum and ACS contribution should be analyzed for different subband configurations.

Observation 5: For co-site inter-sector inter-subband interference, the analysis approach is similar with co-site inter-sector intra-subband gNB-to-gNB. ACLR and ACS can be assumed compared with intra-subband.

Reference

[1] R4-2214377, “WF on feasibility study from RF perspective”, RAN4#104e

[2] R4-2214379, “WF on Simulation assumption for adjacent co-existence study”, RAN4#104e