3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215391

Electronic Meeting, October 10 – 19, 2022

Title: 	Discussion on fully overlapping concurrent MGs for NTN

Source: 	CATT

Agenda item:	4.2.5.1

Document for:	Discussion

- Introduction

In RAN4#104-e meeting, a way forward on NR NTN RRM requirements was approved [1], most issues have reached agreements, but the issue of fully overlapping concurrent MGs still is open. This document will further discuss this issue and present our understandings and proposal.

- Discussion

The state of issue of fully overlapping concurrent MGs is as following.

Issue 4. Fully Overlapping Concurrent MGs

Agreement:

- Option 1: Do not define requirements for fully overlapping concurrent MGs

- Option 2: For fully overlapped case, gap sharing rule is applied during the collided gap occasions, and the scaling factor is 2

  - Option 2A:

    - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

    - A MG with the lowest ID, i.e. 0, gets priority over the other, and the dropping rule starts from SFN=0, i.e. MG-ID#0 is selected and MG-ID#1 is dropped at the first collision instance after SFN=0, and it alternates afterwards.

    - [RAN4 introduce a new UE capability supporting “fully overlapping concurrent MGs” which is limited to NTN-only.]

  - Option 2B:

    - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

    - RAN4 introduce a new UE capability supporting “fully overlapping concurrent MGs” which is limited to NTN-only.

  - Option 2C:

    - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

The measurement gap is configured to UE by network. The scenario of two fully overlapping concurrent MGs configured by network should be an improper configuration. The network should avoid this scenario happened. If this scenario has happened unfortunately, it should belong to error configuration and the result will be UE implementation. RAN4 should not spend time to specify requirement and test for this scenario. So we support option 1.

Proposal: It is proposed that RAN4 do not define requirements for fully overlapping concurrent MGs.

- Conclusion

This document discussed the issues for fully overlapping concurrent MGs and presented the following proposal:

Proposal: It is proposed that RAN4 do not define requirements for fully overlapping concurrent MGs.

- Reference

[1]	R4-2214471, WF on NR NTN RRM requirements, Qualcomm Incorporated