3GPP TSG-RAN WG4 Meeting #104bis-e	R4- 2215396

Electronic Meeting, Oct. 10 – 19, 2022

Title: 	Further discussion on Rel-18 ATG RRM

Source: 	CATT

Agenda item:	6.13.5

Document for:	Discussion

- Introduction

In RAN4#104-e meeting, a way forward on NR ATG RRM core requirements was approved [1]. Most agreements have been achieved, and some open issues have been remained.

This document will further discuss open issues for ATG RRM requirements and give our understanding and proposals.

- Discussion

Issue 1-2-2: TDD impaction

- Option 1: FFS, RAN4 needs to study impact on TDD band operation due to longer propagation delay between ground gNB and ATG UE.

As described in WID, the cell coverage range will be up to 300 kilometres. In this case, the guard period between downlink slot group and uplink slot group will be larger than 2ms. The radio resource efficiency will only be lower than 60% for 5ms UL-DL switch-point periodicity, or lower than 80% for 10ms UL-DL switch-point periodicity. From point of specification, it can be supported by 5G frame configuration, but the radio resource efficiency will be very low. It should be decided by operator whether the TDD band is used for ATG operation.

Observation 1: The radio resource efficiency will be very low, it should be decided by operator whether the TDD band is used for ATG operation.

Issue 1-2-3: UE assistance information

- RAN4 further study ATG UE assistance information

- such as altitude, location, propagation delay difference, flight path, speed, velocity etc., or change in any of these parameters.

It should be relative with that whether the ATG UE does pre-compensation of transmit timing and frequency. If ATG UE is required to do pre-compensation of transmit timing and frequency, the UE assistance information, at least altitude, location, velocity, are need.

Observation 2: The UE assistance information, at least altitude, location, velocity, are needed if ATG UE is required to do pre-compensation of transmit timing and frequency.

Issue 2-2-1-4: NR Conditional Handover

- FFS which kind of CHO will be introduced

- FFS whether ATG specific CHO requirements are needed.

We prefer not consider conditional handover for ATG UE at least in Rel-18.

Proposal 1: Not to consider conditional handover for ATG UE at least in Rel-18.

Issue 3-1-2: The mechanism of Koffset and Kmac

- Option 1: The mechanism of Koffset and Kmac for NTN system should be used for ATG network. (CATT, Ericsson (if NTN as base), LGE)

- Option 2: no need to introduce the mechanism of Koffset and Kmac for ATG system (HW, CMCC, ZTE)

- Option 3: FFS (Apple)

The definition and purpose for Kmac in RAN1 are the estimate of UE-gNB RTT is equal to the sum of UE’s TA and Kmac, and the estimate of gNB-satellite RTT is equal to the sum of NTA, common×TC and Kmac. If a UE is provided with a Kmac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH as agreement in RAN1#105e meeting.

It can be see the Kmac reflects RTT between the timing reference point and gNB. In ATG network, the timing reference point and gNB should be at same point. The Kmac should not be need for ATG network.

As specified in TS38.213, “For a timing advance command received on uplink slot n and for a transmission other than a PUSCH scheduled by a RAR UL grant or a fallbackRAR UL grant as described in clause 8.2A or 8.3, or a PUCCH with HARQ-ACK information in response to a successRAR as described in clause 8.2A, the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot n+k+1+2μ∙Koffset”. Due to larger propagation delay in ATG than TN network. The Koffset should be used to support up to 300km cell coverage range.

Proposal 2: The Kmac should not be needed in ATG network, and the Koffset should be used to support up to 300km cell coverage range.

Issue 3-2-1: Whether to introduce UE based Timing pre-compensation

- Option 1: The ATG UE should do the compensation of transmit frequency based on relative moving velocity and distance between UE and gNB. (CATT, ZTE, Ericsson, LGE)

- Option 2: Further study whether to introduce the UE based UL timing pre-compensation based on necessity and performance gain. (CMCC, Apple, HW, Ericsson, ZTE)

- Option 2-1: Use the current timing adjustment procedure as the baseline. (CMCC, HW)

Issue 3-2-2: Whether to introduce UE based Frequency pre-compensation

- Option 1: The ATG UE should do the compensation of timing based on relative moving velocity and distance between UE and gNB. (CATT, Ericsson, LGE, CMCC)

- Option 2: Further study whether to introduce the UE based UL frequency pre-compensation based on necessity and performance gain. (CMCC, HW, Ericsson, Apple, ZTE)

- Option 2-1: Use the current frequency pre-compensation procedure as the baseline. (HW)

By our understanding, these two issues is correlative. Firstly the preamble formats for PRACH are defined in TS38.211 as following:

Table 6.3.3.1-1: PRACH preamble formats for  and ΔfRA∈1.25, 5 kHz.

| Format |  | ΔfRA |  |  | Support for restricted sets | CP(us) | GP(us) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 839 | 1.25 kHz | 24576κ | 3168κ | Type A, Type B | 103.125 | 96.875 |
| 1 | 839 | 1.25 kHz | 2⋅24576κ | 21024κ | Type A, Type B | 684.375 | 715.625 |
| 2 | 839 | 1.25 kHz | 4⋅24576κ | 4688κ | Type A, Type B | 152.604167 | 647.395833 |
| 3 | 839 | 5 kHz | 4⋅6144κ | 3168κ | Type A, Type B | 103.125 | 96.875 |

Table 6.3.3.1-2: Preamble formats for LRA∈139, 571, 1151 and ΔfRA=15⋅2μ kHz where μ∈0,1,2,3,5,6.

| Format |  |  |  | ΔfRA |  |  | Support for restricted sets |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | μ∈0,1,2,3,5,6 | μ∈0,3 | μ∈1,3, 5 |  |  |  |  |
| A1 | 139 | 1151 | 571 |  |  |  | - |
| A2 | 139 | 1151 | 571 |  |  |  | - |
| A3 | 139 | 1151 | 571 |  |  |  | - |
| B1 | 139 | 1151 | 571 |  |  |  | - |
| B2 | 139 | 1151 | 571 |  |  |  | - |
| B3 | 139 | 1151 | 571 |  |  |  | - |
| B4 | 139 | 1151 | 571 |  |  |  | - |
| C0 | 139 | 1151 | 571 |  |  |  | - |
| C2 | 139 | 1151 | 571 |  |  |  |  |

From these preamble formats, we can see the longest CP is format 1, i.e. 684.375µs. The longest distance supported between UE and gNB is 684.375us * 299792458 m/s /1*106 / 2 = 102585.231721875m ≈ 102.585km. NR system can’t support up to 300km cell coverage range if not introducing UE based transmit timing advance pre-compensation. So The ATG UE should do the compensation of transmit timing advance as NTN UE based on distance between UE and gNB.

For doing UL timing pre-compensation, UE must know gNB location, and measure position itself using GNSS or other method. The relative moving velocity between UE and gNB can also be gotten by ATG UE meanwhile. On the other hand, the modulation performance will be degraded if the Doppler frequency offset is large like study in HST (only up to 500km/h). The ATG UE moving velocity will be up to 1200km/h. The modulation performance degrade will be expected if ATG UE not doing the compensation of transmit frequency. So the ATG UE should do the compensation of transmit frequency based on relative moving velocity between UE and gNB.

Proposal 3: The ATG UE should do the compensation of transmit frequency and timing based on relative moving velocity and distance between UE and gNB.

Issue 3-3-1-1: Initial transmit timing requirements Te

- Option 1: Need defined RRM requirements for ATG UE. (CATT, Apple, CMCC, HW, Ericsson)

- FFS if UE specific TA shall be considered in the Te requirement design, like in NTN (Apple, CMCC, HW)

- Introduce UE specific TA in the Te requirement design. (CATT)

As discussed in Issue 3-2-1, UE based the timing pre-compensation is needed and the UE specific TA like NTN UE should be introduced. The initial transmit timing requirement Te need to be defined for ATG UE, and the requirements for NTN UE can be used as baseline.

Proposal 4: The initial transmit timing requirement Te need to be defined for ATG UE, and the requirements for NTN UE can be used as baseline.

Issue 3-3-1-2: Gradual timing adjustment

- Tp and Tq shall be updated for ATG UE

- FFS if TN or NTN rel-17 is used as baseline

- FFS how to define the exact value

As discussed in NTN, the TN gradual timing adjustment requirement can be reused.

Proposal 5: The requirements of gradual timing adjustment for TN or NTN in Rel-17 can be used as baseline.

Issue 3-3-3: Timing advance

- FFS on the necessity of considering the open loop TA (UE specific TA if needed) and close loop (TAC based adjustment) for the TA adjustment requirement, like in NTN. (CMCC, Apple, Ericsson, LGE, ZTE)

As discussed in issue 3-2-1 and 3-2-2, UE specific TA is needed. The open loop TA and close loop TA are necessity for TAG UE.

Proposal 6: The open loop TA (UE specific TA) and close loop TA (TAC based adjustment) are necessity for TAG UE.

Issue 3-3-4: Cell phase synchronization accuracy

- Cell phase synchronization accuracy will be defined for ATG, the legacy TN requirement can be the baseline

- FFS whether to tighten the requirements or not

The necessity to tighten the requirements for cell phase synchronization accuracy is not seen. The legacy TN requirement for cell phase synchronization accuracy can be reused.

Proposal 7: The legacy TN requirement for cell phase synchronization accuracy can be reused.

Issue 3-3-5: deriveSSB-IndexFromCell tolerance

- Option 1: Need defined RRM requirements for ATG UE. (Apple, CMCC, ZTE)

- Option 1-1: The time misalignment tolerance for ‘deriveSSB-IndexFromCell= true’ shall be revisited due to the extreme large radius of ATG cell. (Apple, LGE)

- Option 1-2: The legacy TN requirement can be reused (CMCC)

- Option 1-3: The propagation delay different may impact the tolerance. FFS details. (HW, Ericsson)

- Option 2: Not applicable for R18 ATG (CATT)

We think the CA and DC will not be considered for Rel-18 ATG. The feature of deriveSSB-IndexFromCell may not be considered in Rel-18, or the legacy TN requirement can be reused.

Proposal 8: Not to define deriveSSB-IndexFromCell tolerance in Rel-18, or the legacy TN requirement can be reused.

Issue 6-1-1: How to involve ATG RRM core requirements in TS38.133

- Option 1: The RRM requirements for ATG UE can be defined in new sections of section number with suffix D in specification.

- Option 2: Add ATG related requirements in the current corresponding section, similar as HST.

By our current understanding, most legacy TN RRM requirements can be reused for ATG UE except requirements for initial transmit timing error due to UE transmit timing and frequency pre-compensation is necessary, and the measurement requirements may be defined refer to requirements for HST. It should be applicable that add ATG related requirements in the current corresponding section, similar as HST.

Observation 3: Option 2 should be applicable, i.e. add ATG related requirements in the current corresponding section, similar as HST.

- Conclusion

This document discussed some issues for ATG RRM requirements and presented following observations and proposals:

Observation 1: The radio resource efficiency will be low, it should be decided by operator whether the TDD band is used for ATG operation.

Observation 2: The UE assistance information, at least altitude, location, velocity, are needed if ATG UE is required to do pre-compensation of transmit timing and frequency.

Observation 3: Option 2 should be applicable, i.e. add ATG related requirements in the current corresponding section, similar as HST.

Proposal 1: Not to consider conditional handover for ATG UE at least in Rel-18.

Proposal 2: The Kmac should not be needed in ATG network, and the Koffset should be used to support up to 300km cell coverage range.

Proposal 3: The ATG UE should do the compensation of transmit frequency and timing based on relative moving velocity and distance between UE and gNB.

Proposal 4: The initial transmit timing requirement Te need to be defined for ATG UE, and the requirements for NTN UE can be used as baseline.

Proposal 5: The requirements of gradual timing adjustment for TN or NTN in Rel-17 can be used as baseline.

Proposal 6: The open loop TA (UE specific TA) and close loop TA (TAC based adjustment) are necessity for TAG UE.

Proposal 7: The legacy TN requirement for cell phase synchronization accuracy can be reused.

Proposal 8: Not to define deriveSSB-IndexFromCell tolerance in Rel-18, or the legacy TN requirement can be reused.

- Reference

[1]	R4-2214347, WF on NR ATG RRM core requirements, CMCC