3GPP TSG-RAN WG4 Meeting #104-bis-e                                                                                   R4-2215413

Electronic Meeting, October 10 – October 19, 2022

Source: 	CATT

Title: 	General consideration on mmWave multi-band BS

Agenda item:	6.4.2

Document for:	Approval

- Introduction

In RAN4#104-e meeting, a WF [1] on investigation of mmWave multi-band BS was agreed. Some open issues in WF need further discussion.

In this meeting, this contribution provides our further consideration on mmWave multi-band BS.

- Discussion

Definition of FR2 multi-band BS

As per the WF [1], the remaining issues concerning definition of FR2 multi-band BS are shown as below:

## Definition of FR2 multi-band BS - To investigate if the FR1 definition of multi-band RIB can be re-used for FR2 - To revisit the definition of multi-band BS for FR2, and clarify whether the following scenarios should be considered as multi-band BS 1) Multi-band transmitter and/or receiver with common active RF components 2) Single-band transmitter and receiver 3) Configurable BS for different bands with the same hardware 4) BS covers full-band or sub-band of band A and band B 5) BS covers consecutive spectrums with different band number, for example, n258+n261 6) BS covers overlapping spectrums with different band number, for example, n258+n257 - FFS whether need to introduce the definition of multi-band BS for FR2 into specification

The multi-band RIB and single-band RIB defined in TS 38.104 are copied in follows:

multi-band RIB: operating band specific RIB associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band. single-band RIB: operating band specific RIB supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band RIB.

From definition of multi-band RIB, the transmitter or receiver of multi-band RIB dependent BS have ability to process two or more carriers from the different bands in common active RF components simultaneously. From our understanding, because common active RF components for different bands are used, so one band is on, and other band is on simultaneously. In this case, the emissions in at least two bands always exist simultaneously, so the requirements for this multi-band RIB dependent BS in two bands operation need specific additions or exclusions. For this scenario in FR2, it also need specific additions or exclusions, we think existing multi-band RIB definition for FR1 is applicable for FR2.

Observation 1: FR1 definition of multi-band RIB can be re-used for FR2.

According to definition of FR1 multi-band BS, FR1 multi-band BS was defined in TS 38.104 as below:

BS type 1-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 1-O capability of operation in multiple operating bands.

From above description, the BS type 1-O capability of operation in multiple operating bands can be summarized into the following two types:

- BS with multi-band RIB.

-  BS without multi-band RIB.

There is only BS with multi-band RIB need specific additions or exclusions for requirement. For BS without multi-band RIB, only single-band requirements apply. The definition of multi-band RIB has two key points. One key point is that the common active RF components of transmitter or receiver can process two or more carriers from different band simultaneously, and another key point is that different operating band is not a sub-band or superseding-band of another supported operating band. So we only need to discuss which scenario in WF [1] is BS with multi-band RIB.

| Scenario | BS with multi-band RIB | Reason |
| --- | --- | --- |
| 1) Multi-band transmitter and/or receiver with common active RF components | Yes | Meet definition of multi-band RIB |
| 2) Single-band transmitter and receiver | No | Don’t meet definition of multi-band RIB |
| 3) Configurable BS for different bands with the same hardware | No | Don’t meet definition of multi-band RIB, only one band is on. |
| 4) BS covers full-band or sub-band of band A and band B | Depend on whether transmitter or receiver have ability to process two or more carriers from the different bands in common active RF components simultaneously | Band A and Band B is different band, which meets “this different operating band is not a sub-band or superseding-band of another supported operating band” in definition of multi-band RIB. |
| 5) BS covers consecutive spectrums with different band number, for example, n258+n261 | Depend on whether transmitter or receiver have ability to process two or more carriers from the different bands in common active RF components simultaneously | n258 and n261 is different band, which meets “this different operating band is not a sub-band or superseding-band of another supported operating band” in definition of multi-band RIB. |
| 6) BS covers overlapping spectrums with different band number, for example, n258+n257 | No | Because BS covers overlapping spectrums with different band number, n258 is overlapping with n257, for some carrier in band n258, it can be considered as sub-band of n257. So this scenario doesn’t meet “this different operating band is not a sub-band or superseding-band of another supported operating band” in definition of multi-band RIB. From spectrum number in same region, this overlapping spectrum does not exist. |

Based on above analysis, existing definition of multi-band RIB can cover 6 scenarios in WF. So, 1) is BS with multi-band RIB; 2), 3) and 6) is BS without multi-band RIB; for 4) and 5) whether they are BS with multi-band RIB depends on whether transmitter or receiver have ability to process two or more carriers from the different bands in common active RF components simultaneously.

Observation 2: 1) is BS with multi-band RIB; 2), 3) and 6) is BS without multi-band RIB; for 4) and 5) whether they are BS with multi-band RIB depends on whether transmitter or receiver have ability to process two or more carriers from the different bands in common active RF components simultaneously.

According to definition of FR1 multi-band BS in TS 38.104, definition of FR1 multi-band BS are introduced into specification to illustrate  the possible implementations supported by this specification. So for FR2 multi-band BS, introducing the definition of multi-band BS for FR2 into specification is reasonable.

Observation 3: Introducing the definition of multi-band BS for FR2 into specification to illustrate the possible implementations supported by this specification is reasonable.

If there is BS with multi-band RIB can be implemented, the FR2 multi-band BS can be defined as following:

BS type 2-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 2-O capability of operation in multiple operating bands.

Otherwise, the FR2 multi-band BS can be defined as following:

BS type 2-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

We will discuss the feasibility of FR2 multi-band BS in the later clause of this contribution.

Observation 4: Definition of FR2 multi-band BS may be,

If there is BS with multi-band RIB can be implemented, the FR2 multi-band BS can be defined as following:

BS type 2-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 2-O capability of operation in multiple operating bands.

Otherwise, the FR2 multi-band BS can be defined as following:

BS type 2-O may be capable of supporting operation in multiple operating bands with the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

Feasibility of FR2 multi-band BS

As per the WF [1], the remaining issues concerning feasibility of FR2 multi-band BS are shown as below:

## Feasibility of FR2 multi-band BS - The following technical challenges need to be studied for FR2 multi-band BS - RF front-end - Antenna array - Phase shifters - Beamforming architectures - Others are not excluded - The largest feasible bandwidth for an FR2 multi-band BS should be studied and decided in the SI.

The radiated reference points for BS type 2-O defined in TS 38.104 is shown in Figure 2-1, for multi-band BS type 2-O, we need to discuss the feasibility and implementation structure of multi-band TRXUA(Transceiver Unit Array) and multi-band AA(Antenna Array). From perspective of BS products, RF filter before antenna is also needed. Band RF filter is single-band passive RF component, so definition of multi-band RIB is independent on Band RF filter.

Figure 2-1: Radiated reference points for BS type 2-O

Referring to currently mmWave BS implementation, the TRXUA (Transceiver Unit Array) is made up of multiple beamformer ICs. Each beamformer IC include PAs (LNAs), Switched phase shifters, combiner, etc. So RF front-end, phase shifters, and beamforming architectures in WF are integrated in beamformer IC, we only need to discuss feasibility of beamformer IC is sufficient. Since beamformer IC is active RF component, so definition of multi-band RIB is dependent on beamformer IC. Antenna array is passive RF component, so definition of multi-band RIB is independent on Antenna array (AA).

Below, we discuss the feasibility of multi-band beamformer IC and multi-band Antenna array.

Feasibility of Antenna array

The FBW (Fractional bandwidth) of FR2-1 operating bands and possible multi-band frequency range is shown in table 2-1.

Table 2-1 FBW of FR2-1 operating band

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode | Fractional bandwidth(FBW) |
| --- | --- | --- | --- |
| n258 | 24250 MHz – 27500 MHz | TDD | 12.6% |
| n257 | 26500 MHz – 29500 MHz | TDD | 10.7% |
| n261 | 27500 MHz – 28350 MHz | TDD | 3% |
| Frequency range 24-29 GHz which includes n257/n258/n261 | 24250 MHz –29500 MHz | TDD | 19.5% |
| n260 | 37000 MHz – 40000 MHz | TDD | 7.8% |
| n259 | 39500 MHz – 43500 MHz | TDD | 9.6% |
| n262 | 47200 MHz – 48200 MHz | TDD | 2.1% |
| Frequency range 37-48 GHz which includes n260/n259/n262 | 37000 MHz – 48200 MHz | TDD | 26.3% |
| Example bands:26+28 GHz: n258 + n261 | 24250 MHz – 29500 MHz | TDD | 19.5% |
| Example bands:28+39 GHz: n257/n261 + n260 | 26500 MHz – 40000 MHz | TDD | 40.6% |
| Example bands:26+40 GHz: n258 + n259/n262 | 24250 MHz – 48200 MHz | TDD | 66.1% |
| Example bands:28+40 GHz: n257/n261 + n259/n262 | 26500 MHz – 48200 MHz | TDD | 58.1% |

Referring to multi-band base station passive antennas specifications for mobile communication in China [2], for multi-band antenna with common radiated element, FBW of single-band antenna is less than 15%, FBW of wide-band antenna is between 15% and 25%, and FBW of ultra-wide band antenna is larger than 25%. When FBW is higher than 35%, as FBW becomes larger, it becomes more difficult to implement.

From Table 2-1, FBW of 26+28 GHz is 19.5%. From perspective of antenna product, multi-band AA (Antenna Array) with common radiated element with 19.5% FBW in 26+28 GHz combination is feasible.

Observation 5: Multi-band AA (Antenna Array) with common radiated element with 19.5% FBW in 26+28 GHz combinations is feasible.

From Table 2-1, the FBW of Frequency range 24-29 GHz which includes n257/n258/n261 and Frequency range 37-48 GHz which includes n260/n259/n262 is 19.5% and 26.3%, respectively.  From perspective of antenna product, for FBW 19.5% and 26.3%, multi-band AA(Antenna Array) with common radiated element with  19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261, or with 26.3% FBW in frequency range 37-48 GHz which includes n260/n259/n262 is feasible.

Observation 6: Multi-band AA (Antenna Array) with common radiated element with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261, or with 26.3% FBW in frequency range 37-48 GHz which includes n260/n259/n262 is feasible.

From Table 2-1, FBW of 28+39 GHz, 26+40 GHz, and 28+40 GHz are 40.6%, 66.1%, and 58.1% respectively. From perspective of antenna product, for such larger FBW, multi-band AA (Antenna Array) with common radiated element for 28+39 GHz, 26+40 GHz, and 28+40 GHz combinations is not feasible. In this case, it is possible to use multiple separate single-band AAs (Antenna Array) for ultra-wide band combination, for example, one single-band AA is for frequency range 24-29 GHz  which includes n257/n258/n261, and another single-band AA is for frequency range 37-48 GHz which includes n260/n259/n262.

Observation 7: Multi-band AA (Antenna Array) with common radiated element with higher than 40.6% FBW in 28+39 GHz, 26+40 GHz, and 28+40 GHz combinations is not feasible.

Observation 8: It is possible to use multiple separate  single-band AAs (Antenna Array) for ultra-wide band combination, for example, one single-band AA is for frequency range 24-29 GHz  which includes n257/n258/n261, and another single-band AA is for frequency range 37-48 GHz which includes n260/n259/n262.

Feasibility of beamformer IC

Because the mmWave BS implementation is dependent on beamformer IC, some chip companies supported the beamformer IC in 24-29.5GHz, FBW in frequency range 24-29.5 GHz which includes n257/n258/n261 is 19.5%, The parameters of beamformer IC in 24-29.5GHz are shown in Table 2-2, and the PA bias optimized for P1dB for 24GHz and 28GHz for beamformer IC in 24-29.5GHz is shown in Figure 2-2. So the Multi-band beamformer IC with common active RF components with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261 is feasible.

Observation 9: Multi-band beamformer IC with common active RF components with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261 is feasible.

Table 2-2: beamformer IC in 24-29.5GHz

| Parameter | Beamformer |
| --- | --- |
| Frequency Band | 24-29.5GHz |
| Tx Op1dB/OIP3 | 21dBm/ 25dBm |
| Tx Pdiss/Ch @ 12dBm Pout | 300mW |
| Pout @3% EVM w/ 64QAM | 12dBm |
| Rx Single Channel NF | 4dB |
| Rx Pdiss | 130mW |
| Instantaneous Bandwidth | 1600MHz |

Figure 2-2: PA bias optimized for P1dB for 24GHz (left), and 28GHz (right)

As FBW increases, the performance of AA and beamformer decreases. The largest feasible FBW (or bandwidth) for an FR2 multi-band RIB dependent BS is dependent on the trade-off between FBW and performance degradation. According to typical FR1 dual band n8+n20 with 19.3% FBW and above beamformer IC and AA in frequency range 24-29.5 GHz which includes n257/n258/n261 with 19.5% FBW,  [20%]  can be considered as the largest feasible FBW for an FR2 multi-band RIB dependent BS.

Observation 10: [20%] can be considered as the largest feasible FBW for an FR2 multi-band RIB dependent BS.

Requirement for FR2 multi-band operation

As per the WF [1], the remaining issues concerning requirement for FR2 multi-band operation are shown as below:

- The following topics should be considered for investigation of FR2 multi-band BS: 1) Additional declarations for FR2 multi-band BS 2) The applicability of multi-band requirements 3) OTA transmitter OFF power 4) OTA Adjacent Channel Leakage Power Ratio (ACLR) 5) OTA operating band unwanted emissions 6) OTA transmitter spurious emissions 7) OTA adjacent channel selectivity 8) OTA in-band blocking 9) OTA out-of-band blocking 10) OTA receiver spurious emissions 11) OTA receiver intermodulation 12) OTA EVM 13) EIRP accuracy

Figure2-1: Graphical description of suggested terminology [3]

In TR 38.712, the graphical description of suggested terminology for FR1 multi-band methods are as defined in Figure 2-1. The definition of inter RF bandwidth gap, Base Station RF bandwidth in one operating band (for example, RF bandwidth of Band X, RF bandwidth of Band Y) in Figure 2-1 are defined in the following from TS 38.104.

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands

Base Station RF Bandwidth: RF bandwidth in which a base station transmits and/or receives single or multiple carrier(s) within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the BS channel bandwidth.

As shown in Figure 2-1, the existing terminology inter RF bandwidth gap, Base Station RF bandwidth accurately describe the frequency gap relationship between two different operating bands. We believe that existing definitions of Inter RF Bandwidth gap and Base Station RF Bandwidth for multi-band operation can be reused for FR2 multi-band operation.

Observation 11: Existing definitions of Inter RF Bandwidth gap and Base Station RF Bandwidth for multi-band operation can be reused for FR2 multi-band operation.

Referring to TS 37.104[4], for BS capable of multi-band operation, the RF requirements defined for transmitter and receiver for each supported operating band are applicable unless otherwise stated. For some requirement, to add specific additions or exclusions to the requirement apply for BS capable of multi-band operation.

The specific additions or exclusions to OTA requirement for FR1 multi-band operation in TS 38.104 are listed in Table 2-2 below.

Table 2-2 specific additions or exclusions to OTA requirement for FR1 multi-band operation

| 9.5.2 OTA transmitter OFF power | For multi-band RIBs and single band RIBs supporting transmission in multiple bands, the OTA transmitter OFF power requirement is only applicable during the transmitter OFF period in all supported operating bands. |
| --- | --- |
| 9.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | For a multi-band RIB, the ACLR and CACLR requirement shall apply in Inter RF Bandwidth gaps for the frequency ranges. |
| 9.7.4 OTA operating band unwanted emissions | For a multi-band RIB, the OTA operating band unwanted emissions requirements shall apply inside any Inter RF Bandwidth gap for the frequency ranges. |
| 9.7.5 OTA transmitter spurious emissions | For multi-band RIB each supported operating band and ΔfOBUE MHz around each band are excluded from the OTA transmitter spurious emissions requirements. |
| 10.5.1 OTA adjacent channel selectivity | For multi-band RIBs, the OTA ACS requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap. |
| 10.5.2 OTA in-band blocking | For multi-band RIBs, the OTA in-band blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset. For a multi-band RIBs, the OTA narrowband blocking requirements apply in the narrowband blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the interfering signal minimum offset. |
| 10.6 OTA out-of-band blocking | For a multi-band RIB, the OTA out-of-band requirement shall apply for each supported operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to clause 7.4.2.2 shall be excluded from the OTA out-of-band blocking requirement. |
| 10.7 OTA receiver spurious emissions | For RX only multi-band RIB, the OTA RX spurious emissions requirements are subject to exclusion zones in each supported operating band. |
| 10.8 OTA receiver intermodulation | For multi-band RIBs, the OTA intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the NR interfering signal centre frequency offset from the Base Station RF Bandwidth edge. For multi-band RIBs, the OTA narrowband intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the NR interfering signal The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap. |

As shown in Table 2-2, the inter RF bandwidth gap is used for defining requirement for FR1 OTA multi-band operation in TS 38.104. For FR1 OTA BS capable of multi-band operation, the RF requirements defined for radiated transmitter and receiver for each supported operating band are applicable unless otherwise stated. For some requirement, to add specific additions or exclusions to the requirement apply for BS capable of multi-band operation.

We think existing method for requirement for FR1 multi-band operation can be reused for that for FR2 multi-band operation.

Observation 12:  The existing method for requirement for FR1 multi-band operation can be reused for that for FR2 multi-band operation.

- Conclusion

This contribution provides our general consideration on mmWave multi-band BS. The following observations are concluded as follows:

Observation 1: FR1 definition of multi-band RIB can be re-used for FR2.

Observation 2: 1) is BS with multi-band RIB; 2), 3) and 6) is BS without multi-band RIB; for 4) and 5) whether they are BS with multi-band RIB depends on whether transmitter or receiver have ability to process two or more carriers from the different bands in common active RF components simultaneously.

Observation 3: Introducing the definition of multi-band BS for FR2 into specification to illustrate the possible implementations supported by this specification is reasonable.

Observation 4: Definition of FR2 multi-band BS may be,

If there is BS with multi-band RIB can be implemented, the FR2 multi-band BS can be defined as following:

BS type 2-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 2-O capability of operation in multiple operating bands.

Otherwise, the FR2 multi-band BS can be defined as following:

BS type 2-O may be capable of supporting operation in multiple operating bands with the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

Observation 5: Multi-band AA (Antenna Array) with common radiated element with 19.5% FBW in 26+28 GHz combinations is feasible.

Observation 6: Multi-band AA (Antenna Array) with common radiated element with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261, or with 26.3% FBW in frequency range 37-48 GHz which includes n260/n259/n262 is feasible.

Observation 7: Multi-band AA (Antenna Array) with common radiated element with higher than 40.6% FBW in 28+39 GHz, 26+40 GHz, and 28+40 GHz combinations is not feasible.

Observation 8: It is possible to use multiple separate  single-band AAs (Antenna Array) for ultra-wide band combination, for example, one single-band AA is for frequency range 24-29 GHz  which includes n257/n258/n261, and another single-band AA is for frequency range 37-48 GHz which includes n260/n259/n262.

Observation 9: Multi-band beamformer IC with common active RF components with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261 is feasible.

Observation 10: [20%] can be considered as the largest feasible FBW for an FR2 multi-band RIB dependent BS.

Observation 11: Existing definitions of Inter RF Bandwidth gap and Base Station RF Bandwidth for multi-band operation can be reused for FR2 multi-band operation.

Observation 12:  The existing method for requirement for FR1 multi-band operation can be reused for that for FR2 multi-band operation.

- References

- R4-2214375, WF on investigation of mmWave multi-band BS, Huawei, HiSilicon, RAN4#104-e

- RP-221001, New SID: NR BS RF requirement evolution, Huawei, HiSilicon, RAN#95 -e

- YD/T 2867-2015，Multiple bands base station passive antennas specifications for mobile communication

- 3GPP TR 37.812 V11.3.0

- 3GPP TS 37.104 V17.6.0